3GPP TSG-RAN WG3 #123bis	R3-242228

Changsha, China, 15-19 April 2024

Agenda Item:	11.4

Source:	Ericsson, ZTE

Title:	TP on multi-hops UE Trajectory

Document for:	Discussion, Agreement

# TP to TR38.743

<<<<<< Start of CHANGES >>>>>

## 5.5	Multiple-hop UE trajectory across gNBs

### 5.5.1 Use case description

A multi-hop UE trajectory consists of a list of cells in which the UE will be served in RRC_CONNECTED mode. The cells are listed in chronological order and belong to at least two different gNBs.

During handover request, a gNB with a trained and capable AI/ML model, upon inferring a multi-hop UE trajectory prediction, may decide to transmit the prediction to the subsequent gNB wherein the predicted cells reside.

A gNB may request for a measured UE trajectory that span across multiple gNBs.

<<<<<< End of CHANGES >>>>>