3GPP TSG RAN3 123-bis	R3-241512

Changsha, China, 15 - 19 April, 2024

3GPP TSG-RAN WG2 Meeting #125                                                                  R2-2401644

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Title:	Questions on RAN1 parameter list

Response to:	R1-2312661/R2-2400031

Release:	Release 18

Work Item:	NR_pos_enh2-Core

Source:	RAN2

To:	RAN1

Cc:	RAN3, RAN4

Contact Person:

Name:	Jianxiang Li

E-mail Address:	lijianxiang@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 thanks RAN1 for the Rel-18 parameter list in LS R1-2312661. We would like to further investigate the following issues with RAN1.

Issue 1: DL-PRS Rx hopping for NR DL-AoD positioning method

According to the RRC parameter list in R1-2312697 included in the LS, LMF can request UE to perform DL-PRS measurements based on receiving multiple hops of DL-PRS using the parameter nr-Requested-DL-PRS-measurementBasedOnMultihopRx. According to R1-2312697, the parameter nr-ReportedDL-PRS-measurementBasedOnSingleOrMultihopRx, which indicates that the reported measurement is based on receiving single or multiple hops of DL-PRS, is only applicable for NR DL-TDOA and NR Multi-RTT and does not mention other positioning methods. However, from RAN2’s perspective, if LMF can request UE to perform DL-PRS Rx hopping for NR DL-TDOA and NR Multi-RTT, it should also be able to request the reported measurement for NR DL-AoD.

Question 1: Does the parameter nr-ReportedDL-PRS-measurementBasedOnSingleOrMultihopRx also apply to NR DL-AoD positioning?

Issue 2: Clarification on the DL-PRS ID associated with aggregated measurement report

RAN2 observed that there is neither dl-PRS-ID nor dl-PRS-IDs included in the RRC parameter list R1-2312697. It is unclear to RAN2 how to include dl-PRS-ID when reporting the nr-aggregated-DL-PRS-ResourceSetIDList in NR-DL-TDOA-MeasElement and NR-Multi-RTT-MeasElement.

Question 2: Is there only one dl-PRS-ID or are there multiple dl-PRS-IDs associated with the aggregated main and additional measurement, respectively?

Question 3: If there are multiple dl-PRS-IDs associated with main and additional measurements, respectively, should the list of the dl-PRS-IDs in additional measurements be included in the list of dl-PRS-IDs in the main measurement?

2. Actions:

To RAN1

ACTION:	 RAN2 kindly asks RAN1 to answer the above questions.

3. Date of Next RAN2 Meetings:

TSG-RAN WG2#125-bis                      2024-04-15 to 2024-04-19		Changsha, Hunan Province , CN

TSG-RAN WG2#126                            2024-05-20 to 2024-05-24		Fukuoka City, Fukuoka , JP