3GPP TSG-RAN WG3 Meeting #123	R3-240040

# Athens, Greece, 26 February – 01 March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR | Draft | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BL CR to TS 38.305) Support of NR Positioning Enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, CATT, Huawei, Ericsson, Xiaomi, ZTE, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CR introduces functionality for support of NR positioning enhancements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description of procedures for area-specific SRS configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing stage 2 for support of NR positioning enhancements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | RAN3#121bis (post): Inclusion of agreed TP (R3-235812) RAN3#122 (pre): Resubmission to RAN3#122 RAN3#122 (post): Inclusion of agreed TP (R3-237917) RAN3#123 (pre): Resubmission to RAN3#123 |  |  |  |  |  |  |  |  |

Start of change

## 7.x	Procedures for Area-specific SRS Configuration

### 7.x.1	General

The Area-specific SRS Configuration procedure is used by the network to configure the area-specific SRS configuration for the UE, including the Area-specific SRS Configuration Allocation procedure and Area-specific SRS Configuration Update procedure.

### 7.x.2	Area-specific SRS Configuration Allocation Procedure

Figure 7.x.2-1 shows the Area-specific SRS Configuration Allocation procedure.

Figure 7.x.2-1: Area-specific SRS Configuration Allocation Procedure

1.	The LMF initiates NRPPa Positioning Information Request towards the serving gNB, including Positioning Validity Area Cell List in the Requested SRS Transmission Characteristics.

2.	The serving gNB allocates the area-specific SRS configuration, and moves the UE back to RRC_INACTIVE by sending RRCRelease message, which includes the area-specific SRS configuration.

3.	The serving gNB responds with the NRPPa Positioning Information Response to the LMF, including the Positioning Validity Area Cell List.

4.	The LMF notifies the gNBs within the LPHAP validity area to reserve the SRS configuration.

### 7.x.3	Area-specific SRS Configuration Update Procedure

Figure 7.x.3-1 shows the Area-specific SRS Configuration Update procedure.

Figure 7.x.3-1: Area-specific SRS Configuration Update Procedure

0.	The UE in RRC_INACTIVE is configured with an area-specific SRS configuration.

1.	The UE requests for new SRS configuration in case of e.g. it moves out of the configured validity area.

2.	The receiving gNB which receives the request from the UE triggers the Retrieve UE Context procedure towards the last serving gNB.

3.	The last serving gNB notifies the LMF the UE moved out of the validity area by providing the Cell ID of the receiving gNB where the UE resumes from.

4.	The last serving gNB relocates the full UE context to the receiving gNB.

5.	The receiving gNB triggers the Path Switch Request procedure towards the AMF.

6.	The AMF responds with the Path Switch Request Acknowledge.

7.	The LMF requests the receiving gNB to allocate new SRS configuration for the UE. If Positioning Validity Area Cell List is included in the Requested SRS Transmission Characteristics, the Area-specific SRS Configuration Allocation procedure as specified in section 7.x.2 is applied. Otherwise, the legacy SRS allocation procedure is applied.

8.	The receiving gNB indicates the last serving gNB to release the UE context.

End of change