3GPP TSG-RAN WG3 Meeting #123	R3-240090

Athens, GR, Feb 26 – Mar 01, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0320 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TS 38.401 on NR MBS enh |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Qualcomm, CBN |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_enh-Core |  |  |  |  |  | Date: |  |  | 2024-01-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. it is not clear which session type the description is about in various places. 2. the condition for the shared F1-U resources is established and kept is not correct, it should be “at least one PLMN keeping the broadcast MBS session”, in section “7.7.2.2 Support of resource efficiency for MOCN”. 3. the resource efficiency for network sharing is only applied to broadcast MBS session instead of multicast services, in section “7.7.2.2 Support of resource efficiency for MOCN”. 4. clarify that it is the “entity controlling the involved gNB-DUs sharing the same physical cell resources” who to resolve different QoS requirements and also slicing information received from the participating gNB-CUs in an implementation specific way in section “7.7.2.3 Support of resource efficiency for RAN Sharing with multiple cell-ID broadcast”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - update in section “7.7.1 Support of dynamic PTP and PTM switching” and “7.7.3 Support of Multicast reception for UEs in RRC_INACTIVE” it should be for multicast MBS session; - update the condition for the shared F1-U resources is established and kept to “at least one PLMN keeping the broadcast MBS session”, in section “7.7.2.2 Support of resource efficiency for MOCN”. - update in section “7.7.2.2 Support of resource efficiency for MOCN” it should be for “broadcast MBS session” instead of “multicast service”. - clarify in section “7.7.2.3 Support of resource efficiency for RAN Sharing with multiple cell-ID broadcast”, that it is the “entity controlling the involved gNB-DUs sharing the same physical cell resources” who to resolve different QoS requirements and also slicing information received from the participating gNB-CUs in an implementation specific way. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - ambiguity exists in understanding which session type the feature is for. - error exists for the condition on the shared F1-U resources is established and kept is not correct. - error exists for the session type. - readability could be enhanced, and information missed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.1, 7.7.2.2, 7.7.2.3, 7.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

7.7	Support for NR MBS

The Support of NR MBS in non-split gNB case is specified in TS 38.300 [2].

### 7.7.1	Support of dynamic PTP and PTM switching

For UEs in RRC_CONNECTED, NG-RAN supports dynamic switch between PTP and PTM for multicast MBS session as specified in TS 38.300 [2].

In case of split gNB architecture, for a MRB with common PDCP involving both PTP (RLC leg) and PTM (RLC leg), upon receiving the MBS data from the gNB-CU via a shared F1-U tunnel, the gNB-DU makes decision of using PTP (RLC leg) or PTM (RLC leg) or both.

For UEs in RRC_INACTIVE only PTM is supported as specified in TS 38.300 for multicast MBS session [2].

### 7.7.2	Support of resource efficiency for RAN Sharing

#### 7.7.2.1	General

Resource efficiency for RAN sharing is supported in both MOCN scenario and Multiple Cell-ID Broadcast scenario.

#### 7.7.2.2	Support of resource efficiency for MOCN

Assistance Information to enable identifying broadcast MBS sessions for which RAN resources could be shared by the involved gNB-DUs and the gNB-CU-UP is provided by the gNB-CU-CP. Only one set of shared F1-U resources is established and kept as long as there is at least one PLMN keeping the broadcast MBS session.

#### 7.7.2.3	Support of resource efficiency for RAN Sharing with multiple cell-ID broadcast

gNB-DUs sharing the same physical cell resources receive via F1-C information to enable identifying broadcast MBS sessions providing identical content. The identification is based on Associated Session ID, for location dependent MBS services, the MBS Service Area is also taken into account.

While applying resource efficiency for RAN Sharing with multiple cell-ID broadcast:

-	the entity controlling the involved gNB-DUs sharing the same physical cell resources resolves different QoS requirements and slicing information received from the participating gNB-CUs in an implementation specific way.

-	F1-U resources are established towards either all involved gNB-CUs or only some of them which is decided by the entity controlling the involved gNB-DUs sharing the same physical cell resources. The gNB-DU is able to trigger the gNB-CU to establish F1-U resources.

-	the gNB-CU-CP takes into account the gNB-DUs decision of F1-U resources have been established or not to decide whether to establish NG-U resources.

### 7.7.3	Support of Multicast reception for UEs in RRC_INACTIVE

F1AP supports to enable and disable multicast reception for UEs in RRC_INACTIVE state for a specific multicast MBS session on cell level.

The gNB-CU decides whether multicast reception in RRC_INACTIVE state is applied or not.

<<<<<<<<<<<<<<<<<<<< Change  Ends>>>>>>>>>>>>>>>>>>>