3GPP TSG-RAN WG3 Meeting #123	R3-240124

# Athens, Greece, 26th February– 1st March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.415 | CR | 0040 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 38.415 PDU Set Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some fields which are mandatory and some fields which are optional, are mixing in one octet which has length of 0 or 1. More exactly, the EPDU and PSI (as mandatory IEs) and EDB as optional are mixing in an octet which has length of “0 or 1”. The procedure text describes that may include some mandatory fields i.e. the PDU Set Sequence Number, PDU Sequence Number within a PDU Set and PDU Set Importance, the “may include” is not correct”. The End of Data Burst (EDB) is defined as 3 bits however it does not align withe 26.522 which has 1 bit. The PDU Set Importance (PS) its description does not align with 26.522 which describe "when the sender cannot define importance, is 0". The PDU Set Size (PSSize), it is not clear whether the size is in byte or not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The EPDU and PSI (as mandatory IEs) to be in separate octect which is mandatory presence in the DL PDU SET INFORMATION (PDU Type 0) frame. To have a proper description in the procedure text for the inclusion of fields. For the End of Data Burst (EDB), to have 1 bit to align with 26.522. For the PDU Set Importance (PSI), to describe in a way to align with 26.522 For the PDU Set Size (PSSize), to have clarity that the size is in bytes. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) on the PDU Set Information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The fields of the DL PDU SET INFORMATION (PDU Type 0) is not correctly defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.4.1.1, 6.5.2.1, 6.5.3.8, 6.5.2.9, 6.5.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change part

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: "NextGen Radio Access Network (NG-RAN); Overall description; Stage 2".

[3]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[4]	3GPP TS 37.324: "E-UTRA and NR; Service Data Application Protocol (SDAP) specification".

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	IETF RFC 5905 (2010-06): "Network Time Protocol Version 4: Protocol and Algorithms Specification".

[7]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[8]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[x1]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

< skip unchanged part >

6	PDU Set Information user plane protocol

6.1	General

The PDU Set Information UP layer uses the services of the Transport Network Layer in order to send its packets over the interface.

6.2	PDU Set Information user plane protocol layer services

The following functions are provided by the PDU Set Information user plane protocol:

-	Provision of PDU Set information elements associated with a QoS flow.

-	Provision of Indication of End of Data Burst for a QoS flow.

6.3	Services expected from the Transport Network Layer

The PDU set information UP layer expects the following services from the Transport Network Layer:

-	Transfer of PDU Set Information User Plane PDUs.

6.4	Elementary procedures

6.4.1	Transfer of DL PDU Set Information

6.4.1.1	Successful operation

The purpose of the Transfer of DL PDU Set Information procedure is to send PDU Set information and indication of End of Data Burst related to a QoS flow from UPF to NG-RAN node or between NG-RAN nodes, or from gNB-CU to gNB-DU.

The DL PDU SET INFORMATION frame includes a QoS Flow Identifier (QFI) field associated with the transferred packet. The NG-RAN shall use the received QFI to determine the QoS flow and QoS profile which are associated with the received packet.

The DL PDU SET INFORMATION frame includes the PDU Set Information (i.e., PDU Set Sequence Number, PDU Sequence Number within a PDU Set and PDU Set Importance) as specified in TS 23.501 [5].

The DL PDU SET INFORMATION frame may include indication of End of Data Burst, PDU Set Size and End PDU of the PDU Set as specified in TS 23.501 [5].

Figure 6.4.1.1-1: Successful Transfer of DL PDU Set Information

Figure 6.4.1.1-2: Successful Transfer of DL PDU Set Information

6.4.1.2	Unsuccessful operation

Void.

6.5	Elements for the PDU Set Information user plane protocol

6.5.1	General

The structure of frames is specified as in the section 5.5.1.

6.5.2	Frame format for the PDU Set Information user plane protocol

6.5.2.1	DL PDU SET INFORMATION (PDU Type 0)

This frame format is defined to allow the NG-RAN to receive PDU Set Information and indication of End of Data Burst of a QoS flow.

The following shows the respective DL PDU SET INFORMATION frame.

| Bits |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 |  | 2 | 1 |  | 0 |  |
| PDU Type (=0) |  |  |  | Spare |  |  | PSSI |  | EDBI | 1 |
| Spare |  | QoS Flow Identifier (QFI) |  |  |  |  |  |  |  | 1 |
| PSSN |  |  |  |  |  |  |  |  |  | 2 |
| PSN |  |  |  |  |  |  |  |  |  | 1 |
| EPDU | spare |  |  |  | PSI |  |  |  |  | 1 |
| spare |  |  |  |  |  |  |  | EDB |  | 0 or 1 |
| PSSize |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Padding |  |  |  |  |  |  |  |  |  | 0-3 |

Figure 6.5.2.1-1: DL PDU SET INFORMATION (PDU Type 0) Format

6.5.3	Coding of information elements in frames

6.5.3.1	PDU Type

Description: The PDU Type indicates the structure of the PDU Set UP frame. The field takes the value of the PDU Type it identifies; i.e. "0" for PDU Type 0. The PDU type is in bit 4 to bit 7 in the first octet of the frame.

Value range: {0= DL PDU SET INFORMATION, 1-15=reserved for future PDU type extensions}.

Field length: 4 bits.

6.5.3.2	Spare

Description: The spare field is set to "0" by the sender and should not be interpreted by the receiver. This field is reserved for later versions.

Value range: (0–2n-1).

Field Length: n bits.

6.5.3.3	QoS Flow Identifier (QFI)

Description: When present this parameter indicates the QoS Flow Identifier of the QoS flow to which the transferred packet belongs.

Value range: {0..26-1}.

Field length: 6 bits.

6.5.3.5	PSSI (PDU Set Size indicator)

Description: This parameter indicates the presence of PDU Set Size (PSSize).

Value range: {0= PSSize not present, 1= PSSize present}.

Field length: 1 bit.

6.5.3.6	EDBI (End Data Burst indicator)

Description: This parameter indicates the presence of End of Data Burst (EDB).

Value range: {0= EDB not present, 1= EDB present}.

Field length: 1 bit.

6.5.3.7	End PDU of the PDU Set (EPDU)

Description: This parameter indicates whether the current PDU is the last PDU of the PDU set.

Value range: {0= all other PDUs of the PDU Set, 1= last PDU of the PDU set}.

Field length: 1 bit.

6.5.3.8	End of Data Burst (EDB)

Description: This parameter indicates the end of a Data Burst.

Value range: {0= all other PDUs of the PDU Burst, 1= last PDU of the Data Burst}.

Field length: 1 bit.

6.5.3.9	PDU Set Importance (PSI)

Description: This parameter indicates the importance of the current PDU Set compared to other PDU Sets within the same QoS flow. Higher values shall indicate a higher importance with the exception that value “0” means sender cannot define importance . PDU Set with the highest importance PDU Set is indicated by 15 and the lowest importance PDU Set is indicated by 1.

Value range: {0..24-1}.

Field length: 4 bits.

6.5.3.10	PDU Set Sequence Number (PSSN)

Description: This parameter indicates the sequence number of the PDU Set to which the current PDU belongs acting as an identifier for the PDU Set.

Value range: {0..216-1}.

Field length: 2 octets.

6.5.3.11	PDU Sequence Number within a PDU Set (PSN)

Description: This parameter indicates the sequence number of the current PDU within the PDU Set. The PSN shall be set to 0 for the first PDU in the PDU Set and incremented monotonically for every PDU in the PDU set in order of transmission from the sender.

Value range: {0..28-1}.

Field length: 1 octet.

6.5.3.12	PDU Set Size (PSSize)

Description: This parameter indicates the total size of all PDUs in bytes of the PDU Set to which the current PDU belongs.

Value range: {0..224-1}.

Field length: 3 octets.

6.5.4	Timers

Void.

6.6	Handling of unknown, unforeseen and erroneous protocol data

Void.

< skip unchanged part >