3GPP TSG-RAN WG3 Meeting #123								R3-240137

Athens, Greece, Feb, 26th – Mar, 1st 2014

Agenda Item:	9.1.7.1

Source:	NEC

Title:	Discussion on the issue of destination ID awareness for DU

Document for:	Decision

1	Introduction

Currently, the NR sidelink relay enhancement WID has been declared as completed. However, there are still some remaining issues for NR sidelink U2U relay. Therefore, in this contribution, we will further discuss and resolve those remaining issues.

2	Discussion

In legacy NR V2X, the reporting mechanism from sidelink UE is that UE needs to report the destination ID it needs to be maintained for different cast types via SidelinkUEInformation message when the UE is in RRC_CONNECTED state, so that network will further perform sequentially index of the destination ID into destination index. Later when UE perform BSR reporting it will fill the corresponding field with destination index so that network can be aware of which destination ID the UE is currently requesting resource for.

Furthermore, under CU-DU split architecture, CU would be the entity which receiving the SUI reporting from UE. However, DU would be the other entity which receiving SL BSR reporting from UE. If CU does not forward the relevant SUI information towards DU, DU then cannot understand the indication of destination index field within the SL-BSR, which is also explained in our companion CR[x][x][x][x][x].

So, it is proposed for NR SL U2U relay service, under CU-DU split architecture, CU needs to forward the destination ID reporting related information within SUI message of both U2U remote UE and U2U relay UE towards DU.

It is proposed for NR SL U2U relay service, under CU-DU split architecture, CU needs to forward the destination ID reporting related information within SUI message of both U2U remote UE and U2U relay UE towards DU.

According to RAN2 latest progress, the reporting behaviour for both U2U remote UE and U2U relay UE is not determined yet, i.e., it is not finalized that whether U2U remote UE/ U2U relay UE needs to report one or two destination IDs for both hops of U2U relay service. Also whether an indication is needed to associating two destination IDs if the two destination ID option is applied. Correspondingly, the signalling design in RAN3 cannot be made yet. Therefore, it is proposed that either RAN3 wait for RAN2’s progress or RAN3 trigger the LS exchange to get the latest progress of RAN2 related to the U2U ID reporting mechanism.

Regarding to the detailed signalling design, RAN3 can down select from the following two alternatives:

- RAN3 wait for RAN2’s progress

- RAN3 send the LS towards RAN2

3	Conclusion and Proposals

In this contribution, we have discussed the issue of NR V2X and SL relay under CU-DU split architecture, and the following observations and proposals are given out:

Proposal1.	It is proposed for NR SL U2U relay service, under CU-DU split architecture, CU needs to forward the destination ID reporting related information within SUI message of both U2U remote UE and U2U relay UE towards DU.

Proposal2.	Regarding to the detailed signalling design, RAN3 can down select from the following two alternatives:

-	RAN3 wait for RAN2’s progress

-	RAN3 send the LS towards RAN2

4	References