3GPP TSG-RAN WG3 #123								R3-240165

Athens, Greece, 26th Feb – 1st Mar 2024

Agenda Item:	9.1.9.1

Source:	China Telecom

Title: 	Support of non-inter DRX Cycles over F1 Interface

Document for:	Discussion

Introduction

The new short and long DRX cycles with non-integer periodicities were introduced in TS38.331. Therefore, it is need to discuss how to support DRX with non-integer periodicities.

Discussion

RAN2 had made the following agreements on short and long DRX cycles with non-integer periodicities:

RAN2 #123bis Agreements

- New DRX cycles in rational numbers are supported for both short and long DRX cycles.

- If short DRX cycle in rational number is configured, the length of the long DRX cycle shall be an integer multiple of the short DRX cycle, as in legacy.

- The new DRX parameter(s) for non-integer DRX cycles are common to both DRX groups

- At least use legacy formula and add floor () operation.

- We will have normative text to avoid rounding errors.

- specify the DRX cycle by different fields under a CHOICE structure and specify in the field description the correspondence between different fields and DRX cycles

RAN2#124 Agreements

1.	The following cycles are supported for short DRX cycle

ms1001/240, ms25over6, ms25over3, ms1001over120, ms100over9, ms125over9, ms50over3, ms1001over60, ms200over9, ms100over3, ms1001over30, ms125over3, ms1001over24, ms200over3

2.	The following cycles are supported for longDRX cycle (additional values requested by companies in red, additional values to handle multiple of short DRX cycle in blue):

ms1001/240, ms25over6, ms25over3, ms1001over120, ms100over9, ms125over9, ms50over3, ms1001over60, ms200over9, ms250over9, ms100over3, ms1001over30, ms125over3, ms1001over24, ms200over3, ms1001over15, ms250over3, ms1001over12, ms400over3.

Therefore, new DRX cycles in rational number for both short and long DRX cycles need to be introduced in CU/DU split architecture. Currently , the gNB-CU may signal the DRX cycle information which corresponds to the preferredDRX-LongCycle, preferredDRX-ShortCycl and preferredDRX-ShortCycleTimer contained in the UEAssistanceInformation message to the gNB-DU via UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST message. However, according to the following agreement in RAN2#124, the non-integer cycles will be not contained in UEAssistanceInformation message.

Agreements

1	RAN2 will not extend the preferred DRX cycle field in UEAssistanceInformation message for non-integer cycles.

Therefore, there is no need to extend the DRX Cycle IE in clause 9.3.1.24 to support non-integer cycles.

Proposal 1: there is no need to extend the DRX Cycle IE in clause 9.3.1.24 to support non-integer cycles.

In DU to CU RRC Information IE, the gNB-DU may signal the DRX Long Cycle Start Offset which corresponds to the drx-LongCycleStartOffset IE contained in the DRX-Config IE as defined in TS 38.331. Similarly, the DRX Long Cycle Start Offset for non-integer cycles corresponds to the drx-NonIntegerLongCycleStartOffset-r18 also need to be included in DU to CU RRC Information IE.

Proposal 2: the DRX Long Cycle Start Offset for non-integer cycles corresponds to the drx-NonIntegerLongCycleStartOffset-r18 also need to be included in DU to CU RRC Information IE.

Proposal

Based on the above analysis, we have the following proposal:

Proposal 1: there is no need to extend the DRX Cycle IE in clause 9.3.1.24 to support non-integer cycles.

Proposal 2: the DRX Long Cycle Start Offset for non-integer cycles corresponds to the drx-NonIntegerLongCycleStartOffset-r18 also need to be included in DU to CU RRC Information IE.

Reference

- TR 38.835 V2.0.0, Study on XR enhancements for NR, March 2023.

- Report of 3GPP TSG RAN WG2 meeting #123bis, Xiamen, China

- Report of 3GPP TSG RAN WG2 meeting #124, Chicago, USA