3GPP TSG-RAN WG3 #123                                                                        R3-240181

# Athens, Greece, 26th Feb – 1th Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 38.300 for QoE Measurement Continuity for Mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE_enh-Core |  |  |  |  |  | Date: |  |  | 2024-1-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For the handover from NR to LTE, the source NR node may not able to decide which QoE measurement can be continued. So, RAN3 agree to keep only one QoE measurement may be not feasible. While RAN2 decide to release all NR QoE configurations by NR node and then configure LTE QoE configuration by LTE node. RAN2’s solution is relatively reasonable. - QoE measurements continuity when switching between MBS multicast and unicast transmission modes is captured in stage2. However, there is another case on QoE Measurement Continuity when UE moving to a cell not providing the MBS broadcast. This case is captured in 16.10.6.5.1 subsection in TS 38.300:” This allows the UE, e.g., to request unicast reception of the service before moving to a cell not providing the MBS broadcast service(s) using PTM transmission”. QoE measurements for ongoing sessions should also be continued when switching between MBS broadcast and unicast transmission modes, and this case should also be captured in stage2 description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - On QoE measurement collection continuity for intra-system inter-RAT handover, capture RAN2 agreement: “release all NR QoE configurations by NR node and then configure LTE QoE configuration by LTE node” in stage2 description. - On QoE Measurement Continuity, capture the case of UE switching between MBS broadcast and unicast transmission modes in stage2 description. Impact Analysis: Impact assessment towards the previous version of the specification (same release): The impact can be considered isolated because only text is modified. The CR is backwards compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - QoE measurement collection continuity for intra-system inter-RAT handovers cannot be supported. - It is not supported that QoE measurements for ongoing sessions is continued when switching between MBS broadcast and unicast transmission modes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

21.3	QoE Measurement Continuity for Mobility

QoE measurement collection continuity for intra-system intra-RAT handover is supported, with the Area Scope parameters configured by the OAM, where the network is responsible for keeping track of whether the UE is inside or outside the area scope. A UE continues an ongoing QoE measurement even if it leaves the area scope, unless the network indicates to the UE to release the application layer measurement configuration.

For the handover, the source gNB may transmit the information related to one or more application layer measurement configurations of the UE to the target gNB via XnAP or NGAP. For signalling-based QoE, the service type indication, QoE reference, and, optionally, the MCE IP address, measurement configuration application layer ID, MDT alignment information, area scope, slice support list for QMC, available RAN visible QoE metrics and measurement status are passed to the target gNB. For management-based QoE, the service type indication, measurement configuration application layer ID, the MCE IP address and QoE measurement status are passed to the target gNB. For RRC_INACTIVE state mobility, QoE measurement configuration(s) of a specific UE can be retrieved from the gNB hosting the UE context when it resumes to the RRC_CONNECTED state.

For signalling-based QoE, at handover to a target gNB that supports QoE measurement collection, the target gNB decides which of the application layer measurement configurations should be kept or released, e.g., based on application layer measurement configuration information received from the source gNB in Xn/NG signalling.

The continuity of QoE measurement configuration and reporting in NR-DC scenario is supported as specified in TS 37.340 [21].

For QoE sessions pertaining to data flows received via MBS broadcast, QoE measurement collection may continue during the RRC_INACTIVE and RRC_IDLE.

Upon UE's transition from RRC_IDLE to RRC_CONNECTED, the gNB serving the UE should ensure that it does not release an already configured signalling-based QoE measurement configuration for the sake of configuring a new management-based QoE measurement configuration.

QoE measurements for ongoing sessions should be continued when switching between MBS multicast and unicast transmission modes or switching between MBS broadcast and unicast transmission modes.

When the UE resumes the connection with a gNB that does not support QoE, the UE releases all application layer measurement configurations.

QoE measurement collection continuity for intra-system inter-RAT handover is supported. For the handover from NR to LTE, UE should release all NR QoE configurations and apply LTE QoE configuration if received. For the handover from LTE to NR, UE should release all LTE QoE configurations and apply NR QoE configuration if received.

For intra-5GC handover from E-UTRA to NR, the UE releases the LTE QoE configuration if received from the source RAT and the UE applies NR QoE configuration(s) if received from the target RAT.

For intra-5GC handover from NR to E-UTRA, the UE releases all NR QoE configuration(s) if received from the source RAT, and the UE applies the LTE QoE configuration if received from the target RAT.

End of change