3GPP TSG-RAN WG3 Meeting #123	R3-240201

Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0332 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - CSI report configuration As defined in TS 38.331, the CU generates the LTM CSI resource configuration, which is common for all the LTM candidate cells. On receipt of the LTM CSI resource configuration, the source DU/candidate DU generates the LTM CSI report configuration separately within the ServingCellConfig IE for source cell and each LTM candidate cell. With the above statements, it can be understood that the CU does not need to send the LTM CSI report configuration for each LTM candidate cell to the source DU, and does not need to send the LTM CSI report configuration for the source cell or other LTM candidate cells to the candidate DU. But in current stage 2 specific, it was wrongly captured. - Early TA acquisition Source DU may receive early TA value list for some candidate cells. In subsequent-LTM, UE will continue keeping the LTM candidate configuration after completing cell switching. If source DU does not transfer the stored early TA value list, UE will be expected to get early TA value again. Therefore, for avoidance of unnecessary early TA acquisition by the target DU, the source DU may provide the stored early TA values of candidate cells using LTM Cell Change Notification message to the target DU via CU. - Single early RACH configuration In the RAN2 LS R3-240018, for each LTM candidate configuration, the network configures a configuration for early RACH, which is used regardless in which cell the UE receives the PDCCH order to trigger early RACH. However, RAN3#121bis agreed that the early RACH resources could be shared only among the UEs in a single DU to avoid the RACH access conflict between UEs from different DUs. A possible solution is: When a candidate DU provides early RACH resources to a source DU, it indicates the subset of RACH resources that the source DU can use. For example, the candidate DU2 indicates the preamble 0~4 to the source DU, while indicates the preamble 5~8 to the candidate DU1 for subsequent LTM. - Rachless indication According to RAN2 discussion, if the LTM is performed without RACH procedure, the target cell may need to perform dynamic scheduling and assigns a UL grant to UE to transmit the first UL message after LTM without RACH. Therefore, it is beneficial to let target DU know if a triggered LTM is with or without RACH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For intra-DU LTM: - Change the CSI report configuration to CSI resource configuration in step 5. - Remove the CSI report configuration in the NOTE 2. For inter-DU LTM: - Change the CSI report configuration to CSI resource configuration in step 5. - Remove the CSI report configuration in step 7. - Add the valid TA values of other candidate cells in other candidate DU(s) in the step 19 and step 20. - Update the description of step 14 and step 15. - Add the preamble index list of the RACH configuration in the step 4, step 5 and step 7. - Add Rachless indication in the step 19 and step 20. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has no impact with the previous version of the specification (same release) because it only impacts the LTM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Rel18 LTM will not work properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.4, 8.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.473 CR 1272 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of the Change-----------

8.2.1.4	Intra-gNB-DU LTM

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for LTM. Figure 8.2.1.4-1 shows the intra-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.4-1: Intra-gNB-DU LTM

1.	The UE sends a MeasurementReport message (L3 measurement result) to the gNB-DU containing measurements of neighbouring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate LTM configuration.

3.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU containing one target candidate cell ID, the LTM configuration ID of the candidate cell, and LTM configuration ID mapping list, the CSI resource configuration. The gNB-CU requests PRACH resources from the gNB-DU. The gNB-CU may request the gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration.

4.	If the gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations (e.g., TCI state configuration, RACH configuration, and the CSI report configuration) for the accepted target candidate cell.

NOTE 1:	Steps 3 and 4 may be initiated multiple times for LTM candidate cell preparation of multiple cells including the source cell.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the collected TCI state configurations and the gNB-CU may send the CSI resource configuration for all the accepted target candidate cells.

6.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the CSI report configuration of the source cell.

NOTE 2:	In case of subsequent LTM, the CU-initiated UE Context Modification procedure may be invoked per each candidate cell to transfer to the candidate gNB-DU the TCI state information, RACH configuration, and the LTM configuration IDs of the candidate cells.

7.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

8.	The gNB-DU forwards the received RRCReconfiguration message to the UE.

9.	The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

10.	The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

11.	Early synchronization may be performed as specified in TS 38.300 [2].

12.	The UE sends the L1 measurement result to the gNB-DU. The gNB-DU decides to execute LTM.

13.	The gNB-DU sends the Cell Switch command to the UE.

14.	The gNB-DU sends the DU-CU CELL CHANGE NOTIFICATION message to the gNB-CU to indicate the initiation of the Cell Switch command to the UE including the target cell ID and the TCI state ID.

15.	The gNB-DU detects the UE access as specified in TS 38.300 [2].

16.	The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

17.	The UE sends an RRCReconfigurationComplete message to the gNB-DU.

18.	The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

19.	The gNB-CU may send the UE CONTEXT MODIFICATION REQUEST message to the gNB-DU to release the resources of prepared cells.

20.	The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

---------------------------------------------------------Next change -------------------------------------------------------------

8.2.1.5	Inter-gNB-DU LTM

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for LTM. Figure 8.2.1.5-1 shows the inter-gNB-DU LTM procedure for intra-NR.

Figure 8.2.1.5-1: Inter-gNB-DU LTM

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate LTM configuration.

3.	The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s), containing one target candidate cell ID, the LTM configuration ID of the candidate cell, LTM configuration ID mapping list, and the CSI resource configuration. The gNB-CU indicates the source gNB-DU ID, and requests PRACH resources from the candidate gNB-DU. The gNB-CU may request the candidate gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration.

4.	If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configurations (e.g., TCI state configuration, RACH configuration with preamble index list, and the CSI report configuration) for the accepted target candidate cell.

NOTE 1:	The CU-initiated UE Context Modification procedure may be initiated for preparing candidate cells in the source gNB-DU as specified in step 3 and 4 in 8.2.1.4 Intra-gNB-DU LTM.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the collected CSI resource configuration, RACH configuration with preamble index list and the TCI state configuration for the accepted target candidate cell(s) in other gNB-DU(s).

6.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the CSI report configuration.

7.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the candidate gNB-DU(s) containing the TCI state information, RACH Configuration with preamble index list, and the LTM configuration IDs of the candidate cells in other candidate gNB-DU(s). The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s). The gNB-CU may also provide an updated CSI resource configuration to the candidate gNB-DU(s).

NOTE 2: The candidate cell may be the same cell as source cell.

8.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the updated lower layer configuration. The candidate gNB-DU may also respond the updated CSI report configuration.

9.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the LTM configuration.

10.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

11.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

12.	The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

13.	Early synchronization may be performed as specified in TS 38.300 [2].

14 - 15.	The candidate gNB-DU sends the TA value, the associated CFRA resource information, the candidate cell ID and the source gNB-DU ID to the source gNB-CU via the DU-CU TA INFORMATION TRANSFER. Then the gNB CU sends the TA value, the associated CFRA resource information and the candidate cell ID to the source gNB-DU via the CU-DU TA INFORMATION TRANSFER message.

16.	The UE sends the L1 measurement result to the source gNB-DU.

17.	The source gNB-DU decides to execute LTM to a candidate target cell.

18.	The source gNB-DU sends the Cell Switch command to the UE.

19.	The source gNB-DU sends the DU-CU CELL SWITCH NOTIFICATION message to the gNB-CU to indicate the initiation of the Cell Switch command to the UE, for which the message includes the target cell ID and the TCI state ID. The source gNB-DU may also provide the valid TA values of the candidate cells in other candidate gNB-DU(s). In addition, the source gNB-DU may also indicate if the triggered LTM is without a random access towards the target cell.

20.	The gNB-CU forwards the target cell ID and the TCI state ID to the target gNB-DU in the CU-DU CELL SWITCH NOTIFICATION message. The gNB-CU may also provide the valid TA values of the candidate cells in other candidate gNB-DU(s). In addition, the gNB-CU may also indicate if the triggered LTM is without a random access towards the target cell.

21.	The target gNB-DU detects the UE access as specified in TS 38.300 [2].

22.	The target gNB-DU sends the ACCESS SUCCESS message to the gNB-CU with the target cell ID.

23.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

24.	The target gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

25.	The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

26.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

--------------End of the Changes-----------