3GPP TSG-RAN WG3 #123	R3-240230

Athens, Greece, 26 Feb. – 1 Mar. 2024

Title: 	Discussion on MBS RAN sharing leftover issues

Source: 	Samsung

Agenda item:	9.1.6.1.

Document Type:	Discussion

1. Introduction

The documents discuss the small corrections in E1 and F1 for the MBS RAN sharing.

2. Discussion

2.1 MOCN leftovers

Misalign between SA2 agreements and RAN3 agreements

In TS23.347 i40, describe when the NG-RAN initiates the NG-U setup in case of the RAN sharing, as showed in below:

The procedure in this clause is performed when one of the following events occur and resource sharing across broadcast MBS Sessions in network sharing as specified in clause 6.18 is applied: - When a broadcast MBS session, from which the shared NG-RAN receives a DL MBS data stream, is released, and the NG-RAN node determines to request to receive an DL MBS data stream from another operator's CN with a remaining broadcast MBS session. - When the shared NG-RAN fails to receive DL data stream from an operator´s CN (e.g. due to failure in the user plane) and the NG-RAN node determines to request to receive the DL MBS data stream via another operator's CN. Figure 7.3.7-1: Transport change for resource sharing across broadcast MBS Sessions during network sharing 1. The NG-RAN selects another broadcast MBS Session that is delivering the same content via another CN to establish user plane, utilizing the broadcast MBS session context stored in the NG-RAN. If unicast transport of N3mb applies, continue at step 3. NOTE: Which CN is to be selected is up to NG-RAN implementation. 2. If multicast transport of N3mb applies, the NG-RAN joins the multicast group towards the LL SSM provided by the CN and continues at step 8. 3. If unicast transport of N3mb applies, the NG-RAN allocates N3mb DL Tunnel Info, and sends N2 message (e.g. BROADCAST SESSION TRANSPORT REQUEST) to the AMF, including the MBS Session ID, the N3mb DL Tunnel Info and optional Area session ID.

Regarding the second scenario, in line with the SA2 agreement, the CU-UP should be able to inform the CU-CP of any failures in the user plane. This allows the CU-CP to initiate the setup of an alternative N3mb tunnel through a different CN, ensuring transmission continuity.

A previously raised issue highlighted that the NG-RAN may be unaware of user plane failures. Considering the obvious discrepancy between SA2 and RAN3 agreements, there are two potential solutions:

- RAN3 may choose not to support this scenario, requesting SA2 to exclude it.

- NG-RAN could monitor data transmission status, inferring potential user plane issues if there's a lack of downlink data stream for a certain period. This period might be based on implementation. In multiple cell broadcasting scenarios, the DU could detect transmission issues and initiate the F1 transport resource setup to an alternative CU-UP, leading to the establishment of a new NG-U interface.

Proposal 1: Introduce a user plane failure IE in the BC Bearer Context Modify Required message from gNB-CU-UP to gNB-CU-CP. If agreed, we are prepared to draft the CR. Alternative solution is to LS SA2 to remove this scenario to align with RAN3.

2.2 Multiple Cell broadcasting leftovers

According to SA2 agreements, one session may be released earlier than another, for example, CN1 for TMGI 1 sending a BROADCAST SESSION RELEASE REQUEST message to CU-CP1. This message indicates that TMGI1 is released earlier than TMGI 2. CU-CP1 then forwards this release request to the DU and the CU-UP1. Then the DU sends a Broadcast Transport Resource Setup Request message to CU-CP2 for establishing the necessary resources for a new F1 path with CU-UP2.

The detailed signaling flow for this procedure is depicted in the above Figure. Steps 1-8 involve setting up the broadcast session for TMGI1. Steps 9-16 are for setting up procedure for TMGI2. Steps 17-22 cover the release of TMG11, and steps 23-29 are for establishing a new F1-U and NG-U for TMGI2.

We've discussed the PDCP discard issue that arises when the F1-U path changes. The PDCP-SN on the new and old paths can differ, leading to data on the new path falling outside the UE's receiving window. We've considered three methods but haven't reached a consensus:

- Using the same MRB configuration on the new path as the old one, ensuring PDCP SN continuity on the new F1-U path. This requires the gNB-DU to inform the gNB-CU-CP of the first PDCP-SN to use, which is then forwarded to the gNB-CU-UP.

- Implementing a different MRB configuration on the new path. That is the lower layer configuration controlled by the DU is reconfigured on the new path.

- Leaving the solution to individual implementations, which may result in packet drops.

The second method is essentially the MRB reconfiguration process. As per TS 38.331, the manner in which a UE executes MRB reconfiguration is dependent on the UE's implementation. Typically, changing the protocol configuration does not reestablishing the PDCP entity on the UE side. Instead, PDCP reestablishment should be accomplished through MRB release and addition. However, currently, the MBSBroadcastConfiguration message in the MCCH does not support MRB release and addition. This is because there is no MRB ID included in the MCCH message. Without PDCP reestablishment, it's impossible to reset the PDCP receiving parameters, such as RX_DELIV and RX_NEXT, to their initial values. Consequently, the problem of packet discard cannot be solved by using the second method if the UE does not perform PDCP reestablishment during the MRB reconfiguration.

The first approach does not affect the Uu interface; instead, it involves defining a new IE in both F1AP and E1AP protocols. This method is simpler than modifying the Uu interface. While Method 3 is not the ideal solution, it becomes the only viable option if we choose not to make any changes, albeit at the cost of tolerating data loss.

Proposal 2: RAN3 to choose a method to address the PDCP discard issue in multiple cell broadcasting scenario. Suggest to use of the same PDCP configuration and PDCP SN on the new path.

In step 23, the gNB-DU initiates a new F1-U and NG-U setup by sending a BROADCAST TRANSPORT RESOURCE REQUEST to gNB-CU-CP2. In step 10, the bearer context for TMGI2 has been established, but the F1-U for TMGI2 is not yet in place as indicated in the notification to the gNB-CU-UP2 in step 14. It's unclear whether gNB-CU-UP2 releases the MBS bearer context, such as MRB configuration, or keeps it and just releases the F1-U tunnel resource upon receiving an 'F1-U tunnel not established' indication. The following cases are considered:

- gNB-CU-UP2 and gNB-DU may retain all resources, including MRB ID and F1-U tunnel resource. Upon receiving a BROADCAST TRANSPORT RESOURCE REQUEST, gNB-CU-CP2 sends a BROADCAST BEARER MODIFICATION REQUEST to gNB-CU-UP2, including BC MRB To Modify List IE with MRB ID. The downside is resource wastage, as both gNB-CU-UP and gNB-DU reserve F1-U tunnel resources even if the Pl-U tunnel isn't set up. This approach also requires clear specification of reservation behaviour to avoid interoperability issues, hence it's not preferred.

- gNB-CU-UP2 and gNB-DU could not to retain F1-U tunnel resources but only the bearer context, such as MRB ID and its configuration. Upon receiving a BROADCAST TRANSPORT RESOURCE REQUEST, gNB-CU-CP2 sends a BROADCAST BEARER MODIFICATION REQUEST to gNB-CU-UP2, including BC MRB To Modify List IE with MRB ID and possibly BC Bearer Context F1-U TNL Info at DU IE for setting up a new F1-U tunnel. This method is more flexible and addresses the limitations of the first approach. To streamline the signaling, gNB-DU can include the F1-U TNL@DU in the BROADCAST TRANSPORT RESOURCE REQUEST message, thereby avoiding an additional modification procedure.

- If gNB-CU-UP2 and gNB-DU release all resources, including MRB ID and F1-U tunnel resource, when aware of F1-U tunneling not being set, gNB-CU-CP2, upon receiving a BROADCAST TRANSPORT RESOURCE REQUEST, sends a BROADCAST BEARER MODIFICATION REQUEST to gNB-CU-UP2. This includes BC MRB To Setup List IE with MRB ID IE, MRB PDCP Configuration IE, and MBS QoS Flows Information To Be Setup IE. This method is not very realistic. Before the gNB-CU-UP receives a Bearer Release Request message, the gNB-CU-UP at least need to store the MRB ID for this MBS bearer.

By the above comparison, the second approach is preferred.

Proposal 3:	For multiple cell broadcasting scenario, the DU initiates BROADCAST TRANSPORT RESOURCE REQUEST message including F1-U TNL information IE. The gNB-CU-CP includes it in the BC MRB To Modify List IE in BROADCAST BEARER MODIFICATION REQUEST message sending to the gNB-CU-UP.

Proposal 4:  It is proposed to agree the corresponding CR for proposal 3 and proposal 3 in R3-240231.

3 Proposals

In this document, the minor left issues were discussed, and it is proposed:

Proposal 1: Introduce a user plane failure IE in the BC Bearer Context Modify Required message from gNB-CU-UP to gNB-CU-CP. If agreed, we are prepared to draft the CR. Alternative solution is to LS SA2 to remove this scenario to align with RAN3.

Proposal 2: RAN3 to choose a method to address the PDCP discard issue in multiple cell broadcasting scenario. Suggest to use of the same PDCP configuration and PDCP SN on the new path.

Proposal 3:	For multiple cell broadcasting scenario, the DU initiates BROADCAST TRANSPORT RESOURCE REQUEST message including F1-U TNL information IE. The gNB-CU-CP includes it in the BC MRB To Modify List IE in BROADCAST BEARER MODIFICATION REQUEST message sending to the gNB-CU-UP.

Proposal 4:  It is proposed to agree the corresponding CR for proposal 2 and proposal 3 in R3-240231.

- Reference

[1] TS38.473 V18.0.0

[2] TS37.483 V18.0.0

[3] TS 38.413 V18.0.0