3GPP TSG-RAN3 Meeting #123	R3-240250

# Athens, Greece, 26th Feb – 1st Mar 2024

Agenda item:	9.1.9.1

Source:	Nokia, Nokia Shanghai Bell, ZTE, CMCC

Title:	Discussion on PDU Set Discard for NR XR

Document for:	Discussion and Decision

# 1	Introduction

This contribution analyses the remaining issues to support PSI based discard.

# 2	Discussion

Last RAN3 meeting did not discuss the detail on How to support PDU set discard. RAN2#123bis agreed following:

Agreements

- We will use a discard timer mechanism for the low importance PDU set.  We will allow a value of zero for the timer.    The running discard timers are not changed.

- It is up to UE implementation to determine which PSI levels will apply the discard mechanism

- the gNB signals an activation/deactivation indication (e.g. when congestion situation is detection)

- activation/deactivation is signaled using an ON/OFF mechanism on a per UE basis.  Introduce new MAC CE.

RAN2#124 agreed following:

Agreements

1	The initial state of the PSI-Based PDU Discard Activation/Deactivation MAC CE is deactivated.

Agreements:

- In the MAC CE for the activation/deactivation of the PSI-based discard, introduce a bitmap for DRB to efficiently control multiple DRBs separately and simultaneously.

After the gNB-CU configures the UE with the PSI based discard timer, the gNB-DU may need to know it, so the gNB-DU can send MAC CE later for activation/deactivation. For a UE not configured with PSI based discard timer, the gNB-DU does not need to send MAC CE for activation/deactivation.

Observation 1: gNB-DU need to know when UE is configured with PSI based discard timer.

For example, during the UE initial access procedure, the gNB-CU may provide an indication to gNB-DU when the gNB-CU send a RRCReconfiguration including the PSI based discard timer to UE (Step 14).

Proposal 1: Introduce a new indication in F1AP RRC MESSAGE TRANSFER to inform the gNB-DU when gNB-CU configure the PSI based discard timer.

The proposed TS38.473 CR can be found at ([2]).

# 3	Conclusion

In this contribution, we briefly analyzed the required enhancements in F1 to support the PSI based discard. Our proposals are:

Observation 1: gNB-DU need to know when UE is configured with PSI based discard timer.

Proposal 1: Introduce a new indication in F1AP RRC MESSAGE TRANSFER to inform the gNB-DU when gNB-CU configure the PSI based discard timer.

The proposed TS38.473 CR can be found at ([2]).

# References

- RP-230786, WID for NR XR

- R3-240251, (CR for TS38.473) Support for PSI based discard, Nokia, Nokia Shanghai Bell, CMCC, ZTE