3GPP TSG-RAN WG3 Meeting #123	R3-240255

Athens, Greece, 26th Feb – 1st Mar 2024

Agenda item:	 9.1.1.1

Source:		Samsung

Title:	 Leftover issues on SON for CPAC

Document for:		Discussion and Decision

1	Introduction

On SON for CPAC, some open issues were left due to lack of time, as summarized in R3-237786.

This contribution discussed the open issues and the related stage 3 impacts.

3	Discussion

Regarding wrong PSCell change/addition, there are two sub cases: 1) the wrong candidate cell comes from the cell in the list provided by the initiating node or 2) the wrong candidate cell is selected by the candidate or target SN.

In case of 2), the target SN is responsible for optimizing the down selection of the candidate cells indicated by the initiating node.

The candidate or target SN needs to know the following information in order to make the corresponding optimisation:

- The candidate PSCell list recommended by the initiating node

- The candidate PSCell list selected by the candidate or target SN.

For the SCG failure after successful PSCell addition/change, the candidate SN has released the UE context because the UE doesn’t access to this candidate SN. The candidate SN cannot get the above information from the UE context.

With this reason, the candidate PSCell list recommended by the initiating node has been included in the SCG Failure Information Report message from the MN to the candidate SN. Without the “The candidate PSCell list selected by the candidate SN”, the candidate SN cannot make the corresponding optimisation.

In order to let the candidate SN know the candidate PSCell list selected by the candidate SN, Indication whether a candidate PSCell is selected by the candidate target is needed. Because the candidate PSCell list selected by the candidate target is the subset of the candidate PSCell list recommended by the initiating node, there is no need to transmit a second PSCell list.

Proposal 1: Include “Indication whether a candidate PSCell is selected by the candidate target” in SCG FAILURE INFORMATION REPORT from the MN to the candidate SN.

During RAN3#122 meeting, three options were discussed regarding which node select the suitable PSCell:

Option 1:

It is always initiating node, i.e. MN in case of MN initiated and S-SN in case of SN initiated PSCell change/CPC.

Option 2:

It is always MN, both for MN initiated and SN initiated PSCell change/CPC.

Option 3:

It is MN in case of MN initiated and MN or S-SN in case of SN initiated PSCell change/CPC.

Since it is the initiating node to configure the measurement for the UE, the initiating node is the proper node to decide the suitable PSCell based on the measurement result from the UE.

Proposal 2: It is always initiating node, i.e. MN in case of MN initiated and S-SN in case of SN initiated PSCell change/CPC to decide the suitable PSCell.

The next questions are

- whether the suitable PSCell ID should be included in the SCG FAILURE INFORMATION REPORT message from the MN to the (candidate) target SN and

- whether the suitable PSCell ID should be included in the SCG Failure Transfer message from S-SN to MN in case of SN initiated PSCell change

There are two benefits for the candidate SN to know the suitable PSCell ID:

- The candidate SN can finally decide that the the wrong candidate cell is selected by the candidate SN;

- The candidate SN can assure the suitable PSCell ID is in the list of candidate PSCell list selected by the candidate SN for future configurations.

Proposal 3: Include Suitable PSCell ID in SCG FAILURE INFORMATION REPORT from the MN to the candidate SN.

Proposal 4: Include “Suitable PSCell ID” in SCG FAILURE TRANSFER from the source SN to the MN.

The CR for TS38.423 is provided in [2] to capture the Proposal 1, Proposal 3 and Proposal 4.

Proposal 5: Agree the CR for TS38.423 in [2].

3	Conclusion

This contribution discussed the leftover issues on SON for CPAC. We have the following proposals. The CR for TS38.423 are provided in [2].

Proposal 1: Include “Indication whether a candidate PSCell is selected by the candidate target” in SCG FAILURE INFORMATION REPORT from the MN to the candidate SN.

Proposal 2: It is always initiating node, i.e. MN in case of MN initiated and S-SN in case of SN initiated PSCell change/CPC to decide the suitable PSCell.

Proposal 3: Include Suitable PSCell ID in SCG FAILURE INFORMATION REPORT from the MN to the candidate SN.

Proposal 4: Include “Suitable PSCell ID” in SCG FAILURE TRANSFER from the source SN to the MN.

Proposal 5: Agree the CR for TS38.423 in [2].

Reference

[1] R3-237786, Summary of Offline Discussion on MRO, Nokia (moderator)

[2] R3-240256, Correction of SON for CPAC, Samsung