3GPP TSG-RAN WG3 Meeting #123	R3-240257

Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | draftCR | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SDT RRC Release with resume indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Qualcomm Incorporated, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Release 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In R18 TEI, RAN2 feedback in R2-2313759 that it was agreed that a resume indication can be provided in RRCRelease message to terminate SDT and trigger UE to initiate another RRC resume procedure to transit to RRC CONNECTED state, in case the non-SDT data arrival or large size SDT data arrival during the ongoing SDT without anchor relocation, as specified in TS 38.331： resumeIndication Indicates that the UE shall trigger the RRC connection resume procedure after receiving this RRCRelease message, as specified in clause 5.3.8.3. The network only includes this field in the RRCRelease message used to terminate an ongoing SDT procedure. However, the above is not reflected in the RAN3 defined SDT call flows in TS 38.300. With this, the gNB does not have to move the UE to RRC_Inactive in the related scenarios, but in current spec, the gNB has to move the UE to RRC_Inactive or RRC_Idle. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture RRCRelease message with resume indication in the stage 2 procedure for SDT without anchor relocation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Overall procedure incorrect and incomplete for SDT without UE Context Relocation in case of non SDT or large SDT data arrival. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.401 CR 0335 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============================  The Start of Change =======================================

## 18.3	SDT without UE context relocation

The overall procedure for SDT procedure over RACH without UE context relocation is illustrated in the figure 18.3-1.

Figure 18.3-1. RA-based SDT without UE context relocation

1/2. The steps 1/2 are as defined in steps 1/2 in Figure 18.2-1.

3.	The last serving gNB decides not to relocate the full UE context for SDT.

4.	The last serving gNB transfers a partial UE context including the SDT related RLC context.

5.	The receiving gNB acknowledges receiving the partial UE context and provides associated DL TNL address. The UE context is kept at the last serving gNB and the SDT related RLC context is established at the receiving gNB. Then UL/DL GTP-U tunnels are established for DRBs configured for SDT, if any, and the UL SDT data and/or signalling, if any, are forwarded to the last serving gNB, and then delivered to the core network.

NOTE 1:	The DL signalling from the last serving gNB, if any, is forwarded to the receiving gNB via the RRC TRANSFER message, for which the receiving gNB delivers it to the UE.

NOTE 1a:	In case DL non-SDT data or DL non-SDT signalling arrives, or UE assistance information (i.e. UL non-SDT data arrival indication) is received from the UE, the last serving gNB terminates the SDT procedure and directs the UE either to continue in RRC_INACTIVE state by sending the RRCRelease message without resume indication, or to initiate RRC resume procedure to move to RRC_CONNECTED state by sending the RRCRelease message with resume indication.

NOTE 1b:		Based on (e.g. large size of) DL SDT data or DL SDT signalling, the last serving gNB may terminate the SDT procedure and direct the UE either to continue in RRC_INACTIVE state by sending the RRCRelease message without resume indication, or to initiate RRC resume procedure to transition to RRC_CONNECTED state by sending the RRCRelease message with resume indication.

6.	The receiving gNB detects the end of SDT session and sends the RETRIEVE UE CONTEXT CONFIRM message including whether this is a "normal" end of SDT transaction or a radio link problem, or large SDT volume from BSR.

7.	Upon receiving the RETRIEVE UE CONTEXT CONFIRM message and deciding to terminate the SDT, the last serving gNB responds to the receiving gNB with the RETRIEVE UE CONTEXT FAILURE message including an encapsulated RRCRelease message. The receiving gNB shall release the established partial UE context.

NOTE 2:	Void.

NOTE 3:	Void.

8.	The receiving gNB sends the RRCRelease message to the UE.

NOTE 4:	Void.

NOTE 5:	The last serving gNB may terminate the SDT procedure and direct the UE either to continue in RRC_INACTIVE state by sending the RRCRelease message without resume indication, or to initiate RRC resume procedure to transition to RRC_CONNECTED state by sending the RRCRelease message with resume indication upon receiving the indication about large uplink SDT data from the BSR from the receiving gNB in step 6.

9.	The UE moves to RRC_INACTIVE state if the suspend indication is included in the RRCRelease message. If resume indication is included in the RRCRelease message, the UE shall initiate RRC resume procedure to transition to RRC_CONNECTED state. Or else, the UE moves to RRC_IDLE state.

==============================  The Start of Change =======================================