3GPP TSG-RAN WG3 Meeting #123	R3-240394

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda Item:	9.3.2

Source:	Huawei, Lenovo, Samsung, Xiaomi, Ericsson

Title:	Discussion on IAB IP address allocation via OAM

Document for:	Discussion

1	Introduction

In last RAN3-122 meeting, two CRs (R3-237361 and R3-237362) were submitted to solve the issue related to OAM based IAB IP address allocation. The intention is to allow the IAB-node to know the BAP address of the donor DU to which the assigned IP address is associated. There is no consensus on whether to allow the change proposed by the two CRs. And, during the discussion, some companies propose another solution: If the IAB-node’s IP address is configured by the OAM, the OAM should provide the BAP address of the anchor IAB-donor-DU with each IP address to the IAB-node.

In this paper, we continue to discuss this issue and analyse the potential works for the two solutions.

2	Discussion

In Rel-16, considering that the source IP address filtering may be deployed at the IAB-donor-DU as well as the transport network nodes, the IAB-node should determine which source IP address to be used based on the UL mapping configuration. The suitable procedure is: the IAB-node first obtains the target donor-DU’s BAP routing ID according to the UL mapping, and then select the IP address anchored on that donor-DU. This procedure requires that the IAB-node knows the mapping between the IP address and the BAP address of the corresponding IAB-donor-DU.

The IAB-node may obtain IP address(es) either from the IAB-donor or from the OAM system. For the donor-based IP allocation, the donor-CU sends the IP address(es) to the IAB-node together with the BAP address of the corresponding donor-DU.

However, for the OAM-based IP allocation, it is unclear how can the IAB-node know the relationship between the allocated IP address and the associated BAP address of IAB-donor-DU. Then the source IP address cannot be selected properly for UL traffic. In the worst case, some UL packets without proper source IP address may be dropped by the donor-DU in case the source IP filtering is configured.

The two solutions on table will be introduced as following.

Solution 1: IAB-donor-CU provides the mapping realtionsip to the IAB-node

To solve this issue, the simplest way, is that the IAB-donor-CU provides the mapping between donor DU’s BAP address and the IP address allocated to the IAB-node, after receiving the reported IP address(es) of the IAB-node from the IAB-MT via RRC. No stage-3 change is necessary for such method since all the necessary RRC signalings already been supported in Rel-16. The only change needed is to add stage 2 description for the OAM based IP address allocation as the R3-237361.

----------Excerpt from R3-237361----------

8.9.13	IP Address Allocation for IAB-nodes

An IAB-node may obtain IP address(es) either from the IAB-donor or from the OAM system. The IP address(es) is(are) used by the IAB-node for F1 and non-F1 traffic exchange via the backhaul. In case IPsec tunnel mode is used to protect this F1 and non-F1 traffic, the IP address(es) refer to the outer tunnel addresses. The allocation of the inner tunnel IP address(es) is outside of 3GPP scope.

NOTE:	The non-F1 traffic of an IAB-node includes all IP traffic that is not used for the management or transport of F1-C as specified in TS 38.472 [26] or F1-U as specified in TS 38.474 [7]. The non-F1 traffic may include, e.g., OAM traffic if it is transferred using the BH RLC channel.

In case of IAB-donor-based IP address allocation, the IP address(es) is(are) allocated by the IAB-donor-CU or IAB-donor-DU. In both cases, the IAB-node requests the IP address(es) via RRC from the IAB-donor-CU. It includes a separate IP address request for each usage, where the usages defined are all traffic, F1-U, F1-C and non-F1. The IAB-donor-CU may initiate the IAB TNL Address Allocation procedure to obtain IP addresses from the IAB-donor-DU. The IAB-donor-CU sends the IP addresses allocated for each usage to the IAB-node via RRC.

The IAB-node may be allocated one or multiple IPv6 addresses or one 64-bit IPv6 prefix for each usage and/or one or multiple IPv4 addresses for each usage. Each allocated IP address/IPv6 prefix is unique within the IAB network and routable from the wireline network.

In case of OAM-based IP address allocation, the IAB-node informs the IAB-donor-CU via an UL RRC message about the IP address(es) it received for each purpose. Then the IAB-donor-CU sends the DL RRC message to the IAB-node including the mapping of each IP address and the corresponding donor-DU’s BAP address. This occurs before the IAB node uses the IP address(es) for UL and/or DL traffic.

…

Solution 2: OAM provides the mapping realtionsip to the IAB-node

This solution is proposed during the online discussion in RAN3-122. The main idea is: if the IAB-node’s IP address is configured by the OAM, the OAM should provide the BAP address of the anchor IAB-donor-DU with each IP address assigned to the IAB-node.

However, the existing SA5’s Network Resource Model specification does not support providing the BAP address of IAB-donor-DU for the IAB-node. As excerpted from TS 28.541, the parameter of F1-U/C Address only includes the “localAddress” and the “remoteAddress”, there is no information about the IAB-donor-DU.

----------Excerpt from TS 28.541----------

4.3.12.2	Attributes

The EP_F1C IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

Moreover, the BAP address of IAB-node and IAB-donor-DU is allocted by the IAB-donor-CU in R16. Then, another issue for this solution 2 is how to enable the OAM of the IAB-node know the BAP address of the IAB-donor-DU.

- If the OAM is responsible for the BAP address allocation, the OAM of IAB-node should know the whole topology of the access network of an IAB-donor. Such method is totally different from the R16 assumption of BAP address management, and, we are not sure whether it is feasible or not, since this requires efforts from SA5.

- If the BAP address is still allocated by the IAB-donor-CU, it is unclear how can the OAM of IAB-node to communicate with the IAB-donor-CU to obtain the BAP address of IAB-donor-DU. SA5 should be involved for the discussion also.

Observation 1: If solution 1 is adopted, the only change needed for OAM based IP address allocation is the simple stage 2 change for TS38.401.

Observation 2: If solution 2 is adopted, the following two issues should be solved in collaboration with SA5:

- A. Whether and how can the OAM of IAB-node know the BAP address of IAB-donor-DU?

- B. Whether the OAM of IAB-node can provide the BAP address of IAB-donor-DU when assign IP address to the IAB-DU?

The potential change for solution 1 is provided in [1]. For solution 2, it is unclear whether and how the related issues to be solved, at least, we should ask SA5 to ask their viewpoint if someone still prefer this solution 2. And we propose

- RAN3 to adopt option 1 for OAM based IP address allocation to IAB-node. If there is no consensus, RAN3 to send LS to SA5 to check their viewpoint.

3	Conclusion

This document discuss the two solutions for OAM based IP address allocation, the following observations and proposals are provided.

Observation 1: If solution 1 is adopted, the only change needed for OAM based IP address allocation is the simple stage 2 change for TS38.401.

Observation 2: If solution 2 is adopted, the following two issues should be solved in collaboration with SA5:

- A. Whether and how can the OAM of IAB-node know the BAP address of IAB-donor-DU?

- B. Whether the OAM of IAB-node can provide the BAP address of IAB-donor-DU when assign IP address to the IAB-DU?

Proposal 1:	RAN3 to adopt option 1 for OAM based IP address allocation to IAB-node. If there is no consensus, RAN3 to send LS to SA5 to check their viewpoint.

4	Reference

- [1] R3-240395 Correction on IP address allocation for IAB via OAM (revision of R3-237361)

Annex1	LS to SA5 for solution 2.

3GPP TSG-RAN WG3 Meeting #123	R3-24xxxx

Athens, GR, 26 Feb – 01 Mar, 2024

Title:	[DRAFT] LS on OAM based IP address allocation for IAB

Response to:

Release:	Rel-16

Work Item:	NR_IAB

Source:	Huawei [will be RAN3]

To:	SA5

Cc:

Contact person:	Yuanpingzhu

zhuyuanping@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

In Rel-16 IAB, it is assumed that each IAB-node can obtain one or more IP address from IAB-donor-CU or from OAM, and should know the BAP address of the IAB-donor-DU with which the IP address is associated. The BAP address of each node is unique in network served by one IAB-donor-CU, it is assigned to the IAB-node or IAB-donor-DU by the IAB-donor-CU.

The issue for R16 IAB is: for the OAM based IP address allocation, the IAB-node is not able to get the association of IP address and the IAB-donor-DU’s BAP address. RAN3 discussed this issue and has the following two solutions on table:

Solution 1: IAB-donor-CU provide the association relationship between IP address and the BAP address of IAb-donor-DU to the IAB-node, after receiving the IP address information report from the IAB-node.

Solution 2: When OAM providing IP address to the IAB-node, the associated BAP address of IAB-donor-DU is included.

For solution 1, all stage 3 signaling already been supported by existing RAN specidication, the only necessary change is for stage 2 description in TS 38.401 which is managed by RAN3.

For solution 2, the consequent issue is how can the OAM of IAB-node know the BAP address of the IAB-donor-DU, RAN3 believe that solving such issue should require great efforts from SA5.

RAN3 would like to ask SA5 to evaluate the necessity and feasibility of solution 2 and provide feedback if any.

2	Actions

To SA5

ACTION: 	RAN3 respectively asks SA5 to evaluate the necessity and feasibility of solution 2 and provide feedback if any.

3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#123-bis	2024-04-15 - 2024-04-19 	China, CN

RAN3#124	2024-05-20 - 2024-05-24		Japan, JP