3GPP TSG-RAN WG3 Meeting #123	R3-240407

Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1306 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of SSBs activation and deactivation for network energy saving |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, ZTE, CATT, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR proposes the following changes. - The SSBs within the cell to be Activated List IE is included in the gNB-CU configuration update message to request to activate these SSB beams. In the ASN.1, this IE was included under the Cells-to-be-Activated-List-Item, which is also used in the F1 setup response message, and the GNB-DU Configuration update acknowledge message. This not only leads to misalignment between the ASN.1 and the Tabular, but also hints this IE can be used in the other two F1AP messages. - In the Tabular, the Cells with SSBs Activated List IE is optional, but should be mandatory in the GNB-CU configuration update acknowledge message. - The Cells Allowed to be Deactivated List IE is contained in the GNB-CU configuration update message. In ASN.1, the Cells-Allowed-to-be-Deactivated-List-ItemIEs should be mandatory but is optional since this is a single container. - The Coverage Modification Cause IE is included in the Coverage Modification Notification IE associated with NR CGI. But in ASN.1, the id-Coverage-Modification-Cause is included under SSBCoverageModification-Item. - In the Tabular, the Recommended SSBs for Paging List Item IE is optional, but should be mandatory. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the SSBs within the cell to be Activated List IE, and add the Cells with SSBs to be Activated List IE in the gNB-CU CONFIGURATION UPDATE message. - Update the Cells with SSBs Activated List IE in the Tabular to be mandatory. - Update the Cells-Allowed-to-be-Deactivated-List-Item IE in the ASN.1 to be mandatory. - Change the Coverage-Modification-Cause under Coverage-Modification-Item in ASN.1. - Update the Recommended SSBs for Paging List Item IE in the Tabular to be mandatory. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors in the tabular and the ASN.1 as indicated above. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.5.2, 9.2.1.10, 9.2.1.11, 9.3.1.297, 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specification |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included.

If the Cells with SSBs to be Activated List IE is included in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, only activate those SSB beams indicated by the SSB Index IE for the cell indicated by the NR CGI IE.

If at least one requested SSB beam in the Cells with SSBs to be Activated List IE is activated, the gNB-DU includes the Cells with SSBs Activated List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message. The gNB-CU shall consider that the SSB beams indicated by the SSBs activated List IE as activated.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| Cells to be Deactivated List |  | 0..1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| gNB-CU TNL Association To Add List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Add Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Information | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | M |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| gNB-CU TNL Association To Remove List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Remove Item IEs |  | 1..<maxnoofTNLAssociation> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Transport Layer Address gNB-DU | O |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-DU. | YES | reject |
| gNB-CU TNL Association To Update List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Update Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | O |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| Cells to be barred List |  | 0..1 |  | List of cells to be barred. | YES | ignore |
| >Cells to be barred List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell Barred | M |  | ENUMERATED (barred, not-barred, ...) |  | - |  |
| >>IAB Barred | O |  | ENUMERATED (barred, not-barred, ...) | Corresponds to information provided in the iab-Support contained in the PLMN-IdentityInfo IE or contained in the NPN-IdentityInfo IE as defined in TS 38.331 [8]. The codepoint value “barred” indicates that the iab-Support is not sent in SIB1, and the codepoint value “not-barred” indicates that the iab-Support is sent in SIB1. | - |  |
| Protected E-UTRA Resources List |  | 0..1 |  | List of Protected E-UTRA Resources. | YES | reject |
| >Protected E-UTRA Resources List Item |  | 1.. <maxCellineNB> |  |  | EACH | reject |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxCellineNB) | Indicates the E-UTRA cells involved in resource coordination with the NR cells affiliated with the same Spectrum Sharing Group ID. | - |  |
| >>E-UTRA Cells List |  | 1 |  | List of applicable E-UTRA cells. | - |  |
| >>>E-UTRA Cells List Item |  | 1 .. <maxCellineNB> |  |  | - |  |
| >>>>EUTRA Cell ID | M |  | BIT STRING (SIZE(28)) | Indicates the E-UTRAN Cell Identifier IE contained in the ECGI as defined in subclause 9.2.14 in TS 36.423 [9]. | - |  |
| >>>>Served E-UTRA Cell Information | M |  | 9.3.1.64 |  | - |  |
| Neighbour Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Neighbour Cell Information List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | - |  |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Uplink BH Non-UP Traffic Mapping | O |  | 9.3.1.103 |  | YES | reject |
| BAP Address | O |  | 9.3.1.111 | Indicates a BAP address assigned to the IAB-donor-DU. | YES | ignore |
| CCO Assistance Information | O |  | 9.3.1.211 | Indicates CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU. | YES | Ignore |
| Cells for SON List | O |  | 9.3.1.214 |  | YES | ignore |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Cells with SSBs to be Activated List |  | 0..1 |  | List of SSB beams within the cell requested to be activated. | YES | ignore |
| >Cells with SSBs to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>SSBs to be activated List |  | 1 .. < maxnoofSSBAreas > |  |  | - |  |
| >>>SSB Index | M |  | INTEGER (0..63) | Identifier of SSB beam requested to be activated. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

#### 9.2.1.11	GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a gNB-DU to a gNB-CU to acknowledge update of information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instance, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells Failed to be Activated List |  | 0..1 |  | List of cells which are failed to be activated | YES | reject |
| >Cells Failed to be Activated Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cause | M |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| gNB-CU TNL Association Setup List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association Setup Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU | - |  |
| gNB-CU TNL Association Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association Failed To Setup Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU | - |  |
| >>Cause | M |  | 9.3.1.2 |  | - |  |
| Dedicated SI Delivery Needed UE List |  | 0..1 |  | List of UEs unable to receive system information from broadcast | YES | ignore |
| >Dedicated SI Delivery Needed UE List |  | 1 .. <maxnoofUEIDs> |  |  | EACH | ignore |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - | - |
| >>NR CGI | M |  | 9.3.1.12 |  | - | - |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Cells with SSBs Activated List |  | 0..1 |  |  | YES | ignore |
| >Cells with SSBs Activated List |  | 1.. <maxCellingNBDU> |  |  | - |  |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>SSBs activated List |  | 1 .. < maxnoofSSBAreas > |  |  | - |  |
| >>>SSB Index | M |  | INTEGER (0..63) | Identifier of the SSB beam activated. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum no. of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxnoofUEIDs | Maximum no. of UEs that can be served by a gNB-DU. Value is 65536. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< For Information Only>>>>>>>>>>>>>>>>>>>>

#### 9.3.1.213	Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding coverage configuration selected by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Coverage Modification List |  | 1 |  |  |  |  |
| >Coverage Modification Item |  | 1 .. <maxCellingNBDU> |  |  | - |  |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell Coverage State |  |  | INTEGER (0..63, …) | Value '0' indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | - |  |
| >>SSB Coverage Modification List |  | 0..1 |  |  | - |  |
| >>>SSB Coverage Modification Item |  | 1..<maxnoofSSBAreas> |  |  | - |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>SSB Coverage State | M |  | INTEGER (0..15, …) | Value '0' indicates that the SS/PBCH block is inactive. Other values Indicates that the SS/PBCH block is active and also indicates the coverage configuration of the concerned SS/PBCH block. | - |  |
| >>Coverage Modification Cause | O |  | ENUMERATED(coverage, cell edge capacity, …, network energy saving) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.297	Recommended SSBs for Paging List

This IE indicates the recommended SSBs for paging list.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Recommended SSBs for Paging List Item |  | 1 .. < maxCellingNBDU> |  |  |
| >NR CGI | M |  | 9.3.1.12 |  |
| >SSBs for Paging List |  | 1 .. < maxnoofSSBAreas > |  |  |
| >>SSB Index | M |  | INTEGER (0..63) | Identifier of the recommended SSB beam for paging |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MBSMulticastSessionReceptionState,

F1UTunnelNotEstablished,

MulticastCU2DUCommonRRCInfo,

NRA2XServicesAuthorized,

LTEA2XServicesAuthorized,

NReRedCapUEIndication,

NRPaginglongeDRXInformationforRRCINACTIVE,

Cells-With-SSBs-Activated-List,

Recommended-SSBs-for-Paging-List,

Cells-With-SSBs-to-be-Activated-List-Item

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-F1UTunnelNotEstablished,

id-MulticastCU2DUCommonRRCInfo,

id-NRA2XServicesAuthorized,

id-LTEA2XServicesAuthorized,

id-NRUESidelinkAggregateMaximumBitrateForA2X,

id-LTEUESidelinkAggregateMaximumBitrateForA2X,

id-NReRedCapUEIndication,

id-NRPaginglongeDRXInformationforRRCINACTIVE,

id-Cells-With-SSBs-to-be-Activated-List,

id-Cells-With-SSBs-to-be-Activated-List-Item,

maxCellingNBDU,

maxnoofCandidateSpCells,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- GNB-CU CONFIGURATION UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- GNB-CU CONFIGURATION UPDATE

--

-- **************************************************************

GNBCUConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNBCUConfigurationUpdateIEs} },

...

}

GNBCUConfigurationUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE	TransactionID											PRESENCE mandatory	}|

{ ID id-Cells-to-be-Activated-List				CRITICALITY reject	TYPE	Cells-to-be-Activated-List							PRESENCE optional	}|

{ ID id-Cells-to-be-Deactivated-List			CRITICALITY reject	TYPE	Cells-to-be-Deactivated-List						PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Add-List		CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Add-List				PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Remove-List	CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Remove-List			PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Update-List	CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Update-List			PRESENCE optional	}|

{ ID id-Cells-to-be-Barred-List					CRITICALITY ignore	TYPE	Cells-to-be-Barred-List								PRESENCE optional	}|

{ ID id-Protected-EUTRA-Resources-List			CRITICALITY reject	TYPE	Protected-EUTRA-Resources-List					PRESENCE optional	}|

{ ID id-Neighbour-Cell-Information-List			CRITICALITY ignore	TYPE	Neighbour-Cell-Information-List					PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info			CRITICALITY ignore	TYPE	Transport-Layer-Address-Info						PRESENCE optional	}|

{ ID id-UL-BH-Non-UP-Traffic-Mapping			CRITICALITY reject	TYPE	UL-BH-Non-UP-Traffic-Mapping						PRESENCE optional	}|

{ ID id-BAPAddress								CRITICALITY ignore  TYPE 	BAPAddress												PRESENCE optional }|

{ ID id-CCO-Assistance-Information				CRITICALITY ignore	TYPE 	CCO-Assistance-Information							PRESENCE optional	}|

{ ID id-CellsForSON-List						CRITICALITY ignore	TYPE	CellsForSON-List										PRESENCE optional }|

{ ID id-gNB-CU-Name								CRITICALITY ignore	TYPE	GNB-CU-Name												PRESENCE optional	}|

{ ID id-Extended-GNB-CU-Name					CRITICALITY ignore	TYPE	Extended-GNB-CU-Name									PRESENCE optional	}|

{ ID id-Cells-Allowed-to-be-Deactivated-List			CRITICALITY ignore	TYPE	Cells-Allowed-to-be-Deactivated-List		PRESENCE optional	}|

{ ID id-Cells-With-SSBs-to-be-Activated-List	CRITICALITY ignore	TYPE	Cells-With-SSBs-to-be-Activated-List		PRESENCE optional	},

...

}

Cells-to-be-Deactivated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-to-be-Deactivated-List-ItemIEs } }

GNB-CU-TNL-Association-To-Add-List		::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-To-Add-ItemIEs } }

GNB-CU-TNL-Association-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-To-Remove-ItemIEs } }

GNB-CU-TNL-Association-To-Update-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-To-Update-ItemIEs } }

Cells-to-be-Barred-List			::= SEQUENCE(SIZE(1.. maxCellingNBDU)) OF ProtocolIE-SingleContainer { { Cells-to-be-Barred-ItemIEs } }

Cells-Allowed-to-be-Deactivated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-Allowed-to-be-Deactivated-List-ItemIEs } }

Cells-Allowed-to-be-Deactivated-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-Allowed-to-be-Deactivated-List-Item	CRITICALITY ignore	TYPE	Cells-Allowed-to-be-Deactivated-List-Item	PRESENCE mandatory },

...

}

Cells-to-be-Deactivated-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-to-be-Deactivated-List-Item						CRITICALITY reject	TYPE	Cells-to-be-Deactivated-List-Item					PRESENCE mandatory	},

...

}

Cells-With-SSBs-to-be-Activated-List ::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF ProtocolIE-SingleContainer { { Cells-With-SSBs-to-be-Activated-List-ItemIEs } }

Cells-With-SSBs-to-be-Activated-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-With-SSBs-to-be-Activated-List-Item	CRITICALITY ignore	TYPE	Cells-With-SSBs-to-be-Activated-List-Item	PRESENCE mandatory },

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-SDTRLCBearerConfiguration,

id-SRBMappingInfo,

id-DRBMappingInfo,

id-LastUsedCellIndication,

id-Recommended-SSBs-List,

id-SIB17-message,

id-MUSIM-GapConfig,

id-SIB20-message,

id-pathPower,

id-DU-RX-MT-RX-Extend,

id-DU-TX-MT-TX-Extend,

id-DU-RX-MT-TX-Extend,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Cells-to-be-Activated-List-Item ::= SEQUENCE {

nRCGI		NRCGI,

nRPCI		NRPCI		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { Cells-to-be-Activated-List-ItemExtIEs} }	OPTIONAL,

...

}

Cells-to-be-Activated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-gNB-CUSystemInformation			CRITICALITY reject	EXTENSION GNB-CUSystemInformation				PRESENCE optional }|

{ ID id-AvailablePLMNList				CRITICALITY ignore	EXTENSION AvailablePLMNList						PRESENCE optional }|

{ ID id-ExtendedAvailablePLMN-List		CRITICALITY ignore	EXTENSION ExtendedAvailablePLMN-List			PRESENCE optional }|

{ ID id-IAB-Info-IAB-donor-CU			CRITICALITY ignore	EXTENSION IAB-Info-IAB-donor-CU					PRESENCE optional}|

{ ID id-AvailableSNPN-ID-List			CRITICALITY ignore	EXTENSION AvailableSNPN-ID-List					PRESENCE optional }|

{ ID id-MBS-Broadcast-NeighbourCellList	CRITICALITY ignore	EXTENSION MBS-Broadcast-NeighbourCellList	PRESENCE optional }

,

...

}

Cells-With-SSBs-Activated-List ::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF Cells-With-SSBs-Activated-List-Item

Cells-With-SSBs-Activated-List-Item::= SEQUENCE {

nGCGI			 			NRCGI,

sSBs-activated-List 		SSBs-activated-List,

iE-Extensions				ProtocolExtensionContainer { { Cells-With-SSBs-Activated-List-Item-ExtIEs} } OPTIONAL

}

Cells-With-SSBs-Activated-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-With-SSBs-to-be-Activated-List-Item::= SEQUENCE {

nRCGI			 			NRCGI,

sSBs-to-be-activated-List 	SSBs-to-be-activated-List,

iE-Extensions				ProtocolExtensionContainer { { Cells-With-SSBs-to-be-Activated-List-ItemExtIEs} } OPTIONAL,

...

}

Cells-With-SSBs-to-be-Activated-List-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Coverage-Modification-Item ::= SEQUENCE {

nRCGI								NRCGI,

cellCoverageState					CellCoverageState,

sSBCoverageModificationList			SSBCoverageModification-List OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { Coverage-Modification-Item-ExtIEs} } 				OPTIONAL,

...

}

Coverage-Modification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-Coverage-Modification-Cause	CRITICALITY ignore	EXTENSION CCO-issue-detection		PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

SSBCoverageModification-Item::= SEQUENCE {

sSBIndex						INTEGER(0..63),

sSBCoverageState				SSBCoverageState,

iE-Extensions		ProtocolExtensionContainer { { SSBCoverageModification-Item-ExtIEs} }	OPTIONAL,

...

}

SSBCoverageModification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

SSBs-activated-List ::= SEQUENCE (SIZE(1.. maxnoofSSBAreas)) OF SSB-Index

SSBs-forPaging-List ::= SEQUENCE (SIZE(1.. maxnoofSSBAreas)) OF SSB-Index

SSBs-to-be-activated-List ::= SEQUENCE (SIZE(1.. maxnoofSSBAreas)) OF SSB-Index

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-Recommended-SSBs-for-Paging-List					ProtocolIE-ID ::= 743

id-Cells-With-SSBs-Activated-List					ProtocolIE-ID ::= 745

id-Cells-Allowed-to-be-Deactivated-List				ProtocolIE-ID ::= 746

id-Cells-Allowed-to-be-Deactivated-List-Item		ProtocolIE-ID ::= 747

id-Coverage-Modification-Cause						ProtocolIE-ID ::= 748

id-RANTSSRequestType								ProtocolIE-ID ::= 749

id-RANTimingSynchronisationStatusInfo				ProtocolIE-ID ::= 750

id-TSCTrafficCharacteristicsFeedback				ProtocolIE-ID ::= 751

id-RANfeedbacktype									ProtocolIE-ID ::= 752

id-Mobile-TRP-LocationInformation					ProtocolIE-ID ::= 753

id-Mobile-IAB-MT-UE-ID								ProtocolIE-ID ::= 754

id-Target-gNB-ID									ProtocolIE-ID ::= 755

id-Target-gNB-IP-address							ProtocolIE-ID ::= 756

id-Target-SeGW-IP-address							ProtocolIE-ID ::= 757

id-Activated-Cells-Mapping-List						ProtocolIE-ID ::= 758

id-Activated-Cells-Mapping-List-Item				ProtocolIE-ID ::= 759

id-F1SetupOutcome									ProtocolIE-ID ::= 760

id-RRC-Terminating-IAB-Donor-Related-Info			ProtocolIE-ID ::= 761

id-RRC-Terminating-IAB-Donor-gNB-ID					ProtocolIE-ID ::= 762

id-NCGI-to-be-Updated-List							ProtocolIE-ID ::= 763

id-NCGI-to-be-Updated-List-Item						ProtocolIE-ID ::= 764

id-Mobile-IAB-MTUserLocationInformation				ProtocolIE-ID ::= 765

id-MobileAccessPointLocation						ProtocolIE-ID ::= 766

id-AssociatedSessionID								ProtocolIE-ID ::= 767

id-IndicationMCInactiveReception					ProtocolIE-ID ::= 768

id-MulticastCU2DURRCInfo							ProtocolIE-ID ::= 769

id-MBSMulticastSessionReceptionState				ProtocolIE-ID ::= 770

id-F1UTunnelNotEstablished							ProtocolIE-ID ::= 771

id-MulticastDU2CURRCInfo							ProtocolIE-ID ::= 772

id-SIBX-message										ProtocolIE-ID ::= 773

id-MulticastCU2DUCommonRRCInfo						ProtocolIE-ID ::= 774

id-PDUSetQoSParameters								ProtocolIE-ID ::= 775

id-N6JitterInformation								ProtocolIE-ID ::= 776

id-ECNMarkingorCongestionInformationReportingRequest	ProtocolIE-ID ::= 777

id-ECNMarkingorCongestionInformationReportingStatus	ProtocolIE-ID ::= 778

id-NRA2XServicesAuthorized							ProtocolIE-ID ::= 779

id-LTEA2XServicesAuthorized							ProtocolIE-ID ::= 780

id-NRUESidelinkAggregateMaximumBitrateForA2X		ProtocolIE-ID ::= 781

id-LTEUESidelinkAggregateMaximumBitrateForA2X		ProtocolIE-ID ::= 782

id-NReRedCapUEIndication 							ProtocolIE-ID ::= 783

id-ERedcap-Bcast-Information 						ProtocolIE-ID ::= 784

id-NRPaginglongeDRXInformationforRRCINACTIVE		ProtocolIE-ID ::= 785

id-Cells-With-SSBs-to-be-Activated-List				ProtocolIE-ID ::= aaa

id-Cells-With-SSBs-to-be-Activated-List-Item		ProtocolIE-ID ::= bbb

Change Ends