3GPP TSG-RAN WG3 Meeting #123	R3-240441

Athens, Greece, Feb 26 – Mar 01, 2024

Title: 	Inconsistency in PPID between IANA and 3GPP for E1AP

Source: 	Rakuten Mobile.

Agenda item:	7.0

Document Type:	Discussion and decision

# 1. Introduction

In this contribution, point out the inconsistency in Protocol Payload Identifier (PPID) between IANA and 3GPP and propose correction for the issue.

# 2. Discussion

## 2.1 SCTP Payload Protocol Identifier for application protocols

SCTP PPIDs are allocated by IANA to all RAN application protocols on a first come first serve basis and has been the procedure since long. This can be found from the below hyperlink.

https://www.iana.org/assignments/sctp-parameters/sctp-parameters.xhtml#sctp-parameters-25

From the above, it is clear that Xn-AP and E1-AP have a PPID of 67 and 69 respectively.

| 66 | 3GPP NGAP over DTLS over SCTP | [ 3GPP TS 38.413][Yang_Xudong] | 2019-05-24 |
| --- | --- | --- | --- |
| 67 | 3GPP XnAP over DTLS over SCTP | [ 3GPP TS 38.423][Yang_Xudong] | 2019-05-24 |
| 68 | 3GPP F1AP over DTLS over SCTP | [ 3GPP TS 38.473][Yang_Xudong] | 2019-05-24 |
| 69 | 3GPP E1AP over DTLS over SCTP | [ 3GPP TS 38.463][Yang_Xudong] | 2019-05-24 |

From 3GPP TS 37482-h30 version 17.3.0 Release 17, Sec 7,

“SCTP (IETF RFC 4960 [6]) shall be supported as the transport layer of E1 signalling bearer. The Payload Protocol Identifier assigned by IANA to be used by SCTP for the application layer protocol E1AP is 64 and 67 for DTLS over SCTP (IETF RFC 6083 [11]). The byte order of the ppid shall be big-endian.”

The IANA allocated PPID for DTLS over SCTP is 69, while the 3GPP assigned value is 67. This value needs to be consistent with IANA allocation.

Proposal 1: RAN3 discusses and agrees to correct the 3GPP assigned PPID for 3GPP E1AP over DTLS over SCTP to be consistent with IANA.

# 3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Proposal 1: RAN3 discusses and agrees to correct the 3GPP assigned PPID for 3GPP E1AP over DTLS over SCTP to be consistent with IANA.

.

# 4. Reference