3GPP TSG-RAN WG3 Meeting #123	R3-240459

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda Item:	9.1.1.1

Source:	Huawei, Deutsche Telekom, China Unicom

Title:	RACH, NR-U and NPN corrections

Document for:	Discussion

1	Introduction

In this contribution, we present our proposed update to RACH, SON for NR-U and NPN based on the latest specifications.

2.	Discussion

2.1 	SON for NR-U

The source node requesting the target node to report the DL LBT Failure Information during handover execution has been agreed. This is however not supported over F1AP. This is needed since the information should be coming from the DU. This has impact on 38.470 and 38.473 since we would need a new message to transfer the information. It is therefore proposed to add a new message from DU to CU to carry access and mobility information.

Addd a new message to transfer DL LBT Failure Information from DU to CU.

Note: The CR to 38.470 is submitted to agenda item 9.1.1.1 , and the related CR to 38.473 is found in the other agenda item since it has ASN.1 impact.

We also note that the procedure text for XnAP is missing.

Add missing procedure text for DL LBT Failure Information List in XnAP.

2.2 	MDT for NPN

The following procedural texts are captured in sections of successful operation and abnormal conditions in Handover Request, Retrieve UE Context and Trace Start messages:

- If the PNI-NPN Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection in PNI-NPN.

- If both the PNI-NPN Area Scope of MDT IE and the Area Scope of MDT-NR IE are included in the MDT Configuration-NR IE in the HANDOVER REQUEST message, and the Area Scope of MDT-NR IE is set to "PNI-NPN based", the target NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collections in PNI-NPN areas, and ignore the PNI-NPN Area Scope of MDT IE.

- If the PNI-NPN Area Scope of MDT IE is included in the MDT Configuration-NR IE in the HANDOVER REQUEST message, and the Area Scope of MDT-NR IE is not included, the target NG-RAN node shall ignore the PNI-NPN Area Scope of MDT IE, and consider that the MDT Configuration for NR is applied to all PLMNs indicated in the MDT PLMN List described in TS 32.422 [23].

Basically, for area scope configuration for PNI-NPN, there are the following cases for the presence of PNI-NPN Area Scope of MDT IE and Area Scope of MDT-NR IE.

- Case 1: both Area Scope of MDT-NR IE and PNI-NPN Area Scope of MDT IE are present

  - Case 1a, Area Scope of MDT-NR IE is NOT set to "PNI-NPN based”, this is actually the case addressed by bullet 1 above. Procedural text in bullet 1 should be refined. If not, bullet 1 will cover the case of bullet 2, which may cause ambiguity.

  - Case 1b, Area Scope of MDT-NR IE is set to "PNI-NPN based”, Abnormal case, covered by bullet 2 above.

- Case 2: Area Scope of MDT-NR IE is present and PNI-NPN Area Scope of MDT IE is NOT present. This is the normal case.

- Case 3: Area Scope of MDT-NR IE is NOT present and PNI-NPN Area Scope of MDT IE is present. Abnormal case. Covered by bullet 3 above

To resolve the ambiguity in case 1a, we propose to revise the procedural text description when both Area Scope of MDT-NR IE and PNI-NPN Area Scope of MDT IE are present in the Handover Request, Retrieve UE Context and Trace Start messages.

Revise the text for PNI-NPN Area Scope of MDT to clarify that this applies when the Area Scope of MDT-NR IE is present and not set to "PNI-NPN based".

2.3 	RACH

First proposed correction is for 37.340, where the current text does not specify that only the NR-RA report is forwarded. Furhter, the sentence on where the information can be forwarded would benefit from some simplification.

Clarify in 37.340 that only NR report is forwarded and simplify the text specifying where the information is forwarded.

- Next change is related to the RRC reference that is used in our specifications. Our current spec uses “PScellListNR” but the corrct reference should be “CellIdListNR”.

Correct reference in X2AP and XnAP from “PScellListNR” to “CellIdListNR”.

Next change is related to the description of the Self-optimisation support procedure in 38.470. We note that the current text has added a separate bulletlist for SON related information on top of the the original description for failure and mobility information. Since SON Related Information may also cover the failure and mobility related information. Keeping them separately may cause misunderstanding. Here, we would prefer to simplify by just keeping a single list for SON functionality.

Generalize the structure in 38.470 to only keep “SON related information”.

Next change is related to the RACH Indication procedure in F1AP. Clause 10 of F1AP specified the following rules for transaction ID for non-UE associated procedures.

Clause 10 of TS 38.413 [3] is applicable for the purposes of the present document, with the following additions for non-UE-associated procedures:

-	In case of Abstract Syntax Error, when reporting the Criticality Diagnostics IE for not comprehended IE/IEgroups or missing IE/IE groups, the Transaction ID IE shall also be included;

-	In case of Logical Error, when reporting the Criticality Diagnostics IE, the Transaction ID IE shall also be included;

Howerver, the transaction ID in RACH Indication procedure is stil missing.

Add the transaction ID in RACH Indication procedure in F1AP.

3	Conclusion

This document proposes:

Proposal 1:	Addd a new message to transfer DL LBT Failure Information from DU to CU.

Proposal 2:	Add missing procedure text for DL LBT Failure Information List in XnAP.

Proposal 3:	Revise the text for PNI-NPN Area Scope of MDT to clarify that this applies when the Area Scope of MDT-NR IE is present and not set to "PNI-NPN based".

Proposal 4:	Clarify in 37.420 that only NR report is forwarded and simplify the text specifying where the information is forwarded.

Proposal 5:	Correct reference in X2AP and XnAP from “PScellListNR” to “CellIdListNR”.

Proposal 6:	Generalize the structure in 38.470 to only keep “SON related information”.

Proposal 7: 	Add the transaction ID in RACH Indication procedure in F1AP.

The corresponding CRs are provided in [1] ~ [9].

4.	Reference

[1] R3-240460	Correction on MDT for NPN

[2] R3-240461	Correction on SON for NR-U

[3] R3-240462	Correction on SON for NR-U

[4] R3-240463	Correction on SON for NR-U

[5] R3-240464	Correction on RACH Optimisation

[6] R3-240465	Correction on RACH Optimisation

[7] R3-240466	Correction on RACH Optimisation

[8] R3-240467	Correction on RACH Optimisation

[9] R3-240468	Correction on RACH Optimisation