3GPP TSG-RAN WG3 Meeting #123	R3-240492

Athens, Greece, 26th Feb – 1st Mar, 2024

Agenda item:	9.1.9.1

Source: 	Huawei, Qualcomm, Lenovo

Title: 	Support of PSI based discard

Document for:	Discussion and Decision

1. Introduction

PSI based discard has been supported for XR. In case of CU-DU split, there are some RAN3 specification impacts. In this contribution, we will give the analysis on the potential changes and provide the related CRs in [6-8].

2. Discussion

2.1 UL PSI based PDU set discard

In order to mitigate the network congestion, R18 XR supports to discard the PDCP SDU/PDUs based on the PDU set importance (PSI). Specifically, for UL, the gNB can configure two PDCP discard timers with different values for one DRB. When congestion happens, the gNB can indicate the UE to activate the shorter PDCP discard timer for the SDUs belonging to a low important PDU set via MAC CE, so that the low important data will be discarded early to reduce the resource consumption. Furthermore, when the congestion disappears, the gNB can also indicate the deactivation of the shorter discard timer. The procedure is specified in TS 38.323 [1], as excerpted below.

5.2.1 Transmit operation At reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall: - if discardTimerForLowImportance is configured and PSI based SDU discard is activated, and the PDCP SDU belongs to a low importance PDU Set: - start the discardTimerForLowImportance associated with this PDCP SDU; - else: - start the discardTimer associated with this PDCP SDU (if configured). NOTE 0: Identification of PSI of a PDU Set and determination of low importance PDU Set are left up to UE implementation.

During the above procedure, the gNB shall monitor the network congestion level and take it into account to decide whether activation/deactivation for PSI based SDU discard is needed. In case of CU-DU split, the gNB-DU is responsible to generate the activation/deactivation MAC CE, and is aware of the network congestion.

However, in the current specification, the gNB-DU does not know whether the UE is configured with PSI based SDU discard at all. So the gNB-DU may not perform congestion monitoring and will not send the activation/deactivation MAC CE as expected, result in that the PSI based discard functionality is invalid under the CU-DU split architecture. To resolve this issue, the gNB-CU should notify the gNB-DU about the configuration of PSI based SDU discard.

Moreover, considering the activation/deactivation for PSI based SDU discard is controlled in per DRB level, as shown in the following text quoted from TS 38.321[2], it is proper to notify the gNB-DU which DRBs are configured with PSI based discard. The information can be included in the DRB Information IE within the UE CONTEXT SETUP REQUEST message and the UE CONTEXT MODIFICATION REQUEST message.

6.1.3.73 PSI-Based SDU Discard Activation/Deactivation MAC CE The PSI-Based SDU Discard Activation/Deactivation MAC CE is identified by MAC subheader with an one-octet eLCID as specified in Table 6.2.1-1b. It has a fixed size and consists of one octet defined as follows (Figure 6.1.3.73-1): - Di: This field indicates the activation/deactivation status of the PSI-based SDU discard of DRB i, where i is the ascending order of the DRB ID among the DRBs configured with PSI-based SDU discard. The Di field set to 1 indicates that the PSI-based SDU discard shall be activated for DRB i. The Di field set to 0 indicates that the PSI-based SDU discard shall be deactivated for DRB i. Figure 6.1.3.73-1: PSI-based SDU Discard Activation/Deactivation MAC CE

Proposal 1: The gNB-CU-CP can notify the gNB-DU about per DRB UL PSI based SDU discard configuration in the UE CONTEXT SETUP REQUEST message and the UE CONTEXT MODIFICATION REQUEST message.

2.2 DL PSI based PDU set discard

According to the description of PSI given in clause 5.37.5.2 in TS 23.501 [3], the gNB can also perform DL PDU set discard based on the PSI received from the CN in presence of congestion.

5.37.5.2 PDU Set Information and Identification - PDU Set Importance, which identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow. The NG-RAN may use the Priority Level (see clause 5.7.3.3) across QoS Flows and PDU Set Importance within a QoS Flow for PDU Set level packet discarding in presence of congestion.

Similar to the UL case, the method using two PDCP discard timers can also be applied for DL. Considering the CP-UP split scenario, the gNB-CU-CP should configure the gNB-CU-UP with two discard timers, one is the legacy and one is additional for the low important PDU sets. The additional timer would be shorter than the legacy one so that the gNB-CU-UP can apply the additional timer for the low important PDU sets in presence of congestion. Which PSI levels are regarded as low important can be left to the gNB-CU-UP implementation. The value of the additional discard timer used for PSI based discard can be included in the PDCP Configuration IE, together with the legacy discard timer.

Proposal 2: The gNB-CU-CP can additionally configure a shorter discard timer for DL PSI based discard to the gNB-CU-UP. How to judge the importance of each packets should be up to CU-UP implementation based on the PSI contained in the packets.

Another issue is when the gNB-CU-UP should activate the shorter discard timer. The intention of PSI based discard is to mitigate the network congestion, so it should only be activated when the network is congested. However, it is difficult for the gNB-CU-UP to judge the congestion level since it is not aware of the transmission condition over the Uu interface. Instead, the gNB-DU is much more aware of the network congestion level. To enable the functionality, some interactions between the gNB-DU and the gNB-CU-UP would be needed.

Actually, the similar issue also exists for PDCP duplication, which is used to improve the reliability when the air interface link quality is not good enough. For DL, the gNB-DU can measure the link quality and suggest the gNB-CU-UP to perform PDCP duplication when needed. Specifically, the gNB-CU-CP indicates the gNB-DU to setup the RLC entities used for PDCP duplication during the UE Context Setup procedure and the UE Context Modification procedure. Then the gNB-DU can send PDCP duplication activation suggestion to the gNB-CU-UP via the ASSISTANCE INFORMATION DATA frame so that the gNB-CU-UP can perform DL PDCP duplication accordingly. The above mechanism is described in TS 38.473 and TS 35.425 as excerpted below.

TS 38.473: ### 8.3.1 UE Context Setup If one or two Additional PDCP Duplication UP TNL Information IEs are included in the UE CONTEXT SETUP REQUEST message for a DRB, the gNB-DU shall, if supported, include one or two Additional PDCP Duplication UP TNL Information IEs in the UE CONTEXT SETUP RESPONSE message and setup one or two additional RLC entities for the indicated DRB. The gNB-CU and the gNB-DU use the Additional PDCP Duplication UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2]. 8.3.4 UE Context Modification (gNB-CU initiated) If one or two Additional PDCP Duplication UP TNL Information IEs are included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall, if supported, include one or two Additional PDCP Duplication UP TNL Information IEs in the UE CONTEXT MODIFICATION RESPONSE message and setup one or two additional RLC entities for the indicated DRB. The gNB-CU and the gNB-DU use the Additional PDCP Duplication UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2]. TS 38.425: 5.5.2.3 ASSISTANCE INFORMATION DATA (PDU Type 2) This frame format is defined to allow the node hosting the PDCP entity to receive assistance information. The following shows the respective ASSISTANCE INFORMATION DATA frame. NOTE 1: All information elements defined in Figure 5.5.2.3-1 are also applicable to E-UTRA PDCP unless specified otherwise in section 5.5.3. Bits; Number of Octets. 7; 6; 5; 4; 3; 2; 1; 0. PDU Type (=2); PDCP Dupl. Ind.; Assistance Info. Ind.; UL Delay Ind.; DL Delay Ind.; 1. Spare; UL Congestion Information Ind.; DL Congestion Information Ind.; PDCP Duplication Activation Suggestion; 1. ……; …….

A similar mechanism should be designed for DL PSI based discard taking PDCP duplication as the reference. First, to inform the gNB-DU to monitor the network congestion and judge whether PSI based discard is needed, the gNB-CU-CP shall notify the gNB-DU about per DRB DL PSI based discard configuration. This can be done during the UE context setup and modification procedures. After getting the notification, the gNB-DU can determine whether DL PSI based discard should be activated or deactivated for the DRB based on the congestion level and it can further send the suggestion to the gNB-CU-UP.

Proposal 3: The gNB-CU-CP can notify the gNB-DU about per DRB DL PSI based SDU discard configuration in the UE CONTEXT SETUP REQUEST message and the UE CONTEXT MODIFICATION REQUEST message.

Proposal 4: The gNB-DU can send the suggestion on DL PSI based SDU discard activation/deactivation to the gNB-CU-UP if DL PSI-based discard is configured.

3. Conclusion

In this contribution, we analyzed the potential specification impacts to support UL and DL PSI based discard. The following proposals were made:

Proposal 1: The gNB-CU-CP can notify the gNB-DU about per DRB UL PSI based SDU discard configuration in the UE CONTEXT SETUP REQUEST message and the UE CONTEXT MODIFICATION REQUEST message.

Proposal 2: The gNB-CU-CP can additionally configure a shorter discard timer for DL PSI based discard to the gNB-CU-UP. How to judge the importance of each packets should be up to the gNB-CU-UP implementation based on the PSI contained in the packets.

Proposal 3: The gNB-CU-CP can notify the gNB-DU about per DRB DL PSI based SDU discard configuration in the UE CONTEXT SETUP REQUEST message and the UE CONTEXT MODIFICATION REQUEST message.

Proposal 4: The gNB-DU can send the suggestion on DL PSI based SDU discard activation/deactivation to the gNB-CU-UP if DL PSI-based discard is configured.

4. Reference

[1]. 3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[2]. 3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[3]. 3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]. 3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[5]. 3GPP TS 38.425: "NG-RAN; NR user plane protocol".

[6] R3-240493, Correction on PSI based discard (CR for 38.473)

[7] R3-240494, Correction on PSI based discard (CR for 37.483)

[8] R3-240495, Correction on PSI based discard (CR for 38.425)