3GPP TSG-RAN WG3 Meeting #123	R3-240501

# Athens, Greece, 26th Feb. – 1st Mar. 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1929 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to S1AP Time Based Handover Information IE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) S (adding to the sourcing companies’ CR statistics) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current S1AP and X2AP, the Assigned Criticality for the new Time Based Handover Information IE and Conditional Handover Time Based Information IE are “ignore”. This is different to the Assigned Criticality of other conditional HO related IEs (e.g., Conditional Handover Information Request IE) that is set to “reject”. By current Assigned Criticality “ignore”, source eNB incorrectly considers a non-supporting target eNB support HO with Time-based trigger condition. This cause problem later, when the source eNB initiates other CHO related procedure, e.g. initiate early status transfer procedure, etc. The Assigned Criticality should be “reject” to allow a non-supporting target RAN node reject the CHO with Time-based Trigger Condition. Similar issue also applies to NR NTN. In addition, the Presence for Time Based Handover Information IE should be "O", and delete the Range field. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the Assigned Criticality for Time Based Handover Information IE to “reject”. Change the Presence for Time Based Handover Information IE to "O", and delete the Range field. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Source RAN node incorrectly considers target RAN node support CHO with Time-based Trigger Condition, and cause problems to other CHO related procedures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.7, 9.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.423 CR 1773 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<< START OF CHANGE >>>>>>

#### 9.2.1.7	Source eNB to Target eNB Transparent Container

The Source eNB to target eNB Transparent Container IE is an information element that is produced by the source eNB and is transmitted to the target eNB. For inter-system handovers to E-UTRAN, the IE is transmitted from the external handover source to the target eNB.

This IE is transparent to the EPC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| RRC Container | M |  | OCTET STRING | Includes the HandoverPreparation Information message as defined in subclause 10.2.2 of TS 36.331 [16]. | - |  |
| E-RABs Information List |  | 0..1 |  |  | - |  |
| >E-RABs Information Item |  | 1 .. <maxnoof E-RABs> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |
| >>DL Forwarding | O |  | 9.2.3.14 |  | - |  |
| >>DAPS Request Information | O |  | 9.2.1.155 |  | YES | ignore |
| >>Source Transport Layer Address | O |  | 9.2.2.1 | Identifies the TNL address used by the sending node for direct data forwarding towards the target eNB | YES | ignore |
| >>Security Indication | O |  | 9.2.1.163 |  | YES | ignore |
| >>Source Node Transport Layer Address | O |  | 9.2.2.1 | Identifies the TNL address used by the source SN node for direct data forwarding towards the target eNB | YES | ignore |
| Target Cell ID | M |  | E-UTRAN CGI 9.2.1.38 |  | - |  |
| Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.1.39 |  | - |  |
| UE History Information | M |  | 9.2.1.42 |  | - |  |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the external handover source provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | OCTET STRING | Includes the VisitedCellInfoList IE contained in the UEInformationResponse message (TS 36.331 [16]) | YES | ignore |
| IMS voice EPS fallback from 5G | O |  | ENUMERATED (true, …) |  | YES | ignore |
| Additional RRM Policy Index | O |  | 9.2.1.39a |  | YES | ignore |
| UE Context Reference at Source | O |  | 9.2.1.144 |  | YES | ignore |
| Inter-system measurement Configuration | O |  | 9.2.1.151 |  | YES | ignore |
| Source Node ID | O |  | 9.2.1.152 |  | YES | ignore |
| Emergency Indicator | O |  | ENUMERATED (true, …) | Indicates an emergency EPS voice fallback | YES | ignore |
| UE Context Reference at Source eNB | O |  | eNB UE S1AP ID 9.2.3.4 | This IE is used for NTN operation. | YES | ignore |
| Source SN ID | O |  | Global RAN Node ID 9.2.1.131 |  | YES | ignore |
| Direct Forwarding Path Availability | O |  | 9.2.3.15 | Indicates whether a direct forwarding path between the source RAN node and the target eNB is available | YES | ignore |
| Time Based Handover Information | O |  |  | This IE only applies to IoT NTN. | YES | reject |
| >Handover Window Start | M |  | INTEGER (0..549755813887) | Corresponds to information provided in the t1-Threshold contained in the ReportConfigEUTRA IE as defined in TS 36.331 [16] | - |  |
| >Handover Window Duration | M |  | INTEGER (1..6000) | Corresponds to information provided in the duration contained in the condEventT1 contained in the ReportConfigEUTRA IE as defined in TS 36.331 [16] | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RABs for one UE. Value is 256. |

<<<<<< NEXT CHANGE >>>>>>

### 9.3.4	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

** Unchanged text skipped **

SourceeNB-ToTargeteNB-TransparentContainer-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

{ID id-MobilityInformation					CRITICALITY ignore	EXTENSION MobilityInformation							PRESENCE optional}|

{ID id-uE-HistoryInformationFromTheUE		CRITICALITY ignore	EXTENSION UE-HistoryInformationFromTheUE			PRESENCE optional}|

{ID id-IMSvoiceEPSfallbackfrom5G			CRITICALITY ignore	EXTENSION IMSvoiceEPSfallbackfrom5G					PRESENCE optional}|

{ID id-AdditionalRRMPriorityIndex			CRITICALITY ignore	EXTENSION AdditionalRRMPriorityIndex				PRESENCE optional}|

{ID id-ContextatSource						CRITICALITY ignore	EXTENSION ContextatSource								PRESENCE optional}|

{ID id-IntersystemMeasurementConfiguration	CRITICALITY ignore	EXTENSION IntersystemMeasurementConfiguration	PRESENCE optional}|

{ID id-SourceNodeID							CRITICALITY ignore	EXTENSION SourceNodeID									PRESENCE optional}|

{ID id-EmergencyIndicator					CRITICALITY ignore	EXTENSION EmergencyIndicator							PRESENCE optional}|

{ID id-UEContextReferenceatSourceeNB		CRITICALITY ignore	EXTENSION ENB-UE-S1AP-ID										PRESENCE optional}|

{ID id-SourceSNID							CRITICALITY ignore	EXTENSION Global-RAN-NODE-ID							PRESENCE optional}|

{ID id-Direct-Forwarding-Path-Availability		CRITICALITY ignore	EXTENSION Direct-Forwarding-Path-Availability		PRESENCE optional}|

{ID id-TimeBasedHandoverInformation			CRITICALITY reject	EXTENSION TimeBasedHandoverInformation				PRESENCE optional},

...

}

<<<<<< END OF CHANGE >>>>>>