3GPP TSG-RAN WG3 Meeting #123	R3-240606

Athens, Greece, February 26th – March 1st 2024

| +6CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1194 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Corrections of QMC - New IEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE_enh-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN3 specified an IE for the SN to indicate to the MN that a configuration has been released. However, this IE does not indicate whether only the RVQoE or both QoE and RVQoE configuration were released. Moreover, as of today, the indication can only be sent from the SN to the MN, and not vice versa. This is an issue because, in some cases, the SN may assist the MN with respect to measurement collection, so the SN should be notified in case a configuration is released. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Changed the codepoints of the Configuration Release Indication IE for “ENUMERATED (true, …)” to “ENUMERATED (rvqoe, qoe and rvqoe …)”. -Added a new Session Status Indication IE to indicate to the peer node the status of the session. This CR is not backwards compatible from an ASN.1 point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The QMC NR-DC coordination framework does not work properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.197, 9.2.3.198, 9.3.5. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Start of changes-------------------------------------------

#### 9.2.3.197	QMC Coordination Request

This IE contains the information that the S-NG-RAN node needs to provide to the M-NG-RAN node, or the information that the M-NG-RAN node needs to provide to the S-NG-RAN node, for managing configuration and reporting of one or more QoE and/or RAN visible QoE measurements.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MN to SN QMC Coordination Request List |  | 0..1 |  |  |
| >MN to SN QMC Coordination Request Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are coming with the QMC activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3 bytes Octet String. |
| >>Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, ...) |  |
| >>Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 | The IP address of the entity receiving the QoE measurement report. |
| >>QoE Reporting Path Request | O |  | ENUMERATED (srb4, srb5, …) | This IE indicates the preferred SRB for receiving the QoE reports. |
| >>RAN Visible QoE Reporting Path Request | O |  | ENUMERATED (srb4, srb5, …) | This IE indicates the preferred SRB for receiving the RAN Visible QoE reports. |
| >>Further RAN Visible QoE Interest Inquiry | O |  | ENUMERATED (true, ...) | This IE is used to request from the S-NG-RAN node to indicate whether it is interested in receiving further RVQoE reports. |
| >>Further RAN Visible QoE Reporting Path | O |  | ENUMERATED (true, ...) | This IE indicates the preferred SRB for receiving further RAN Visible QoE reports. |
| >>Current RAN Visible QoE Configuration | O |  | RAN Visible QoE Configuration 9.2.3.201 | This IE is to indicate the current RAN Visible QoE configuration and inquire about the RAN Visible QoE Configuration preference of the S-NG-RAN node. |
| >>Configuration Release Indication | O |  | ENUMERATED (rvqoe, qoe and rvqoe, ...) | This IE indicates that the configuration has been released by the MN. |
| >>QoE Session Status | O |  | ENUMERATED (started, ended, …) | This IE indicates the status of the application session. |
| SN to MN QMC Coordination Request List |  | 0..1 |  |  |
| >SN to MN QMC Coordination Request Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are coming with the QMC activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3 bytes Octet String. |
| >>Measurement Collection Entity IP Address | O |  | Transport Layer Address 9.2.3.29 | The IP address of the entity receiving the QoE measurement report. |
| >>QoE Reporting Path Request | O |  | ENUMERATED (srb4, srb5, ...) | This IE indicates the preferred SRB for receiving the QoE reports. |
| >>RAN Visible QoE Reporting Path Request | O |  | ENUMERATED (srb4, srb5, ...) | This IE indicates the preferred SRB for receiving the RAN Visible QoE reports. |
| >>Further RAN Visible QoE Interest Inquiry | O |  | ENUMERATED (true, ...) | This IE is used to request from the M-NG-RAN node to indicate whether it is interested in receiving further RVQoE reports. |
| >>Further RAN Visible QoE Reporting Path Inquiry | O |  | ENUMERATED (true, ...) | This IE is used to request from the M-NG-RAN node the preferred SRB for receiving further RAN Visible QoE reports. |
| >>Current RAN Visible QoE Configuration | O |  | RAN Visible QoE Configuration 9.2.3.201 | This IE is to indicate the current RAN Visible QoE configuration and inquire about the RAN Visible QoE Configuration preference of the M-NG-RAN node. |
| >>Configuration Release Indication | O |  | ENUMERATED (rvqoe, qoe and rvqoe,, ...) | This IE indicates that the configuration has been released by the SN. |
| >>QoE Session Status | O |  | ENUMERATED (started, ended, …) | This IE indicates the status of the application session. |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

#### 9.2.3.198	QMC Coordination Response

This IE contains the information that the M-NG-RAN node needs to provide to the S-NG-RAN node, or the information that the S-NG-RAN node needs to provide to the S-NG-RAN node in response to the QMC Coordination Request.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MN to SN QMC Coordination Response List |  | 0..1 |  |  |
| >MN to SN QMC Coordination Response Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are coming with the QMC activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3 bytes Octet String. |
| >>Measurement Configuration Application Layer ID | O |  | INTEGER (0..15, ...) |  |
| >>QoE Configuration Sending Path | O |  | ENUMERATED (mn, sn, …) |  |
| >>QoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  |
| >>RVQoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  |
| >>Further RAN Visible QoE Interest Response | O |  | ENUMERATED (rvqoe reports desired, rvqoe reports not desired, ...) | This IE is to indicate whether the M-NG-RAN node is interested in receiving further RAN Visible QoE reports. |
| >>Further RAN Visible QoE Reporting Path | O |  | ENUMERATED (srb4, srb5, ...) | This IE is to indicate the preferred path for further RAN Visible QoE reporting. |
| >>Preferred RAN Visible QoE Configuration | O |  | RAN Visible QoE Configuration 9.2.3.201 | The RAN Visible QoE Configuration preference of the M-NG-RAN node. |
| >>QoE Session Status | O |  | ENUMERATED (started, ended, …) | This IE indicates the status of the application session. |
| SN to MN QMC Coordination Response List |  | 0..1 |  |  |
| >SN to MN QMC Coordination Response Item |  | 1..<maxnoofUEAppLayerMeas> |  |  |
| >>QoE Reference | M |  | OCTET STRING (SIZE(6)) | QoE Reference, as defined in clause 5.2 of TS 28.405 [55]. It consists of MCC+MNC+QMC ID, where the MCC and MNC are coming with the QMC activation request from the management system to identify one PLMN containing the management system, and QMC ID is a 3 bytes Octet String. |
| >>QoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  |
| >>RVQoE Reporting Path Response | O |  | ENUMERATED (accepted, rejected, …) |  |
| >>Further RAN Visible QoE Interest Response | O |  | ENUMERATED (interested, not-interested, ...) | This IE is to indicate whether the S-NG-RAN node is interested in receiving further RAN Visible QoE reports. |
| >>Further RAN Visible QoE Reporting Path | O |  | ENUMERATED (srb4, srb5, ...) | This IE is to indicate the preferred path for further RAN Visible QoE reportring. |
| >>Preferred RAN Visible QoE Configuration | O |  | RAN Visible QoE Configuration 9.2.3.201 | The RAN Visible QoE Configuration preference of the S-NG-RAN node. |
| >>QoE Session Status | O |  | ENUMERATED (started, ended, …) | This IE indicates the status of the application session. |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEAppLayerMeas | Maximum no. of simultaneous QoE measurement configurations at a UE. In this version of the specification, the value is 16. |

-------------------------------------------Next change-------------------------------------------

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

-- Q

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

QMCConfigInfo ::= SEQUENCE {

uEAppLayerMeasInfoList			UEAppLayerMeasInfoList,

iE-Extensions					ProtocolExtensionContainer { {QMCConfigInfo-ExtIEs} }	OPTIONAL,

...

}

QMCConfigInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

UEAppLayerMeasInfoList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF UEAppLayerMeasInfo-Item

UEAppLayerMeasInfo-Item ::= SEQUENCE {

uEAppLayerMeasConfigInfo	UEAppLayerMeasConfigInfo,

iE-Extensions				ProtocolExtensionContainer { { UEAppLayerMeasInfo-Item-ExtIEs} } 	OPTIONAL,

...

}

UEAppLayerMeasInfo-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QMCCoordinationRequest ::= SEQUENCE {

mN-to-SN-QMCCoordRequestList	MN-to-SN-QMCCoordRequestList,

sN-to-MN-QMCCoordRequestList	SN-to-MN-QMCCoordRequestList,

iE-Extensions					ProtocolExtensionContainer { {QMCCoordinationRequest-ExtIEs} }	OPTIONAL,

...

}

QMCCoordinationRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MN-to-SN-QMCCoordRequestList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF MN-to-SN-QMCCoordRequestList-Item

MN-to-SN-QMCCoordRequestList-Item ::= SEQUENCE {

qOEReference					QOEReference,

qOEMeasConfigAppLayerID			QOEMeasConfAppLayerID				OPTIONAL,

measCollectionEntityIPAddress	MeasCollectionEntityIPAddress		OPTIONAL,

qoEReportingPathRequest			ENUMERATED{srb4, srb5, ...}			OPTIONAL,

rVQoEReportingPathRequest		ENUMERATED{srb4, srb5, ...}			OPTIONAL,

furtherRVQoEInterestInquiry		ENUMERATED{true, ...}				OPTIONAL,

furtherRVQoEReportingPath		ENUMERATED{true, ...}				OPTIONAL,

currentRVQoEConfig				RVQoEConfig							OPTIONAL,

qoESessionStatus				ENUMERATED{started, ended, ...}		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { MN-to-SN-QMCCoordRequestList-Item-ExtIEs} }	OPTIONAL,

...

}

MN-to-SN-QMCCoordRequestList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SN-to-MN-QMCCoordRequestList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF SN-to-MN-QMCCoordRequestList-Item

SN-to-MN-QMCCoordRequestList-Item ::= SEQUENCE {

qOEReference					QOEReference,

measCollectionEntityIPAddress	MeasCollectionEntityIPAddress		OPTIONAL,

qoEReportingPathRequest			ENUMERATED{srb4, srb5, ...}			OPTIONAL,

rVQoEReportingPathRequest		ENUMERATED{srb4, srb5, ...}			OPTIONAL,

furtherRVQoEInterestInquiry		ENUMERATED{true, ...}				OPTIONAL,

furtherRVQoEReportingPath		ENUMERATED{true, ...}				OPTIONAL,

currentRVQoEConfig				RVQoEConfig							OPTIONAL,

configReleaseIndication			ENUMERATED{rvqoe, qoeandrvqoe ...}				OPTIONAL,

qoESessionStatus				ENUMERATED{started, ended, ...}		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SN-to-MN-QMCCoordRequestList-Item-ExtIEs} }	OPTIONAL,

...

}

SN-to-MN-QMCCoordRequestList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QMCCoordinationResponse ::= SEQUENCE {

mN-to-SN-QMCCoordResponseList	MN-to-SN-QMCCoordResponseList,

sN-to-MN-QMCCoordResponseList	SN-to-MN-QMCCoordResponseList,

iE-Extensions					ProtocolExtensionContainer { {QMCCoordinationResponse-ExtIEs} }	OPTIONAL,

...

}

QMCCoordinationResponse-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MN-to-SN-QMCCoordResponseList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF MN-to-SN-QMCCoordResponseList-Item

MN-to-SN-QMCCoordResponseList-Item ::= SEQUENCE {

qOEReference					QOEReference,

qOEMeasConfigAppLayerID			QOEMeasConfAppLayerID						OPTIONAL,

qoEConfigSendingPath			ENUMERATED{srb4, srb5, ...}					OPTIONAL,

qoEReportingPathResponse		ENUMERATED{accepted, rejected, ...}			OPTIONAL,

rVQoEReportingPathResponse		ENUMERATED{accepted, rejected, ...} 			OPTIONAL,

furtherRVQoEInterestResponse	ENUMERATED{interested, not-interested, ...}	OPTIONAL,

furtherRVQoEReportingPath		ENUMERATED{srb4, srb5, ...}					OPTIONAL,

preferredRVQoEConfig			RVQoEConfig									OPTIONAL,

qoESessionStatus				ENUMERATED{started, ended, ...}				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { MN-to-SN-QMCCoordResponseList-Item-ExtIEs} }	OPTIONAL,

...

}

MN-to-SN-QMCCoordResponseList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SN-to-MN-QMCCoordResponseList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF SN-to-MN-QMCCoordResponseList-Item

SN-to-MN-QMCCoordResponseList-Item ::= SEQUENCE {

qOEReference					QOEReference,

qoEReportingPathResponse		ENUMERATED{accepted, rejected, ...}										OPTIONAL,

rVQoEReportingPathResponse		ENUMERATED{accepted, rejected, ...} 										OPTIONAL,

furtherRVQoEInterestResponse	ENUMERATED{rvqoe-reports-desired, rvqoe-reports-not-desired, ...}	OPTIONAL,

furtherRVQoEReportingPath		ENUMERATED{srb4, srb5, ...}												OPTIONAL,

preferredRVQoEConfig			RVQoEConfig																OPTIONAL,

qoESessionStatus				ENUMERATED{started, ended, ...}				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SN-to-MN-QMCCoordResponseList-Item-ExtIEs} }	OPTIONAL,

...

}

SN-to-MN-QMCCoordResponseList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SNRelatedQMCInfoAtMN ::= SEQUENCE {

sNRelatedQMCInfoAtMNList		SNRelatedQMCInfoAtMNList,

iE-Extensions					ProtocolExtensionContainer { {SNRelatedQMCInfoAtMN-ExtIEs} }	OPTIONAL,

...

}

SNRelatedQMCInfoAtMN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SNRelatedQMCInfoAtMNList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF SNRelatedQMCInfoAtMNList-Item

SNRelatedQMCInfoAtMNList-Item ::= SEQUENCE {

qOEReference					OCTET STRING (SIZE(6)),

qoERVQoEReportingPaths			QoERVQoEReportingPaths	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SNRelatedQMCInfoAtMNList-Item-ExtIEs} }	OPTIONAL,

...

}

SNRelatedQMCInfoAtMNList-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

QoERVQoEReportingPaths ::= SEQUENCE {

qoEReportingPath					ENUMERATED{srb4, srb5, ...}	OPTIONAL,

rVQoEReportingPath					ENUMERATED{srb4, srb5, ...}	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {QoERVQoEReportingPaths-ExtIEs} },

...

}

QoERVQoEReportingPaths-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RVQoEConfig ::= SEQUENCE {

availableRANVisibleQoEMetrics		AvailableRVQoEMetrics			OPTIONAL,

reportingPeriodicity				RVQoEReportingPeriodicity		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {RVQoEConfig-ExtIEs} },

...

}

RVQoEConfig-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RVQoEReportingPeriodicity ::= ENUMERATED {

ms120,

ms240,

ms480,

ms640,

ms1024,

...

}

QOEMeasConfAppLayerID ::= INTEGER (0..15, ...)

QOEMeasStatus ::= ENUMERATED {ongoing, ...}

QOEReference ::= OCTET STRING (SIZE (6))

-------------------------------------------End of changes-------------------------------------------