3GPP TSG-RAN WG3 Meeting #123	R3-240660

# Athens, Greece, 26th Feb – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.413 | CR | 0026 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on NGAP IEs that are not applicable to non-3GPP access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Deutsche Telekom, British Telecommunications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) S (adding to the sourcing companies’ CR statistics) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Rel-18 introduced some NGAP IEs that are not applicable to non-3GPP access, for example, Network Controlled Repeater Authorized IE, etc. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the NGAP IEs not applible to non-3GPP access. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear behavior on the receiver when it receives an IE not applicable to non-3GPP access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

## 5.3	Exceptions for NGAP message contents and information element coding when used for non-3GPP access

For the NGAP messages transferred between the Non-3GPP access network node and the AMF, the following exceptions to the specification in TS 38.413 [2] shall be applied:

PDU SESSION RESOURCE SETUP REQUEST message:

-	the following IEs shall be ignored, when received:

-	RAN Paging Priority IE

-	UE Aggregate Maximum Bit Rate IE (except for non-trusted non-3GPP access, trusted non-3GPP access and trusted WLAN access as specified in TS 23.501 [3]).

-	Notification Control IE included in the QoS Flow Level QoS Parameters IE

-	Alternative QoS Parameters Set List IE included in the QoS Flow Level QoS Parameters IE

-	UE Slice Maximum Bit Rate List IE

PDU SESSION RESOURCE RELEASE COMMAND message:

-	the following IEs shall be ignored, when received:

-	RAN Paging Priority IE

PDU SESSION RESOURCE MODIFY REQUEST message:

-	the following IEs shall be ignored, when received:

-	RAN Paging Priority IE

-	Notification Control IE included in the QoS Flow Level QoS Parameters IE

-	Alternative QoS Parameters Set List IE included in the QoS Flow Level QoS Parameters IE

INITIAL CONTEXT SETUP REQUEST	message:

-	the following IEs shall be ignored, when received:

-	Core Network Assistance Information for RRC INACTIVE IE

-	Trace Activation IE

-	Mobility Restriction List IE

-	UE Radio Capability IE

-	Index to RAT/Frequency Selection Priority IE

-	Emergency Fallback Indicator IE

-	RRC Inactive Transition Report Request IE

-	UE Radio Capability for Paging IE

-	Redirection for Voice EPS Fallback IE

-	Location Reporting Request Type IE

-	CN Assisted RAN Parameters Tuning IE

-	SRVCC Operation Possible IE

-	IAB Authorized IE

-	Enhanced Coverage Restriction IE

-	Extended Connected Time IE

-	UE Differentiation Information IE

-	NR V2X Services Authorized IE

-	LTE V2X Services Authorized IE

-	NR UE Sidelink Aggregate Maximum Bit Rate IE

-	LTE UE Sidelink Aggregate Maximum Bit Rate IE

-	PC5 QoS Parameters IE

-	CE-mode-B Restricted IE

-	UE User Plane CIoT Support Indicator IE

-	Management Based MDT PLMN List IE

-	UE Radio Capability ID IE

-	UE Aggregate Maximum Bit Rate IE (except for non-trusted non-3GPP access, trusted non-3GPP access and trusted WLAN access as specified in TS 23.501 [3])

-	UE Security Capabilities IE

-	Time Synchronisation Assistance Information IE

-	QMC Configuration Information IE

-	Target NSSAI Information IE

-	UE Slice Maximum Bit Rate List IE

-	5G ProSe Authorized IE

-	5G ProSe UE PC5 Aggregate Maximum Bit Rate IE

-	5G ProSe PC5 QoS Parameters IE

-	Network Controlled Repeater Authorized IE

-	Aerial UE Subscription Information IE

-	NR A2X Services Authorized IE

-	LTE A2X Services Authorized IE

-	NR A2X UE PC5 Aggregate Maximum Bit Rate IE

-	LTE A2X UE PC5 Aggregate Maximum Bit Rate IE

-	A2X PC5 QoS Parameters IE

-	Mobile IAB Authorized IE

-	if this is the first message received from a new AMF, the N3IWF shall identify the old AMF and the UE using the received RAN UE NGAP ID, release the UE-associated logical NG-connection to the old AMF and create a new UE-associated logical NG-connection to the new AMF.

-	RG Level Wireline Access Characteristics IE: the information given within this IE between the W-AGF and the AMF shall be stored in the UE context by the W-AGF as specified in TS 23.316 [6].

UE CONTEXT RELEASE COMPLETE message:

-	the following IEs shall be ignored, when received:

-	Information on Recommended Cells and RAN Nodes for Paging IE

-	Paging Assistance Data for CE Capable UE IE

UE CONTEXT MODIFICATION REQUEST message:

-	the following IEs shall be ignored, when received:

-	RAN Paging Priority IE

-	Index to RAT/Frequency Selection Priority IE

-	Core Network Assistance Information IE

-	Emergency Fallback Indicator IE

-	RRC Inactive Transition Report Request IE

-	CN Assisted RAN Parameters Tuning IE

-	SRVCC Operation Possible IE

-	IAB Authorized IE

-	NR V2X Services Authorized IE

-	LTE V2X Services Authorized IE

-	NR UE Sidelink Aggregate Maximum Bit Rate IE

-	LTE UE Sidelink Aggregate Maximum Bit Rate IE

-	PC5 QoS Parameters IE

-	UE Radio Capability ID IE

-	UE Aggregate Maximum Bit Rate IE (except for non-trusted non-3GPP access, trusted non-3GPP access and trusted WLAN access as specified in TS 23.501 [3])

-	UE Security Capabilities IE

-	Time Synchronisation Assistance Information IE

-	QMC Configuration Information IE

-	QMC Deactivation IE

-	UE Slice Maximum Bit Rate List IE

-	Management Based MDT PLMN Modification List IE

-	5G ProSe Authorized IE

-	5G ProSe UE PC5 Aggregate Maximum Bit Rate IE

-	5G ProSe PC5 QoS Parameters IE

-	Network Controlled Repeater Authorized IE

-	Aerial UE Subscription Information IE

-	NR A2X Services Authorized IE

-	LTE A2X Services Authorized IE

-	NR A2X UE PC5 Aggregate Maximum Bit Rate IE

-	LTE A2X UE PC5 Aggregate Maximum Bit Rate IE

-	A2X PC5 QoS Parameters IE

-	Mobile IAB Authorized IE

-	if this is the first message received from a new AMF, the N3IWF shall identify the old AMF and the UE using the received RAN UE NGAP ID, release the UE-associated logical NG-connection to the old AMF and create a new UE-associated logical NG-connection to the new AMF.

-	RG Level Wireline Access Characteristics IE: the information given within this IE between the W-AGF and the AMF shall be stored in the UE context by the W-AGF as specified in TS 23.316 [6].

UE CONTEXT MODIFICATION RESPONSE message:

-	the following IEs shall be ignored, when received:

-	RRC State IE

INITIAL UE MESSAGE message:

-	the following IEs shall be ignored, when received:

-	IAB Node Indication IE

-	CE-mode-B Support Indicator IE

-	LTE-M Indication IE

-	EDT Session IE

-	NPN Access Information IE

-	RedCap Indication IE

-	Mobile IAB Node Indication IE

-	RRC Establishment Cause IE: the information given within this IE is to indicate the Establishment cause for non-3GPP access as specified in TS 24.502 [7].

-	Selected PLMN Identity IE: the information given within this IE provides the selected PLMN ID for untrusted non-3GPP access as specified in TS 23.502 [4].

-	Authenticated Indication IE: the information given within this IE between the W-AGF and the AMF is to indicate that the FN-RG has been authenticated by the wireline 5G access network as specified in TS 23.316 [6].

-	Selected PLMN Identity IE: the information given within this IE contains the PLMN Identity for wireline access as specified in TS 23.316 [6], or for trusted non-3GPP access as specified in TS 23.502 [4].

DOWNLINK NAS TRANSPORT message:

-	the following IEs shall be ignored, when received:

-	RAN Paging Priority IE

-	Mobility Restriction List IE

-	Index to RAT/Frequency Selection Priority IE

-	SRVCC Operation Possible IE

-	Enhanced Coverage Restriction IE

-	Extended Connected Time IE

-	UE Differentiation Information IE

-	CE-mode-B Restricted IE

-	UE Radio Capability IE

-	UE Capability Info Request IE

-	End Indication IE

-	UE Radio Capability ID IE

-	UE Aggregate Maximum Bit Rate IE (except for non-trusted non-3GPP access, trusted non-3GPP access and trusted WLAN access as specified in TS 23.501 [3])

-	Target NSSAI Information IE

UPLINK NAS TRANSPORT message:

-	TNGF Identity Information IE: the information given within this IE between the TNGF and the AMF contains a list of identifiers of NG-U terminations at TNGF as specified in TS 23.502 [4].

-	TWIF Identity Information IE: the information given within this IE between the TWIF and the AMF contains a list of identifiers of NG-U terminations at TWIF as specified in TS 23.502 [4].

-	W-AGF Identity Information IE: the information given within this IE between the W-AGF and the AMF contains a list of identifiers of NG-U terminations at W-AGF as specified in TS 23.316 [6].

NG SETUP REQUEST message:

-	the following IEs shall be ignored, when received:

-	Default Paging DRX IE

-	NB-IoT Default Paging DRX IE

NG SETUP RESPONSE message:

-	the following IEs shall be ignored, when received:

-	IAB Supported IE

-	Mobile IAB Supported IE

RAN CONFIGURATION UPDATE message:

-	the following IEs shall be ignored, when received:

-	Default Paging DRX IE

-	NB-IoT Default Paging DRX IE

OVERLOAD START message:

-	AMF Overload Response IE: if the Overload Action IE is included, the contained information is used to identify the related signalling traffic corresponding to the Establishment cause for non-3GPP access as specified in TS 24.502 [7].

-	Slice Overload Response IE: if the Overload Action IE is included, the contained information is used to identify the related signalling traffic corresponding to the Establishment cause for non-3GPP access as specified in TS 24.502 [7].

The Global RAN Node ID IE in the applicable NGAP messages includes the following IEs as specified in TS 38.413 [2]:

-	Global N3IWF ID IE for the untrusted non-3GPP access.

-	Global TNGF ID IE for the trusted non-3GPP access.

-	Global TWIF ID IE for the trusted WLAN access.

-	Global W-AGF ID IE for the wireline 5G access.

The User Location Information IE in the applicable NGAP messages includes the following IEs as specified in TS 38.413 [2]:

-	N3IWF User Location Information IE for the untrusted non-3GPP access.

-	TNGF User Location Information IE for the trusted non-3GPP access.

-	TWIF User Location Information IE for the trusted WLAN access.

-	W-AGF User Location Information IE for the wireline 5G access.

The Security Key IE in the applicable NGAP messages includes the KN3IWF, or the KTNGF, or the KTWIF, or the KWAGF as specified in TS 33.501 [5].

The RAN UE NGAP ID IE in the applicable NGAP messages identifies the UE association over the NG interface within the N3IWF node, or the TNGF node, or the TWIF node, or the W-AGF node, as specified in TS 38.413 [2].

End of Change