3GPP TSG RAN3 Meeting#123	R3-240676

Athens, Greece, 26 February - 1 March, 2024

# Title:	Reply LS on Preparation of NG-RAN resources to serve the alternative S-NSSAI

# Response to:	(R3-240020/S2-2313514), Response to Reply LS on Support of network slices which have area of service not matching deployed tracking areas
(R3-235030/S5-235774) LS on Response to Reply LS on Support of network slices which have area of service not matching deployed tracking areas

# Release:	Rel-18

# Work Item:	eNS_Ph3

# Source:	RAN3

# To:	SA2, SA5

# Cc:	-

Contact Person:

Name:	Angelo Centonza

Tel. Number:

E-mail Address:	Angelo.Centonza@Ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: Endorsed CR to TS38.300

1. Overall Description:

RAN3 thanks SA2 and SA5 for their responses to the LS on Support of network slices which have area of service not matching deployed tracking areas.

RAN3 asked feedback on three approaches concerning how to handle resources for Alternative S-NSSAIs, which are summarised below:

Approach 1: The resources associated to the RRMPolicyRatio(s) (See TS28.541) of which the alternative S-NSSAI is member are adjusted (by OAM) in order to take into account the additional load potentially generated by remapping one or more slices into the alternative S-NSSAI.

Approach 2: By retrieving data from the OAM and making them available to the AMF, the AMF can determine the RRMPolicyRatio utilisation for the resources of potential Alternative S-NSSAIs. In this way, an appropriate Alternative S-NSSAIs can be selected, whose RRMPolicyRatio(s) has free resources. The AMF/SMF is expected to select different Alternative S-NSSAIs on a per UE basis to achieve load distribution.

Approach 3: Coordination is taken to ensure that both the original slice and the one or more potential remapped slices are member of the same RRMPolicyRatio(s) (See TS28.541). This ensures that the resources that would have been used at RAN for the original slice may be used instead for the alternative slice.

From the replies received from SA2 and SA5, RAN3 deduces that Approach 2 is not feasible, while Approach 1 and Approach 3 are feasible, have no system impacts and are supported by legacy procedures.

On the basis of this, RAN3 has endorsed the attached CR to TS38.300 capturing Approach 1 and Approach 3.

2. Actions:

To SA2 and SA5:

ACTION: 	RAN3 kindly asks SA2 and SA5 to take the above into consideration.

3. Date of Next TSG SA WG2 Meetings:

TSG-RAN WG3 Meeting RAN3#123bis			15 - 19 April 2024		Changsha, CN

TSG-RAN WG3 Meeting RAN3#124			20 - 24 May 2024		Fukuoka City, JP