3GPP TSG-RAN WG3 #123	R3-240684

# Athens, Greece, 26th February– 1st March 2024

Agenda Item:	8.1

Source:	Ericsson

Title: 	Discussion on LS on NG-RAN receiving a GTP-U Error Indication

Document for:	Discussion and Approval

# 1	Background

A new incoming LS from CT4 is received, refer to [1].

CT4 discussed the following requirements in clause 5.3.3.1 of 3GPP TS 23.527 for the case when the 5G-AN receives a GTP-U Error Indication and agreed that it would be beneficial that the NG-RAN can indicate to the SMF that the release is due to receiving a GTP-U Error Indication from the NG-U tunnel, so the SMF can use this information to determine to re-establish the PFCP session for the affected PDU session. Upon receipt of a GTP-U Error Indication, the 5G-AN shall proceed as follows: - if the GTP-U Error Indication was received from an UPF over a NG-U tunnel that is not an indirect forwarding tunnel, the 5G-AN shall initiate a PDU Session Resource Notify procedure and release immediately the resources of the PDU session for which the Error Indication was received; ACTION: CT4 kindly asks RAN3 to take the above information into consideration and update their specification as necessary.

This paper discusses the RAN3 action related to the above CT4 requirement.

# 2	Discussion

## 2.1	PDU Session Resource Notify

NG-RAN node may use PDU session resource notify to release the PDU session. As a simple approach, a new cause can be introduced to indicate the GTP-U Error.

It is even preferrable to make an explicit indication.

If the cause code is updated, we should specify that the cause code is mandatory to use in the case of such failure, for example, to specify that:

“When the release of the PDU session resource is due to receiving of GTP-U Error Indication(s) from the UPF over an NG-U tunnel, the NG-RAN should set the cause to “Received GTP-U Error Indication”

Proposal 1: It is proposed that RAN3 discuss how to revise the PDU session resource notify.

Proposal 2: The cause value approach is submitted in [2]. The indication approach is submitted in [3]

## 2.2	When multiple GTP-U tunnels are established for same PDU session

More than one GTP-U tunnels may be established for the same PDU session, for example:

- PDU session is split into two parts

- transport of a QoS flow is made redundant and carried over separate GTP-U paths

When the GTP-U tunnel error is detected, it is not always correct to release the PDU session resource. For example, if the GTP-U for the redundant QoS flow failed, it would be appropriate to notify SMF about the failure, and keep the PDU session to ensure the service continuality.

If the PDU session is split and the SN experienced the GTP-U error, it is more appropriate to release only the QoS flow in the PDU session that are handled by SN.

Proposal 3: It is proposed that RAN3 to discuss and agree that NG-RAN node could notify the SMF the User Plane path failure, instead of releasing the whole PDU session resource, [4].

In gNB CP and UP split situation, the gNB-CU-UP needs to notify the gNB-CU-CP that the GTP-U error has occurred. A suitable message with minor specification impact is the context release request.

Proposal 4: It is proposed that RAN3 to discuss and agree that gNB-CU-UP notify the gNB-CU-CP the User Plane path failure, [5].

A draft LS reply is proposed to indicate to CT4 that:

- NG-RAN node indicates to SMF when PDU session is released due to GTP-U Error;

- NG-RAN node indicates to SMF that the QoS flows associated to the GTP-U Error are released.

Proposal 5: It is proposed that RAN3 to discuss the LS reply to CT4 as submitted in [6].

# 3	Proposals

Proposal 1: It is proposed that RAN3 discuss how to revise the PDU session resource notify.

Proposal 2: The cause value approach is submitted in [2]. The indication approach is submitted in [3]

Proposal 3: It is proposed that RAN3 to discuss and agree that NG-RAN node could notify the SMF the User Plane path failure, instead of releasing the whole PDU session resource, refer to [4]

Proposal 5: It is proposed that RAN3 to discuss the LS reply to CT4 as submitted in [6].

# 4	Reference

[1]	C4-235602	LS on NG-RAN receiving a GTP-U Error Indication

[2]	R3-240685	NG-RAN notify SMF when a GTP-U Error Indication is received, Ericsson,

[3]	R3-240686	NG-RAN indicate to SMF when a GTP-U Error Indication is received, Ericsson,

[4]	R3-240687	When one of the GTP-U tunnel within the PDU session receives GTP-U Error Indication, Ericsson

[5]	R3-240688	gNB-CU-UP indicates to gNB-CU-CP about the GTP-U Error, Ericsson

[6]	R3-240689	[Draft]LS Reply on NG-RAN receiving a GTP-U Error Indication, Ericsson