3GPP TSG-RAN WG3 #123	R3-240692

# Athens, Greece, 26th February- 1st March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.413 | CR | 0029 | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Including the tracing functionality in N3IWF for signalling based 5GC trace activation mechanism |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA5 in R3-240055 LS on Trace functionality extension in N3IWF for non-3GPP access scenarios informed RAN3 their agreement on including the missing tracing functionality in N3IWF for signalling based 5GC trace activation mechanism. RAN3 is requested to update specification accordingly. The change is from Rel-15. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the Trace Activation IE from the list of to be ignored IEs when INITIAL CONTEXT SETUP REQUEST message is received. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). The CR is backwards compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5.3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Incorrect specification |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.3	Exceptions for NGAP message contents and information element coding when used for non-3GPP access

For the NGAP messages transferred between the Non-3GPP access network node and the AMF, the following exceptions to the specification in TS 38.413 [2] shall be applied:

***************Skip the unchanged text *******************

INITIAL CONTEXT SETUP REQUEST	message:

-	the following IEs shall be ignored, when received:

-	Core Network Assistance Information for RRC INACTIVE IE

-	Mobility Restriction List IE

-	UE Radio Capability IE

-	Index to RAT/Frequency Selection Priority IE

-	Emergency Fallback Indicator IE

-	RRC Inactive Transition Report Request IE

-	UE Radio Capability for Paging IE

-	Redirection for Voice EPS Fallback IE

-	Location Reporting Request Type IE

-	CN Assisted RAN Parameters Tuning IE

-	SRVCC Operation Possible IE

-	IAB Authorized IE

-	Enhanced Coverage Restriction IE

-	Extended Connected Time IE

-	UE Differentiation Information IE

-	NR V2X Services Authorized IE

-	LTE V2X Services Authorized IE

-	NR UE Sidelink Aggregate Maximum Bit Rate IE

-	LTE UE Sidelink Aggregate Maximum Bit Rate IE

-	PC5 QoS Parameters IE

-	CE-mode-B Restricted IE

-	UE User Plane CIoT Support Indicator IE

-	Management Based MDT PLMN List IE

-	UE Radio Capability ID IE

-	UE Aggregate Maximum Bit Rate IE (except for non-trusted non-3GPP access, trusted non-3GPP access and trusted WLAN access as specified in TS 23.501 [3])

-	UE Security Capabilities IE

-	Time Synchronisation Assistance Information IE

-	QMC Configuration Information IE

-	Target NSSAI Information IE

-	UE Slice Maximum Bit Rate List IE

-	5G ProSe Authorized IE

-	5G ProSe UE PC5 Aggregate Maximum Bit Rate IE

-	5G ProSe PC5 QoS Parameters IE-	RG Level Wireline Access Characteristics IE: the information given within this IE between the W-AGF and the AMF shall be stored in the UE context by the W-AGF as specified in TS 23.316 [6].

-	Notification Control IE included in the QoS Flow Level QoS Parameters IE

-	Alternative QoS Parameters Set List IE included in the QoS Flow Level QoS Parameters IE