3GPP TSG-RAN WG3 Meeting #123	R3-240714

# Athens, Greece, 26th Feb – 1st Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0138 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Time Stamp and FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The time stamp IE is not aligned between TS 38.473 and TS 38.455. In TS 38.473, the time stamp supports a SCS of 480 kHz and 960 kHz while the TS 38.455 is not supported. The FR2 IE within the Requested SRS Transmission Characteristics IE is not aligned between TS 38.473 and TS 38.455. In TS 38.473, the FR Bandwidth supports 800mHz, 1600mHz and 2000mHz while the TS 38.455 is not supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add SCS 480 kHz and SCS 960 kHz to the Time Stamp IE in TS 38.455 Add 800mHz, 1600mHz and 2000mHz to FR2 IE in TS 38.455 Impact analysis: Impact assessment towards the previous version of the specification (same release): This CR does not have impact from protocol point of view. This CR may have an impact from functional point of view due to the addition of slot index for the time stamp and the Requested SRS Transmission Characteristics. The impact can be considered isolated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Time Stamp IE and FR2 IE in F1-AP and NRPPa are not aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.27, 9.2.42, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Alignement with RRC and LPPa for the Time Stemp might be needed |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

### 9.2.27	Requested SRS Transmission Characteristics

This IE contains the requested SRS configuration for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Number Of Periodic Transmissions | C-ifResourceTypePeriodic |  | INTEGER (0..500,…) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of periodic SRS transmissions. | - |  |
| Resource Type | M |  | ENUMERATED (periodic, semi-persistent, aperiodic, …) |  | - |  |
| CHOICE Bandwidth | M |  |  |  | - |  |
| >FR1 |  |  | ENUMERATED (5mHz, 10mHz, 20mHz, 40mHz, 50mHz, 80mHz, 100mHz, ...) |  |  |  |
| >FR2 |  |  | ENUMERATED (50mHz, 100mHz, 200mHz, 400mHz,… , 800mHz, 1600mHz, 2000mHz) |  |  |  |
| SRS Resource Set List |  | 0.. 1 |  |  | - |  |
| >SRS Resource Set Item |  | 1..< maxnoSRS-ResourceSets> |  |  | - |  |
| >>Number of SRS Resources Per Set | O |  | INTEGER (1..16,...) | The number of SRS Resources per resource set for SRS transmission. | - |  |
| >>Periodicity List |  | 0.. 1 |  |  | - |  |
| >>>Periodicity List Item |  | 1..<maxnoSRS-ResourcePerSet> |  |  | - |  |
| >>>>PeriodicitySRS | M |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) | Milli-seconds | - |  |
| >>Spatial Relation Information | O |  | 9.2.34 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. | - |  |
| >>Pathloss Reference Information | O |  | 9.2.53 |  | - |  |
| >>Spatial Relation Information per SRS Resource | O |  | 9.2.60 |  | - |  |
| SSB Information | O |  | 9.2.54 |  | - |  |
| SRS Frequency | O |  | INTEGER(0..3279165) | NR ARFCN The carrier frequency of SRS transmission bandwidth. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifResourceTypePeriodic | This IE shall be present if the Resource Type IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-ResourceSets | Maximum no of requested SRS Resource Sets for SRS transmission. Value is 16. |
| maxnoSRS-ResourcePerSet | Maximum no of SRS Resources per set. Value is 16. |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.2.42	Time Stamp

This information element contains the time stamp.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| System Frame Number | M |  | INTEGER(0..1023) |  | - |  |
| CHOICE Slot Index | M |  |  |  | - |  |
| >SCS-15 |  |  | INTEGER(0..9) |  | - |  |
| >SCS-30 |  |  | INTEGER(0..19) |  | - |  |
| >SCS-60 |  |  | INTEGER(0..39) |  | - |  |
| >SCS-120 |  |  | INTEGER(0..79) |  | - |  |
| >SCS-480 | M |  | INTEGER(0..319) |  | YES | reject |
| >SCS-960 | M |  | INTEGER(0..639) |  | YES | reject |
| Measurement time | O |  | Relative Time 1900 9.2.36 |  | - |  |

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

id-SRSPortIndex,

id-UETxTimingErrorMargin,

id-nrofSymbolsExtended,

id-repetitionFactorExtended,

id-StartRBHopping,

id-StartRBIndex,

id-transmissionCombn8,

id-ExtendedResourceSymbolOffset,

id-Mobile-TRP-LocationInformation,

id-Mobile-IAB-MT-UE-ID,

id-MobileAccessPointLocation,

id-CommonTAParameters,

id-SCS-480,

id-SCS-960

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

-- B

BandwidthSRS ::= CHOICE {

fR1		ENUMERATED {mHz5, mHz10, mHz20, mHz40, mHz50, mHz80, mHz100, ...},

fR2		ENUMERATED {mHz50, mHz100, mHz200, mHz400, ..., mhz800, mHz1600, mHz2000 },

choice-extension		ProtocolIE-Single-Container { { BandwidthSRS-ExtIEs } }

}

BandwidthSRS-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

-- S

SCS-480	::= INTEGER(0..319)

SCS-960	::= INTEGER(0..639)

SCS-SpecificCarrier ::= SEQUENCE {

offsetToCarrier                     INTEGER (0..2199,...),

subcarrierSpacing                   ENUMERATED {kHz15, kHz30, kHz60, kHz120,..., kHz480, kHz960},

carrierBandwidth                    INTEGER (1..275,...),

iE-Extensions					ProtocolExtensionContainer { { SCS-SpecificCarrier-ExtIEs } } OPTIONAL,

...

}

SCS-SpecificCarrier-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

TimeStamp ::= SEQUENCE {

systemFrameNumber		SystemFrameNumber,

slotIndex				TimeStampSlotIndex,

measurementTime			RelativeTime1900	OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { TimeStamp-ExtIEs} }	OPTIONAL,

...

}

TimeStamp-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TimeStampSlotIndex ::= CHOICE {

sCS-15			INTEGER(0..9),

sCS-30			INTEGER(0..19),

sCS-60			INTEGER(0..39),

sCS-120			INTEGER(0..79),

choice-extension		ProtocolIE-Single-Container { { TimeStampSlotIndex-ExtIEs} }

}

TimeStampSlotIndex-ExtIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-SCS-480		CRITICALITY reject	TYPE SCS-480 PRESENCE mandatory}|

{ ID id-SCS-960		CRITICALITY reject	TYPE SCS-960 PRESENCE mandatory},

...

}

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unchanged Text Omitted >>>>>>>>>>>>>>>>>>>>

id-MeasurementPeriodicityNR-AoA									ProtocolIE-ID ::= 105

id-SRSTransmissionStatus										ProtocolIE-ID ::= 106

id-nrofSymbolsExtended											ProtocolIE-ID ::= 107

id-repetitionFactorExtended										ProtocolIE-ID ::= 108

id-StartRBHopping												ProtocolIE-ID ::= 109

id-StartRBIndex													ProtocolIE-ID ::= 110

id-transmissionCombn8											ProtocolIE-ID ::= 111

id-ExtendedResourceSymbolOffset									ProtocolIE-ID ::= 112

id-NewNRCGI														ProtocolIE-ID ::= 113

id-Mobile-TRP-LocationInformation									ProtocolIE-ID ::= 114

id-Mobile-IAB-MT-UE-ID											ProtocolIE-ID ::= 115

id-MobileAccessPointLocation										ProtocolIE-ID ::= 116

id-CommonTAParameters											ProtocolIE-ID ::= 117

id-SCS-480														ProtocolIE-ID ::= XXX

id-SCS-960														ProtocolIE-ID ::= YYY

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>