3GPP TSG-RAN WG3 Meeting #123	R3-240728

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Agenda item:	8.3

Source: 	Huawei

Title: 	TP to technically endorsed draft CR to 37.320 on the user consent for trace reporting

Document for:	Approval

1. Introduction

This TP tries to reflect the proposal in the discussion paper [1].

2. TP

Start of Change

5.1.3	MDT Initiation

There are two cases that RAN should initiate a MDT measurements collection task. One is that the MDT task is initiated without targeting a specific UE by the cell traffic trace, i.e. management based trace function from OAM. The other is that the MDT task is initiated towards a specific UE by the signalling trace activation messages from CN nodes, i.e. the Initial Context Setup message, the Trace Start message or the Handover request message in E-UTRAN or NR, the CN Invoke Trace message in UTRAN. The detailed procedures to transfer the MDT configurations to RAN are specified in TS 32.422 [6].

For signalling based MDT within a PLMN, the CN shall not initiate MDT towards a particular user unless it is allowed.

For management based MDT within a PLMN, the CN indicates to the RAN whether MDT is allowed to be configured by the RAN for this user considering e.g. user consent and roaming status (see TS 32.422 [6]), by providing management based MDT allowed information. For MDT measurement names that are not subject to user consent, the RAN may initiate MDT measurement towards a particular UE independently of user consent. For E-UTRAN/UTRAN, the MDT allowed information consists of the Management Based MDT Allowed indication and optionally the Management Based MDT PLMN List. For NR, the MDT allowed information only consists of the Management Based MDT PLMN List. The management based MDT allowed information propagates during inter-PLMN handover or inter-PLMN UE context retrieval if the Management Based MDT PLMN List is available and includes the target PLMN.

A UE is configured with an MDT PLMN List if user consent is valid for the RPLMN, or if none of the MDT measurement names of the configured MDT measurements is subject to user consent.

User consent does not apply if the UE is served by an SNPN.

End of Change

3. Reference

[1] R3-240727, Further discussions on the user consent for trace reporting, Huawei