3GPP TSG-RAN WG3 Meeting #123	R3-240734

Athens, Greece, 26 February – 01 March, 2024

Agenda item:	9.1.3.1

Source:	Nokia, Nokia Shanghai Bell, Deutsche Telekom, T-Mobile, Lenovo

Title:	Discussion on corrections related to measurement collection for cell-based UE trajectory and UE performance feedback

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we provide some updates related to measurement collection for cell-based UE trajectory and UE performance triggered by a Handover in order to clarify stage 3 text and align it to stage 2.

# 2	Discussion

TS 38.300 we have described in high-level that collection and reporting of UE performance feedback and UE Trajectory information are configured through the Data Collection Reporting Initiation procedure and that their collection is triggered at successful Handover execution. The term “completion of handover procedure” has not so far been used in TS 38.423 or TS 38.300. However, TS 38.300 defines the term “successful handover”, and it therefore seems appropriate to refer to this term also in stage 3.

Proposal 1: Update the text in TS 38.423 to refer to “successful handover” as the trigger to start UE trajectory and UE performance feedback measurement collection for alignment with TS 38.300.

The IE Measured Trajectory Cell Information contains a list of visited cells and a time of stay of a UE to each cell after the UE has been handed over to the target NG-RAN node. Both the cell identity and the time UE stayed in the cell are mandatory fields in the measured trajectory cell information. However, the description of this IE is missing the time angle.

Proposal 2: Update the description of Measured Trajectory Cell Information IE to include also the time UE stayed in each visited cell.

In Measured Trajectory Cell Information IE, the unit is missing from the Time UE Stayed in Cell IE.

Proposal 3: Include in the semantics description of Time UE Stayed in Cell IE  that it is measured in seconds.

In Rel-18 we have agreed to introduce in the DATA COLLECTION REQUEST message two IEs related to configurations of UE trajectory and UE performance feedback data collection, namely UE Trajectory Collection Configuration IE and UE Performance Collection Configuration IE. In both those IEs, there is a Collection Time Duration IE. However, the two descriptions are not aligned.

Proposal 4: Align the semantics description of Collection Time Duration IE between UE Trajectory Collection Configuration IE and UE Performance Collection Configuration IE.

# 3	Conclusion

In this contribution we make the following proposals:

Proposal 1: Update the text in TS 38.423 to refer to “successful handover” as the trigger to start UE trajectory and UE performance feedback measurement collection for alignment with TS 38.300.

Proposal 2: Update the description of Measured Trajectory Cell Information IE to include also the time UE stayed in each visited cell.

Proposal 3: Include in the semantics description of Time UE Stayed in Cell IE  that it is measured in seconds.

Proposal 4: Align the semantics description of Collection Time Duration IE between UE Trajectory Collection Configuration IE and UE Performance Collection Configuration IE.