| # 3GPP TSG-RAN WG3 Meeting #123 R3-240746 # Athens, Greece, 26th Feb - 1st March 2024 CR-Form-v12.2. CHANGE REQUEST. 38.423; CR; 1225; rev; -; Current version:; 18.0.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; Radio Access Network; X; Core Network. |
| --- |
| Title: | ASN.1 Correction on XnAP for Time resilience and URLLC |  |  |  |  |  |  |  |  |  |
| Source to WG: | ZTE, China Telecom, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC-NR-Core |  |  |  |  |  | Date: |  |  | 2024-2-1 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The ASN.1 part of Clock Quality Acceptance Criteria IE is not aligned with the tabular—— in the tabular, the codepoints (e.g., syncE, pTP, etc.) are defined for the Parent Time Source IE， while those code points are added for clockFrequencyStability in the ASN.1 part. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the code points for Clock Frequency Stability IE and Parent Time Source in Clock Quality Acceptance Criteria in ASN.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ASN.1 part is not aligned with the tabular. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/CR ...CR... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Changes Start

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<unchanged text omitted>

ClockQualityAcceptanceCriteria ::= SEQUENCE {

synchronisationState			BIT STRING

{

locked(0), holdover(1), freeRun(2)

}	(SIZE(8, ...))						OPTIONAL,

traceabletoUTC					ENUMERATED {true, ...}						OPTIONAL,

traceabletoGNSS					ENUMERATED {true, ...}						OPTIONAL,

clockFrequencyStability			BIT STRING (SIZE (16))

OPTIONAL,

clockAccuracy					INTEGER (1..40000000, ...)					OPTIONAL,

parentTImeSource				BIT STRING

{

syncE (0), pTP (1), gNSS (2), atomicClock (3), terrestrialRadio (4),

serialTimeCode (5), nTP (6), handset (7), other (8)

}	(SIZE(16, ...))						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { ClockQualityAcceptanceCriteria-ExtIEs} }	OPTIONAL,

...

}

End of Changes