3GPP TSG-RAN WG3 Meeting #123	R3-240788

Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.320 | CR | draftCR | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction user consent |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During the last RAN3#122 meeting in Chicago, USA, R3-237963, on corrections to TS37.320 related to user consent for trace reporting, was technically endorsed. This CR was not agreed as RAN3 was waiting for SA5 and SA2 to reply to the LS in R3-237964 sent by RAN3. In the meantime, SA2 and SA5 have replied in favour of the solution proposed by RAN3 in R3-237964, hence R3-237963 could be agreed. Nevertheless, certain parts of the text in R3-237963 are confusing and do not reflect the overall agreement in RAN3 which was: “-For MDT measurements that are not subject to user consent, the RAN may initiate MDT towards a particular UE independently of user consent.”, as captured by the chair notes on the agreements related to the user consent discussion. While the agreements in RAN3 indicate that any measurement not subject to user consent can be initiated towards a UE, independently on whether the user gave consent or not, the text in R3-237963 states that such measurements can be initiated towards a UE only if the MDT configuration does not contain any measurement subject to user consent. Moreover, the text in R3-237963 describes what has been captured by SA5 in the stage 2 specification TS32.422. The latter creates duplication of specifications and a future risk of discrepancies between specifications. The reason for change is therefore to provide a new CR introducing the MDT user consent solution agreed by RAN3 in TS37.320. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update TS37.320 so that it reflects accurately current agreements on user consent., namely that “For MDT measurements that are not subject to user consent, the RAN may initiate MDT towards a particular UE independently of user consent”. When appropriate, reference to TS32.422 instead of describing the same procedures again, to avoid out of sync issues. Impact assessment towards the previous version of the specification (same release): This CR has no impact on the protocol. This CR has no functionality impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to initiate MDT measurements towards a UE even if the measurements are not subject to user consent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 5.1.3	MDT Initiation

There are two cases that RAN should initiate a MDT measurements collection task. One is that the MDT task is initiated without targeting a specific UE by the cell traffic trace, i.e. management based trace function from OAM. The other is that the MDT task is initiated towards a specific UE by the signalling trace activation messages from CN nodes, i.e. the Initial Context Setup message, the Trace Start message or the Handover request message in E-UTRAN or NR, the CN Invoke Trace message in UTRAN. The detailed procedures to transfer the MDT configurations to RAN are specified in TS 32.422 [6].

For signalling based MDT measurements subject to user consent, within a PLMN, the CN shall not initiate MDT towards a particular user unless it is allowed. User consent is not considered for all other MDT measurements.

For management based MDT measurements subject to user consent within a PLMN, the CN indicates to the RAN whether MDT is allowed to be configured by the RAN for this user considering e.g. user consent and roaming status (see TS 32.422 [6]), by providing management based MDT allowed information. User consent is not considered for all other MDT measurements, namely for such measurements the CN indicates to the RAN whether MDT is allowed to be configured by the RAN for this user regardless of user consent. For E-UTRAN/UTRAN, the MDT allowed information consists of the Management Based MDT Allowed indication and optionally the Management Based MDT PLMN List. For NR, the MDT allowed information only consists of the Management Based MDT PLMN List. The management based MDT allowed information propagates during inter-PLMN handover or inter-PLMN UE context retrieval if the Management Based MDT PLMN List is available and includes the target PLMN.

For MDT measurements subject to user consent a UE is configured with an MDT PLMN List only if user consent is valid for the RPLMN, while no such constrain is applicable to all other MDT measurements.

User consent does not apply if the UE is served by an SNPN.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>