3GPP TSG-RAN3 Meeting #123	R3-240794

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

Title: 	Response to R3-240348, R3-240565

Source: 	ZTE

Agenda item:	8.1

Document for:	Discussion and Decision

- Introduction

In this meeting, RAN3 received reply LS from RAN2 in R3-240015 [1], as below.

1. Overall Description: RAN2 thanks RAN3 for the LS regarding the ‘SDT signaling optimization for partial context transfer’ in R2-2309436/R3-234589. RAN2 discussed the scenario in RAN3 LS about the DL non-SDT data arrival in case of RA-SDT without UE anchor relocation. RAN2 agrees the benefits of the proposed solution from RAN3. The solution is feasible from RAN2 perspective and has small impacts on RAN2 spec, and RAN2 decided to agree on the related CR for TS 38.331 and 38.306. 2. Actions: To RAN3: ACTION: RAN2 kindly asks RAN3 to take above response into account for RAN3 work.

Two discussion papers (i.e., [3] to [9]) provide two candidate optimized RAN3 solutions to address the issue indicated in this incoming LS.

This response paper analyzes whether optimized solutions are suitable, and then provides observations and proposals.

- Discussion

  - RAN2 solution

The RAN2 CRs agreed to avoid the paging procedure by RAN2 spec enhancement.

In detail, in case of DL non-SDT data/signaling or DL large size of SDT data/signaling arrival, the anchor gNB needs to check UE capability, if supported, it sends a new flag (i.e., resumeIndication) included in the RRCRelease container by Xn retrieve context failure message, as specified in TS 38.331：

resumeIndication

Indicates that the UE shall trigger the RRC connection resume procedure after receiving this RRCRelease message, as specified in clause 5.3.8.3. The network only includes this field in the RRCRelease message used to terminate an ongoing SDT procedure.

In [3], this is shown below:

Figure 1: RAN2 solution (abstracted from [3])

The advantage of the RAN2 solution is that it has no RAN3 impact. However, as indicated in [3], the drawback of the solution is that it only works for UEs supporting UE capability for extra resume bit and wastes OTA signaling.

Observation 1: RAN2 solution has no RAN3 impact but has some drawbacks (e.g., works for UEs supporting UE capability for extra resume bit, consumes OTA signaling).

  - RAN3 candidate optimized solutions

    - Optimized solution 1: Enhance XnAP signaling for non-supporting UEs

In [5], it proposes to introduce a new “non-SDT data” indication in the Xn Retrieve UE context response message, then A more optimized solution would be that anchor gNB triggers immediately anchor relocation and new serving gNB receiving a new “non-SDT data” indication in the Xn Retrieve UE context response triggers RRC setup.

However, this solution introduces much RAN3 signaling impact.

Currently, after Partial UE Context Transfer procedure, only Xn Retrieve UE context failure message can be used to end the ongoing SDT session, which has specified in TS38.423, as below.

TS38.423 Interaction with Partial UE Context Transfer procedure In case of RACH based SDT, if the old NG-RAN node decides to not transfer/relocate the UE Context to the new NG-RAN node, it may trigger the Partial UE Context Transfer procedure as specified in TS 38.300 [9]. After the old NG-RAN node has decided to end the SDT session, it shall terminate the Retrieve UE Context procedure by sending the RETRIEVE UE CONTEXT FAILURE message.

Observation 2: Optimized Solution 1 (Enhance XnAP signalling for non-supporting UEs) has impact on current interaction with Partial UE Context Transfer procedure

    - Optimized solution 2: Enhance XnAP signaling to avoid OTA signaling

In [9], it suggests to introduce a new indication in the XnAP RETRIEVE UE CONTEXT FAILURE message. Upon receiving the new indication from the anchor gNB, the Receiving gNB starts a new XnAP RETRIEVE UE CONTEXT REQUEST message again to fetch the full UE context. Once the UE’s context is fully relocated from the anchor gNB, the receiving gNB can configure the UE to send it to RRC_CONNECTED state directly, without release, avoiding RRC signalling and paging costs.

Although solution 3 can avoid OTA signalling, it will introduce security issue.

In detail, the (previous) anchor gNB has already consumed its key, when the receiving gNB starts a new XnAP RETRIEVE UE CONTEXT REQUEST message again to fetch the full UE context from the (previous) anchor gNB whose key is used, the key does not match between UE and network if UE is not involved.

Observation 3: Optimized Solution 2 (Enhance XnAP signalling to avoid OTA signalling) will introduce security problem.

  - Summary

Based on the above analysis, two optimized solution 1 and solution 2 are not suitable, we propose to fully align with RAN2 solution, and there is no RAN3 signaling impact. RAN3 can clarify it in stage 2 specs

Proposal 1: RAN3 agrees to align with RAN2 solution without any enhancement.

Proposal 2: If proposal 1 is agreed, RAN3 does not need to send reply LS to RAN2.

3. Conclusion

After the above analysis, we provide the following observations and proposals.

Observation 1: RAN2 solution has no RAN3 impact but has some drawbacks (e.g., works for UEs supporting UE capability for extra resume bit, consumes OTA signaling).

Observation 2: Optimized Solution 1 (Enhance XnAP signalling for non-supporting UEs) has impact on current interaction with Partial UE Context Transfer procedure

Observation 3: Optimized Solution 2 (Enhance XnAP signalling to avoid OTA signalling) will introduce security problem.

Proposal 1: RAN3 agrees to align with RAN2 solution without any enhancement.

Proposal 2: If proposal 1 is agreed, RAN3 does not need to send reply LS to RAN2.

4. Reference

- R3-240015 Reply LS on SDT signaling optimization for partial context transfer (RAN2(Qualcomm))	LS in

- R3-240221 Optimization on handling of DL non-SDT in case of RA-SDT without UE anchor relocation (CATT)	draftCR

- R3-240348 Switch to RRC Connected after SDT Partial Context Transfer (Nokia, Nokia Shanghai Bell)	discussion

- R3-240349 Correction of switch to RRC connected after SDT Partial Context Transfer (Nokia, Nokia Shanghai Bell)	draftCR

- R3-240350 Correction of switch to RRC connected after SDT Partial Context Transfer (Nokia, Nokia Shanghai Bell)	CR1161r, TS 38.423 v18.0.0, Rel-18, Cat. F

- R3-240351 Response LS on SDT signaling optimization for partial context transfer (Nokia, Nokia Shanghai Bell)	LS out To: RAN2 CC:

- R3-240257 Correction on SDT RRC Release with resume indication (Huawei, Qualcomm Incorporated, Lenovo)	draftCR

- R3-240258 Correction on SDT RRC Release with resume indication (Huawei, Qualcomm Incorporated, Lenovo)	CR0335r, TS 38.401 v18.0.0, Rel-18, Cat. F

- R3-240565 Discussion on SDT signalling enhancement (Ericsson)	discussion

- R3-240566 SDT context retrieval re-Request (Ericsson)	CR1190r, TS 38.423 v18.0.0, Rel-18, Cat. B

- R3-240567 Update of Note 1a in SDT procedure (Ericsson)	draftCR