3GPP TSG-RAN WG3 Meeting #123	R3-240797

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda Item:	9.1.10.1

Source:	Ericsson

Title:	Response paper to R3-240652, R3-240653, R3-240654, R3-240195, R3-240196

Document for:	Response paper

# 1	Introduction

In this contribution, we discuss the scenario mentioned by [1] and indicate our incomprehension to the issue raised.

# 2	Discussion

In [1], the following table is described.

Below our interpretation of each case:

|  | Arrived DL SDT Data Size |  | SDT Data Size Threshold |  | gNB-CU-UP solution | Spec impact |
| --- | --- | --- | --- | --- | --- | --- |
| Case A | Smaller than 96000 bytes | e.g. 6000 bytes |  |  | Sends E1AP: DL DATA NOTIFICATION message with MT-SDT Information IE, and provide the MT-SDT Data Size. | No impact |
| In this case 6000 bytes is smaller than the SDT threshold 96000, MT-SDT paging is possible. No spect impacts. |  |  |  |  |  |  |
| Case B | Larger than 96000 bytes, and larger than configured threshold | e.g. 13000 bytes | Configured | e.g. 12000 bytes | Sends E1AP: DL DATA NOTIFICATION message with MT-SDT Information IE, and provide the MT-SDT Data Size. OR Sends E1AP: DL DATA NOTIFICATION message with SDT Data Size Threshold Crossed IE | Extend the MT-SDT Data Size OR Update stage2 descriptoin. |
| In this case 13000 bytes is smaller than the SDT threshold 96000, MT-SDT paging is possible. If the gNB-CU-CP configures a threshold of Oversize SDT data to 12000 bytes, and receives 13 000 bytes data, it is up to gNB-CU-CP to decide whether to page with MT-SDT. No spect impacts. Or is it a typo and a “0” is missing?? |  |  |  |  |  |  |
| Case C | Larger than 96000 bytes, no threshold configued | e.g. 13000 bytes | Not Configued |  | Sends E1AP: DL DATA NOTIFICATION message with MT-SDT Information IE, and provide the MT-SDT Data Size. | Extend the MT-SDT Data Size |
| In this case 13000 bytes is smaller than the SDT threshold 96000, MT-SDT paging is possible. No spect impacts. Or is it a typo and a “0” is missing?? |  |  |  |  |  |  |
| Case D | Larger than 96000 bytes, and smaller than configured threshold | e.g. 10000 bytes | Configured | e.g. 12000 bytes | Sends E1AP: DL DATA NOTIFICATION message with MT-SDT Information IE, and provide the MT-SDT Data Size. | Extend the MT-SDT Data Size |
| In this case 10 000 bytes is smaller than the SDT threshold 96000 and smaller than the configured oversize threshold, MT-SDT paging is possible. No spect impacts. Or is it a typo and a “0” is missing?? |  |  |  |  |  |  |

In case there is a typo in the table above and a “0” is missing in the numbers, case C is legacy paging and in case D CU-CP receives the SDT Data Size Threshold Crossed IE and decides whether to page using MT-DT or not. Up to CU-CP implementation.

Overall, we do not understand the issue from [1]. The MD-SDT volume range has been decided by RAN2 in Rel-18 to be up to 96 000 bytes and added in SIB, it should not be overturned.

SDT-ConfigCommonSIB-r17 ::= SEQUENCE { sdt-RSRP-Threshold-r17 RSRP-Range OPTIONAL, -- Need R sdt-LogicalChannelSR-DelayTimer-r17 ENUMERATED { sf20, sf40, sf64, sf128, sf512, sf1024, sf2560, spare1} OPTIONAL, -- Need R sdt-DataVolumeThreshold-r17 ENUMERATED {byte32, byte100, byte200, byte400, byte600, byte800, byte1000, byte2000, byte4000, byte8000, byte9000, byte10000, byte12000, byte24000, byte48000, byte96000}, t319a-r17 ENUMERATED { ms100, ms200, ms300, ms400, ms600, ms1000, ms2000, ms3000, ms4000, spare7, spare6, spare5, spare4, spare3, spare2, spare1} }

Furthermore, the MT-SDT size estimate is based on how long the SDT procedure can continue only for a maximum of 4 seconds (see values of t-319a), and how much data can be transmitted within that time duration to UE to be considered SDT. Therefore, it does not make sense to extend the threshold value over XnAP and E1AP for the MT-SDT Data Size IE above the current agreed threshold, since MT-SDT will not perform well at data volumes as large as 96000 bytes…nor are there any use cases or justification for it!

We note that R3-240654 was submitted to wrong sub-AI and therefore exceeds the quota. Conribution should be withdrawn…

Proposal 1: Do not acknowledge the issue. Do not Extend the XnAP/E1AP : MT-SDT Data Size IE from INTEGER (1..96000,…) to INTEGER (1..192000,…)

Besides, we note that the sdt-DataVolumeThreshold-r17 is not extensible in RRC, therefore the ellipsis in the MT-SDT Information IE should be removed.

#### 9.3.1.142	MT-SDT Information

This IE provides the assistant information for MT-SDT.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MT-SDT Data Size | M |  | INTEGER (1..96000,…) | Indicates the total data size for all SDT bearers. Unit: byte. Corresponds to the SDAP SDU size of the received DL data. |

Proposal 2: Delete the ellipsis in the MT-SDT Information IE value range in XnAP and E1AP

# 3	Conclusion

Proposal 1: Do not acknowledge the issue. Do not Extend the XnAP/E1AP : MT-SDT Data Size IE from INTEGER (1..96000,…) to INTEGER (1..192000,…)

Proposal 2: Delete the ellipsis in the MT-SDT Information IE value range in XnAP and E1AP

# 4	Reference

[1] R3-240652 Correction on oversize DL SDT data arrival, CR to TS 38.423, Huawei

# 5	CR to TS 37.483

3GPP TSG-RAN WG3 Meeting #123	R3-24

Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.483 | CR |  | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on MT-SDT Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MT_SDT-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The SDT data size is defined as non extensible in RRC: sdt-DataVolumeThreshold-r17 ENUMERATED {byte32, byte100, byte200, byte400, byte600, byte800, byte1000, byte2000, byte4000, byte8000, byte9000, byte10000, byte12000, byte24000, byte48000, byte96000}, Therefore, the eelipsis should be removed from the IE value range |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove ellipsis from the MT-SDT Information IE value range |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with RRC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.142, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.423 CR |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------Start of the First Change---------------

#### 9.3.1.142	MT-SDT Information

This IE provides the assistant information for MT-SDT.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MT-SDT Data Size | M |  | INTEGER (1..96000) | Indicates the total data size for all SDT bearers. Unit: byte. Corresponds to the SDAP SDU size of the received DL data. |

-------------Start of the Next Change---------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CommonNetworkInstance,

id-SNSSAI,

id-OldQoSFlowMap-ULendmarkerexpected,

id-DRB-QoS,

id-endpoint-IP-Address-and-Port,

id-NetworkInstance,

id-QoSFlowMappingIndication,

id-TNLAssociationTransportLayerAddressgNBCUUP,

//skip unchanged part

MT-SDT-Information ::= SEQUENCE {

mT-SDT-Data-Size			MT-SDT-Data-Size,

iE-Extensions				ProtocolExtensionContainer { {MT-SDT-Information-ExtIEs} } OPTIONAL

}

MT-SDT-Information-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MT-SDT-Information-Request ::=	ENUMERATED {true, ...}

MT-SDT-Data-Size::= INTEGER (1..96000)

MBS-ServiceArea ::= CHOICE {

locationdependent		MBS-ServiceAreaInformationList,

choice-Extensions		ProtocolIE-SingleContainer { {MBSServiceArea-ExtIEs} }

}

MBSServiceArea-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

-------------End of the Changes---------------

# 6	CR to TS 38.423

3GPP TSG-RAN WG3 Meeting #123	R3-24

Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR |  | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on MT-SDT Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MT_SDT-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The SDT data size is defined as non extensible in RRC: sdt-DataVolumeThreshold-r17 ENUMERATED {byte32, byte100, byte200, byte400, byte600, byte800, byte1000, byte2000, byte4000, byte8000, byte9000, byte10000, byte12000, byte24000, byte48000, byte96000}, Therefore, the eelipsis should be removed from the IE value range |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove ellipsis from the MT-SDT Information IE value range |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with RRC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.172, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 37.483 CR |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------Start of the First Change---------------

#### 9.2.3.172	MT-SDT Information

This IE provides the assistance information for MT-SDT.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MT-SDT Indicator | M |  | ENUMERATED (true, ...) |  |
| MT-SDT Data Size | M |  | INTEGER (1..96000) | Indicates the total data size for DL NAS signalling and user plane data for all SDT bearers. Unit: byte. Corresponds to the PDCP SDU size for the DL NAS signalling and the SDAP SDU size for the received DL user plane data. |

-------------Start of the Next Change---------------

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

id-ConfiguredTACIndication,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-DefaultDRB-Allowed,

//skip unchanged part

MaximumCellListSize ::= INTEGER(1..16384, ...)

MT-SDT-Information ::= SEQUENCE {

mT-SDT-Indicator			MT-SDT-Indicator,

mT-SDT-DataSize			MT-SDT-DataSize,

iE-Extensions				ProtocolExtensionContainer { { MT-SDT-Information-ExtIEs} } OPTIONAL,

...

}

MT-SDT-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

MT-SDT-DataSize	::= INTEGER (1..96000)

MT-SDT-Indicator ::= ENUMERATED {true, ...}

MultiplexingInfo 	::=	SEQUENCE{

iAB-MT-Cell-List 	IAB-MT-Cell-List,

iE-Extensions		ProtocolExtensionContainer { {MultiplexingInfo-ExtIEs} } OPTIONAL,

...

}

MultiplexingInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

-------------End of the Changes---------------