3GPP TSG-RAN WG3 Meeting #123	R3-240798

Athens, Greece, February 26th – March 1st 2024

Agenda item:	9.1.2.1

Source: 	Ericsson

Title: 	Response to R3-240182

Document for:	Agreement

# Discussion

The CR to TS 38.413 in R3-240182 proposes a correction to the semantics description for the NG-RAN CGI IE within the UE Application Layer Measurement Configuration Information IE.

| >>Cell ID List for QMC |  | 1..<maxnoofCellIDforQMC> |  |  | - |  |
| --- | --- | --- | --- | --- | --- | --- |
| >>>NG-RAN CGI | M |  | 9.3.1.73 | This IE can indicate the NR CGI and E-UTRA CGI. QoE measurements for new service type are not supported in E-UTRA cell. | - |  |

As stated in TS 38.410, over NG interface, “The QMC function provides means to manage the QMC sessions and support the mobility of QMC sessions over the NG interface.”.

However, different QMC support is provided in terms of service types by an NG-RAN node, depending on whether this is a gNB, which serves NR cells, or an ng-eNB, which serves E-UTRA cells.

NR cells can support all the service types specified by the Service Type IE within the UE Application Layer Measurement Configuration Information IE in TS 38.413:

| Service Type | M |  | ENUMERATED (QMC for DASH streaming, QMC for MTSI, QMC for VR, ...) | This IE indicates the service type of QoE measurements. | - |  |
| --- | --- | --- | --- | --- | --- | --- |

On the other hand, E-UTRA cells, can only support service types specified by the Service Type IE within the UE Application layer measurement configuration IE in TS 36.413:

| Service Type | M |  | ENUMERATED (QMC for streaming service, QMC for MTSI service, ...) | This IE indicates the service type of UE application layer measurements. | - | - |
| --- | --- | --- | --- | --- | --- | --- |

Hence, the set of service types for which QMC is supported in NR cells is different from the set of service types for which QMC is supported in E-UTRA cells. In addition, RVQoE measurements and alignment with MDT are not supported in E-UTRA cells either. Therefore, instead of the correction proposed in R3-240182, it may be better to explicitly indicate in stage-2 (e.g., in TS 38.300) the service types for which QMC is supported in E-UTRA cells. A CR to TS 38.300 is presented in the Annex.

Proposal 1: Add a clarification in TS 38.300 to indicate the service types for which QMC is supported in E-UTRA cells.

Proposal 2: Agree the CR to TS 38.300 in the Annex.

# Conclusion

In this paper, we provide a response to the CR for TS 38.413 in R3-240182 and propose an alternative correction to TS 38.300.

Proposal 1: Add a clarification in TS 38.300 to indicate the service types for which QMC is supported in E-UTRA cells.

Proposal 2: Agree the CR to TS 38.300 in the Annex.

# Annex: CR for TS 38.300

3GPP TSG-RAN WG3 Meeting #123	R3-24xxxx

Athens, Greece, February 26th – March 1st 2024

| +6CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 correction of QMC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE_enh-Core |  |  |  |  |  | Date: |  |  | 2024-02-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is necessary to clarify that different service types are supported in NR cells and in E-UTRA cells. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified that different service types are supported in NR cells and in E-UTRA cells. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Imprecise specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Start of changes-------------------------------------------

## 21.1	Overview

The QoE Measurement Collection function enables collection of application layer measurements from the UE. QoE measurement collection is supported for the following service types in NR cells:

-	QoE Measurement Collection for DASH streaming;

-	QoE Measurement Collection for MTSI;

-	QoE Measurement Collection for VR.

QoE measurement collection is supported for the following service types in E-UTRA cells:

-	QoE Measurement Collection for DASH streaming;

-	QoE Measurement Collection for MTSI.

For DASH, MTSI, VR, the QoE measurement collection is supported in RRC_CONNECTED state only, unless the application data is delivered via MBS broadcast communication service.

The QoE Measurement Collection function supports collection of QoE measurements for MBS communication service. The measurement collection is supported for the following two communication service types:

-	MBS broadcast;

-	MBS multicast.

QoE measurement collection for application sessions delivered via MBS broadcast is supported in RRC_CONNECTED, RRC_INACTIVE, and RRC_IDLE states. QoE measurement collection for the application sessions delivered via MBS multicast is supported in RRC_CONNECTED state only. Both signalling based and management based QoE measurement collection are supported in NR SA and NR-DC. Further details of NR-DC operation can be found in TS 37.340 [21] and clause 21.7.

NOTE:	The naming QoE Measurement is used in NG, Xn, and interfaces between the OAM and the gNB. In the Uu interface, the naming application layer measurement is used and it is equal to QoE Measurement.

-------------------------------------------End of changes-------------------------------------------