3GPP TSG-RAN WG3 Meeting #123	R3-240901

Athens, Greece, 26th February – 1st March 2024

Agenda Item:	9.1.9.1

Source:	Lenovo (moderator)

Title:	SoD of CB: # XR2_ECNMarkingDiscard

Document for:	Approval

Introduction

This contribution provides the summary of offline discussion on:

CB: # XR2_ECNMarkingDiscard

- Check ECN marking corrections

- Discuss the open issues related to PSU Set Discard and capture agreements

(moderator - Lenovo)

Summary of offline disc R3-240901

For the Chairman’s Notes

UL PSI based discard

Proposal 1: Introduce the new PSI based discard indication with two codepoints {start, stop} in the UE context setup/modification procedure.

Proposal 2: When configuration and reconfiguration of the PSI based discard indication, codepoint ‘start’ is used. When release of the PSI based discard indication, codepoint ‘stop’ is used.

DL PSI based discard

Proposal 3: Add the DL PSI based Discard timer in the PDCP configuration IE in E1AP from gNB-CU-CP to gNB-CU-UP.

ECN Marking

Proposal 4: Add the ECN Marking or Congestion Information Reporting Status IE in the Path Switch Request Transfer IE and Handover Request Acknowledge Transfer IE.

Proposal 5: Add the description about how the gNB perform ECN marking for L4S in 38.300.

CRs

R3-240438 is revised in R3-241083 (Nokia)

R3-240497 is revised in R3-241045 (Huawei)

R3-240493 is revised in R3-241043 (Huawei)

R3-240494 is revised in R3-241044 (Huawei, Xiaomi)

Discussion

UL PSI based discard

Issue 1: Which procedure is used for the new PSI based discard indication, UE context setup/modification procedure or DL RRC message transfer procedure?

During online discussion, it was agreed that:

For UL: Introduce a per DRB new indication in F1AP from CU to DU

But it is still open for which procedure the new indication should be included:

UE context setup/modification procedure or RRC message transfer procedure?

Proposal 1: the UE context setup/modification procedure is used for the new PSI based discard indication.

At least two codepoints are needed {start or stop}. When configuration and reconfiguration of the timer, ‘start is used, when release of the PSI discard timer, ‘stop is used.

DL PSI based discard

Basic function:

Proposal 2: Add the DL PSI based Discard timer in the PDCP configuration IE in E1AP from CP to UP.

How to use the PSI based discard timer for DL PSI based discard is up to gNB-CU-UP’s implementation.

Other enhancements:

| R3-240492 | Support of PSI based discard (Huawei, Qualcomm, Lenovo) | discussion |
| --- | --- | --- |

Proposal 4: The gNB-DU can send the suggestion on DL PSI based SDU discard activation/deactivation to the gNB-CU-UP if DL PSI-based discard is configured.

| R3-240580 | Discussion on PDU Set discarding and other topics (Ericsson) | discussion |
| --- | --- | --- |

Proposal 1: Introduce PSI in F1-U DL USER DATA frame.

Proposal 2: Introduce information on transmitted PDCP PDU Set SN and  discarded PDCP PDU Set SN information in the F1-U DL Data Delivery Status frame.

Proposal 3: Introduce the PDU Set transmission status in the E1 Bearer Modification Request / Response messages and in the Xn SN Status Transfer message

Proposal 5: The gNB-DU provides the discarding status result (on, off) in the UL RRC MESSAGE TRANSFER message.

| R3-240298 | Support of DL PSI-based discard and UL Traffic Information (Xiaomi) | CR0108r, TS 37.483 v18.0.0, Rel-18, Cat. F |
| --- | --- | --- |

Proposal: Introudce a Uplink Traffic Information IE in QoS Flow QoS Parameters List IE

ECN Marking

Issue 1:

| R3-240218 | Corrections on ECN Marking (Lenovo, Huawei, CATT) | CR1065r, TS 38.413 v18.0.0, Rel-18, Cat. F |
| --- | --- | --- |
| R3-240438 | Report the ECN Marking or Congestion Information Reporting Status during Xn-HO and NG-HO (Nokia, Nokia Shanghai Bell) | CR1094r, TS 38.413 v18.0.0, Rel-18, Cat. F |

Proposal 1a: Add the ECN Marking or Congestion Information Reporting Status IE in the Path Switch Request Transfer IE and Handover Request Acknowledge Transfer IE.

Proposal 1b: take R3-240438 as starting point for proposal 1a.

Issue 2:

| R3-240497 | Correction on ECN marking for L4S and data forwarding (Huawei, Lenovo, CMCC, Deutsche Telekom) | draftCR |
| --- | --- | --- |

Proposal 2a: Add the description about how the gNB perform ECN marking for L4S in 38.300.

Proposal 2b: Take R3-240497 as starting point for proposal 1b.

Issue 3:

| R3-240697 | Corrections to TS 37.483 to support XR (ZTE, CATT, CMCC, China Telecom) |
| --- | --- |

Proposal 3a: reports the editorial change to E1 rapporteur. The other change can be discussed in PDU set offline discussion.

References