3GPP TSG-RAN3 Meeting #123	R3-241003

Athens, Greece, 26 Feb - 1 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0326 | rev | 1 | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IAB-node authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Huawei, Qualcomm Incorporated, ZTE, Nokia, Nokia Shanghai Bell, Xiaomi, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2024-2-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support IAB operation, the CN that the IAB-node connects provides authorization status of the IAB-node to the RAN system. In Rel-16, the IAB-node can connect to 5GC via SA based network integration procedure or connect to EPC via NSA (EN-DC mode) based network integration procedure. If the IAB-node connects to 5GC, the AMF initiates the NG procedure to provide the IAB authorization status to the IAB-donor. It’s specified in TS38.413 that if the authorization status is received for an IAB-MT from the AMF, the NG-RAN should use it accordingly, e.g., to ensure that the IAB-node will not serve any UE if the status is set to “not authorized”. 38.413: Upon receipt of the INITIAL CONTEXT SETUP REQUEST message the NG-RAN node shall … - if supported, store the received IAB Authorization information in the UE context, and use it accordingly for the IAB-MT; … Upon receipt of the UE CONTEXT MODIFICATION REQUEST message the NG-RAN node shall - if supported, store the received IAB Authorization information in the UE context. If the IAB Authorized IE is set to "not authorized" for an IAB-MT, the NG-RAN node shall, if supported, initiate actions to ensure that the IAB node will not serve any UE(s). If the IAB Authorized IE is contained in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported, store the received IAB Authorization information in the UE context. If the IAB Authorized IE is set to "not authorized" for an IAB-MT, the NG-RAN node shall, if supported, initiate actions to ensure that the IAB node will not serve any UE(s).. If the IAB-node connects to EPC, the MME provides the IAB authorization status to the eNB which the IAB-node connects. It’s specified in TS36.413 that if the authorization status is received from MME for an IAB-MT, the RAN should use it accordingly, e.g., to ensure that the IAB-node will not serve any UE if the authorization status is “not authorized”. 36.413: Upon receipt of the INITIAL CONTEXT SETUP REQUEST message, the eNB shall … - store the received IAB Authorization Information, if supported, in the UE context, and use it accordingly for the IAB-MT. Upon receipt of the UE CONTEXT MODIFICATION REQUEST message, the eNB shall … - store the received IAB Authorized IE, if supported, in the UE context. If the IAB Authorized IE is set to "not authorized" for an IAB-MT, the eNB shall, if supported, initiate actions to ensure that the IAB node will not serve any UE(s).. However, it has not been specified how the RAN uses the authorization status of IAB-node that is received from the core network. The same issue has been addressed for mobile IAB in Rel-18, where the AMF provides the authorization status of mobile IAB-node to the RRC-terminating donor and the RRC-terminating donor uses it accordingly. The handling for mobile IAB-node authorization should be applied to Rel-16 IAB node as well, but IAB-MT and IAB-DU connect to different IAB-donors is not considered for Rel-16 IAB. In addition, the handling of network integration procedure for NSA and handling the authorization status change during the IAB-node connecting to EPC should be considered for Rel16. Summary of handling authorization status for Rel-16 IAB: - Network integration for SA If the authorization status is “not authorized” for the IAB-node, the IAB-donor will not establish any backhaul resources for this IAB-node. - Authorization status received from 5GC is changed - changed to “not authorized” The IAB-donor will attempt to hand over the UEs/child nodes served by the IAB-node to other cell(s), release the F1 interface towards the IAB-DU, and release all backhaul resources for the IAB-node. - changed to “authorized” Tthe phase 2 and phase 3 of the IAB node integration procedure for SA are carried out. - Network integration for NSA The eNB includes the authorization status received from MME in the SGNB ADDITION REQUEST message to the IAB-donor in phase 1-2 of IAB-node integration as defined in 8.12.2. If “not authorized”, the IAB-donor will not establish any backhaul resource for this IAB-node. - Authorization status received from EPC is changed The eNB sends the updated authorization status to the IAB-donor via a SGNB MODIFICATION REQUEST message. - If “not authorized”, the IAB-donor will attempt to hand over the UEs/child nodes served by the IAB-node to other cell(s), release the F1 interface towards the IAB-DU, and release all backhaul resources for the IAB-node. - If “authorized”, the phase 2 and phase 3 of the IAB node integration procedure for NSA are carried out. |
| Summary of change: |  | Add the handling of RAN-node upon receiving the authorization status for Rel-16 IAB-node from core network. Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has impact on the functional point of view, will impact the RAN handling for the authorization status for an Rel-16 IAB-node from CN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CN sends authorization status for an Rel-16 IAB-node the RAN, the RAN does not know how to use the received authorization status. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.Y (new), 8.9.Y.1 (new), 8.9.Y.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.423 CR 1769 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

| This CR's revision history: | Revision version 1 updates the text based on R3-240167 (version 0). |
| --- | --- |

-------------------------------------------Start of changes-------------------------------------------

8.9.Y IAB node authorization

8.9.Y.1	IAB-node in NSA

During the IAB-node integration procedure, the IAB-MT’s eNB receives the authorization status of the IAB-node from the EPC. The eNB forwards the authorization status to the IAB-node’s IAB-donor-CU in the SGNB ADDITION REQUEST message. If the authorization status is “not authorized”, the IAB-donor-CU neither establishes any backhaul resources nor it allocates any BAP address, TNL address or default BAP configuration for this IAB-node.

When the authorization status for the IAB-node changes, the EPC sends an updated authorization status to the IAB-MT’s eNB. The eNB forwards the authorization status to the IAB-node’s IAB-donor-CU in the SGNB ADDITION REQUEST message or SGNB MODIFICATION REQUEST message.

In case the authorization status is “not authorized”, the IAB-donor-CU performs the following actions in this order: it attempts to hand over the UEs served by the IAB-node to other cell(s), releases the F1 interface towards the IAB-DU, and releases all backhaul resources (including the BAP address, TNL address and default BAP configuration) for this IAB-node. In case the authorization status is changed back from “not authorized” to “authorized”, the phase 2 and phase 3 of the IAB-node integration procedure as defined in clause 8.12 are carried out.

8.9.Y.2	IAB-node in SA

During the IAB-node integration procedure, the IAB-MT’s IAB-donor-CU receives the authorization status of the IAB-node from the 5GC. When the authorization status for the IAB-node changes, the 5GC sends an updated authorization status to the IAB-donor-CU. Upon reception of the authorization status, the IAB-donor-CU performs the SA equivalent of the steps described for NSA in 8.9.Y.1.

-------------------------------------------End of changes-------------------------------------------