3GPP TSG-RAN3 Meeting #123	R3-241011

Athens, Greece, 26 Feb - 1 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0316 | rev | 3 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IAB-node authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Huawei, Qualcomm Incorporated, ZTE, Nokia, Nokia Shanghai Bell, Xiaomi, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core, NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2024-2-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To support IAB operation, the CN that the IAB-node connects provides authorization status of the IAB-node to RAN. In Rel-17, the IAB-node can connect to 5GC via SA based network integration procedure or connect to EPC via NSA (EN-DC mode) based network integration procedure. If the IAB-node connects to 5GC, the AMF initiates the NG procedure to provide the IAB authorization status to the IAB-donor which provides the C-plane connection to 5GC for the IAB-MT. It’s specified in TS38.413 that if the authorization status is received for an IAB-MT from the AMF, the NG-RAN should use it accordingly, e.g., to ensure that the IAB-node will not serve any UE if the status is set to “not authorized”. 38.413: Upon receipt of the INITIAL CONTEXT SETUP REQUEST message the NG-RAN node shall … - if supported, store the received IAB Authorization information in the UE context, and use it accordingly for the IAB-MT; … Upon receipt of the UE CONTEXT MODIFICATION REQUEST message the NG-RAN node shall - if supported, store the received IAB Authorization information in the UE context. If the IAB Authorized IE is set to "not authorized" for an IAB-MT, the NG-RAN node shall, if supported, initiate actions to ensure that the IAB node will not serve any UE(s). If the IAB Authorized IE is contained in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported, store the received IAB Authorization information in the UE context. If the IAB Authorized IE is set to "not authorized" for an IAB-MT, the NG-RAN node shall, if supported, initiate actions to ensure that the IAB node will not serve any UE(s).. If the IAB-node connects to EPC, the MME provides the IAB authorization status to the eNB which the IAB-node connects. It’s specified in TS36.413 that if the authorization status is received from MME for an IAB-MT, the RAN should use it accordingly, e.g., to ensure that the IAB-node will not serve any UE if the authorization status is “not authorized”. 36.413: Upon receipt of the INITIAL CONTEXT SETUP REQUEST message, the eNB shall … - store the received IAB Authorization Information, if supported, in the UE context, and use it accordingly for the IAB-MT. Upon receipt of the UE CONTEXT MODIFICATION REQUEST message, the eNB shall … - store the received IAB Authorized IE, if supported, in the UE context. If the IAB Authorized IE is set to "not authorized" for an IAB-MT, the eNB shall, if supported, initiate actions to ensure that the IAB node will not serve any UE(s).. However, it has not been specified how the RAN node uses the authorization status of IAB-node that is received from the core network. The same issue has been addressed for mobile IAB in Rel-18, where the AMF provides the authorization status of mobile IAB-node to the RRC-terminating donor and the RRC-terminating donor uses it accordingly. The handling for mobile IAB-node authorization should be applied to Rel-17 IAB node as well. In addition, some scenarios should be considered for Rel-17 compared to Rel-18: the network integration procedure for NSA, authorization status received from EPC is changed, IAB-node uses NR-DC with both F1/non-F1 donors, IAB-node uses NR-DC for CP/UP separation when only SN is the donor. Summary of handling the authorization status for Rel-17 IAB: - Network integration for SA If the authorization status is “not authorized” for the IAB-node, the IAB-donor will not establish any backhaul resources for this IAB-node. - Authorization status received from 5GC is changed - changed to “not authorized”, - IAB-node with one IAB-donor in SA The F1 donor will attempt to hand over the UEs/child nodes served by the IAB-node to other cell(s), release the F1 interface towards the IAB-DU, and release all backhaul resources for the IAB-node. In case IAB-node uses NR-DC with only SN is donor, MN sends the updated authorization status to SN via a S-NODE ADDITION REQUEST message or a S-NODE MODIFICATION REQUEST message. The F1 donor will attempt to hand over the UEs/child nodes served by the IAB-node to other cell(s), release the F1 interface towards the IAB-DU, and release all backhaul resources for the IAB-node. In case the IAB-node uses NR-DC where MN is the F1 donor and SN is the non-donor-gNB, the F1-donor will attempt to hand over the UEs/child nodes served by the IAB-node to other cell(s), release the F1 interface towards the IAB-DU, and release all backhaul resources for the IAB-node. - IAB-node with two IAB-donors in SA In case the IAB-node single-connected to the non-F1 donor or uses NR-DC where MN is the non-F1 donor, the non-F1 donor will inform the F1 donor the authorization status of the IAB-node via IAB TRANSPORT MODIFICATION REQUEST message. The F1 donor hands over the UEs or the child IAB-nodes currently connect to the IAB-node’s cell(s) to other cell(s), release the F1 interface towards the IAB-DU and releases all backhaul resources for the IAB-node. The F1 donor sends an IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message to the non-F1 donor with request for all offloaded traffic release. In case the IAB-node uses NR-DC where MN is the F1-donor, the F1-donor will attempt to hand over the UEs/child nodes served by the IAB-node to other cell(s), release the F1 interface towards the IAB-DU, and release all backhaul resources for the IAB-node. - changed to “authorized” The phase 2 and phase 3 of the IAB node integration procedure for SA are carried out. - Network integration for NSA The eNB includes the authorization status received from MME in the SGNB ADDITION REQUEST message to the IAB-donor. If the status is “not authorized”, the IAB-donor will not establish any backhaul resource for this IAB-node. - Authorization status received from EPC is changed The eNB sends the updated authorization status to the IAB-donor via a SGNB MODIFICATION REQUEST message. - If “not authorized”, the IAB-donor will attempt to hand over the UEs/child nodes served by the IAB-node to other cell(s), release the F1 interface towards the IAB-DU, and release all backhaul resources for the IAB-node. - If “authorized”, the phase 2 and phase 3 of the IAB node integration procedure for NSA are carried out. |
| Summary of change: |  | Add the handling of RAN-node upon receiving the authorization status for Rel-17 IAB-node from core network. Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has impact on the functional point of view, will only impact the RAN handling for the authorization status for an Rel-17 IAB-node from CN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CN sends authorization status for an Rel-17 IAB-node the RAN, the RAN does not know how to use the received authorization status. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.Y (new), 8.9.Y.1 (new), 8.9.Y.2 (new), 8.9.Y.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 1166 TS 36.423 CR 1170 TS 38.413 CR 1060 TS 36.413 CR 1926 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

| This CR's revision history: | - Revision version 1 updates the text based on R3-237614 (version 0). - Revision version 2 updates the text based on R3-237888 (version 1). - Revision version 3 updates the text based on R3-240187 (version 0). |
| --- | --- |

-------------------------------------------Start of changes-------------------------------------------

8.9.Y IAB node authorization

8.9.Y.1	IAB-node in NSA

During the IAB-node integration procedure, the eNB receives the authorization status of the IAB-node from the EPC. The eNB forwards the authorization status to the IAB-node’s IAB-donor-CU in the SGNB ADDITION REQUEST message. If the authorization status is “not authorized”, the IAB-donor-CU neither establishes any backhaul resources nor allocate any BAP address, TNL address or default BAP configuration for this IAB-node.

When the authorization status for the IAB-node changes, the EPC sends an updated authorization status to the IAB-MT’s eNB. The eNB forwards the authorization status to the IAB-node’s IAB-donor-CU in the SGNB ADDITION REQUEST message or SGNB MODIFICATION REQUEST message.

In case the authorization status is “not authorized”, the IAB-donor-CU performs the following actions in this order: it attempts to hand over the UEs served by the IAB-node to other cell(s), releases the F1 interface towards the IAB-DU, and releases all backhaul resources (including the BAP address, TNL address and default BAP configuration) for this IAB-node. In case the authorization status is changed back from “not authorized” to “authorized”, the phase 2 and phase 3 of the IAB-node integration procedure as defined in clause 8.12 are carried out.

8.9.Y.2	IAB-node with single IAB-donor in SA

8.9.Y.2.1 IAB-node is single-connected

The IAB-donor-CU receives the authorization status of the IAB-node from the 5GC. During the IAB-node network integration, if the authorization status is “not authorized”, the IAB-donor-CU neither establishes any backhaul resources nor allocates any BAP address, TNL address or default BAP configuration for this IAB-node. When the authorization status for the IAB-node changes, the 5GC sends an updated authorization status to IAB-donor-CU. Upon reception of the authorization status, the IAB-donor-CU performs the SA equivalent of the steps described for NSA in 8.9.Y.1.

8.9.Y.2.2 IAB-node is NR dual-connected

In case the IAB-node is dual-connected to the IAB-donor-CU, the IAB-donor-CU receives the authorization status of the IAB-node from the 5GC. Upon reception of the authorization status, the IAB-donor-CU performs the SA equivalent of the steps described for NSA in 8.9.Y.1.

In case the IAB-node is dual-connected to two IAB-capable gNBs, the MN receives the authorization status of the IAB-node from the 5GC. If the MN is not the IAB-donor-CU and the SN is the IAB-donor-CU, the IAB-MT’s MN forwards the authorization status received from the 5GC to the IAB-node’s IAB-donor-CU in the S-NODE ADDITION REQUEST message or S-NODE MODIFICATION REQUEST message. Upon reception of the authorization status, the IAB-donor-CU performs the SA equivalent of steps described for NSA in 8.9.Y.1. If the MN is the IAB-donor-CU and the SN is not the IAB-donor-CU, the IAB-donor-CU performs the SA equivalent of the steps described for NSA in 8.9.Y.1.

8.9.Y.3	IAB-node is served by two IAB-donors in SA

In case the IAB-MT only connects to the non-F1-terminating IAB-donor-CU or in case the IAB-MT is NR dual-connected with the MN as the non-F1-terminating IAB-donor-CU, the non-F1-terminating IAB-donor-CU sends the authorization status received from the 5GC to the F1-terminating IAB-donor-CU in the IAB-TRANPORT MIGRATION MODIFICATION REQUEST message. Upon reception of the authorization status, the F1-terminating IAB-donor-CU performs the SA equivalent of steps described for NSA in 8.9.Y.1. After that, if the authorization status is “not authorized”, the F1-terminating IAB-donor-CU sends to the non-F1-terminating IAB-donor-CU an IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message requesting the release of all offloaded traffic, after which the non-F1-terminating IAB-donor-CU releases the offloaded traffic and all backhaul resources, BAP address, TNL address and default BAP configuration for the IAB-node.

In case the IAB-MT is NR dual-connected, where the MN is the F1-terminating IAB-donor-CU and the SN is a non-F1-terminating IAB-donor-CU, upon reception of the authorization status the IAB-node’s authorization procedure follows the same steps as described in 8.9.Y.2.

-------------------------------------------End of changes-------------------------------------------