3GPP TSG-RAN3 Meeting #123	R3-240823

- Athens, Greece, 26th Feb – 1st Mar 2024

Agenda Item:	9.3.1

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion for CB: # 8_2RxXRUE

Document for:	Discussion

# Introduction

CB: # 8_2RxXRUE

- Check the CRs in R3-240822 and R3-240139

- N2-NGAP indication for ‘2Rx non-REDCAP XR devices’ from gNB to Core Network based on progress from RAN2?

(moderator - Nok)

Summary of offline disc R3-240823

# For the Chair’s Notes

# Discussion

## SPID

Q1: Please share your view on draft Stage-2 CR in the uploaded draft R3-240822 and R3-240139

| Company | Comment |
| --- | --- |
| Nokia | TS38.300 CR: ok TS36.300 CR: we think it may be better to clarify the new SPID value is not applicable to eNB/EPC, but we are ok if majority prefer no 36.300 CR. |

Summary

Potential proposals:

## NGAP indication for ‘2Rx non-REDCAP XR devices’ from gNB to Core Network

Based on the UE’s radio capability information, gNB send this indication to CN,

Upon the reception of this indication, the AMF may choose to reject the UE’s Registration Request in case the UE’s radio capability indication indicates 2Rx, but the subscription does not include the new SPID value.

Proposal 2: introduce a new indication indicating the UE is 2Rx in UE RADIO CAPABILITY INFO INDICATION message.

Q2: Please share your view on above proposal

| Company | Comment |
| --- | --- |
| Nokia | Agree. |

Summary

Potential proposals:

# TBD