3GPP TSG-RAN WG3 #124	       R3-243110

Fukuoka, Japan, 20th – 24th May 2024                                            (Resubmit  R3-242019)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1788 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PSCell List Container List of RA report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, CATT, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current textual description of how to use PSCell List Container List refer to routing to only en-gNB is not correct. Based on meeting minute RAN2#124 in R2-2313567: UE report all available NR RACH report based on request from LTE BS. It is RAN2’s understanding the RA report retrieved by LTE BS could generated from an en-gNB or from a gNB. The current description limit the scope of target RAN node only relate to en-gNB is not correct and therefore need to be updated. In addition, the IE name PSCell List Container List is misleading since the list not only includes PSCell but also include PCell. Impact Analysis: Impact assessment towards the previous version of the specification (same release): - This CR has no impact since it only updates the textual description of Access and Mobility procedure and IE name of PSCell List Container List. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update IE name from PSCell List Container to NRCell List Container. - To replace en-gNB with NG-RAN node in the Text description of Access and Mobility Indication procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current textual description of PSCell List Container is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.16.2, 9.1.2.50 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.423 CR 1270 TS38.401 CR 0387 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Resubmit R3-242019 |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

8.3.16.2	Successful Operation

Figure 8.3.16.2-1: Access and Mobility Indication. Successful operation – eNB-initiated

Figure 8.3.16.2-2: Access and Mobility Indication. Successful operation – eNB-initiated

The eNB initiates the procedure by sending the ACCESS AND MOBILITY INDICATION message sent to the en-gNB.

The eNB1 initiates the procedure by sending the ACCESS AND MOBILITY INDICATION message to the eNB2.

If the NRCell List Container IE is included in the ACCESS AND MOBILITY INDICATION message, the eNB2 may use it to identify the NG-RAN node to which the RA Report Container IE should be forwarded.

<<<<<<<<<<<<<<<<<<<< Next of Change >>>>>>>>>>>>>>>>>>>>

9.1.2.50	ACCESS AND MOBILITY INDICATION

This message is sent by the eNB to the eNB/en-gNB to transfer access and mobility related information.

Direction: eNB  en-gNB, eNB1  eNB2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NR RA Report List |  | 0..1 |  |  | YES | ignore |
| >NR RA Report List Item |  | 1 .. <maxnoofRAReports> |  |  | EACH | ignore |
| >>NR RA Report Container | M |  | OCTET STRING | Includes the ra-ReportList contained in the UEInformationResponse messageas defined in subclause 6.2.2 in TS 38.331 [31]. | – |  |
| >>UE Assistant Identifier | O |  | en-gNB UE X2AP ID 9.2.100 |  | – |  |
| >>NRCell List Container | O |  | OCTET STRING | Includes the CellIdListNR IE as defined in subclause 6.2.2 in TS 36.331 [9]. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>