3GPP TSG-RAN WG3 Meeting #124	R3-243137

Fukuoka, Japan, 20th – 24th May 2024

Title: 	Discussion on emergency call support for (e)RedCap in barred cells

Source: 	ZTE, China Telecommunication, China Unicom

Agenda item:	8.1

Document for:	Discussion and Decision

- Introduction

RAN2 has sent an LS (R3-243012/ R2-2403997: LS on emergency call support for (e)RedCap in barred cells) to RAN3, details as below.

1. Overall Description: In RAN2#125bis RAN2 have endorsed the attached CRs for (e)RedCap UEs with the description of the CRs as below: If a (e)RedCap UE is barred in a cell where (e)RedCap is enabled since (e)RedCap UEs with 1Rx branch or 2Rx branches or both are barred in the cell, network may allow those (e)RedCap UEs to consider the cell as acceptable cell for emergency calls if cell selection criteria is fulfilled and, if the (e)RedCap UE supports only half duplex FDD operation, HD-FDD operation is allowed in the cell. The agreed RAN2 CRs also include the means for the cell to allow or not, the above operation via a SIB1 field. The CRs (R2-2402902, R2-2402903) when agreed, are considered to be implementable by Rel-17 UEs without any interoperability issue. 2. Actions: To RAN3 ACTION: RAN2 respectfully asks RAN3 to take the above information into account and update RAN3 specifications if needed.

This paper addresses to update RAN3 specifications to align with RAN2 solution on emergency call support for (e)RedCap in barred cells.

- Discussion

  - RAN2 solution

In TS38.331 CR, a new IE ”barringExemptRedCap” is added in the SIB1 message, to allow RedCap UEs to have access to the cell to make an emergency call or receive emergency information broadcast, when possible, if the cell enables access for RedCap UEs but the RedCap UEs consider this cell as barred based on the 1Rx or 2Rx support .

SIB1 message <Skip irrelated part> SIB1-v1700-IEs ::= SEQUENCE { hsdn-Cell-r17 ENUMERATED {true} OPTIONAL, -- Need R uac-BarringInfo-v1700 SEQUENCE { uac-BarringInfoSetList-v1700 UAC-BarringInfoSetList-v1700 } OPTIONAL, -- Cond MINT sdt-ConfigCommon-r17 SDT-ConfigCommonSIB-r17 OPTIONAL, -- Need R redCap-ConfigCommon-r17 RedCap-ConfigCommonSIB-r17 OPTIONAL, -- Need R featurePriorities-r17 SEQUENCE { redCapPriority-r17 FeaturePriority-r17 OPTIONAL, -- Need R slicingPriority-r17 FeaturePriority-r17 OPTIONAL, -- Need R msg3-Repetitions-Priority-r17 FeaturePriority-r17 OPTIONAL, -- Need R sdt-Priority-r17 FeaturePriority-r17 OPTIONAL -- Need R } OPTIONAL, -- Need R si-SchedulingInfo-v1700 SI-SchedulingInfo-v1700 OPTIONAL, -- Need R hyperSFN-r17 BIT STRING (SIZE (10)) OPTIONAL, -- Need R eDRX-AllowedIdle-r17 ENUMERATED {true} OPTIONAL, -- Need R eDRX-AllowedInactive-r17 ENUMERATED {true} OPTIONAL, -- Cond EDRX-RC intraFreqReselectionRedCap-r17 ENUMERATED {allowed, notAllowed} OPTIONAL, -- Need S cellBarredNTN-r17 ENUMERATED {barred, notBarred} OPTIONAL, -- Need S nonCriticalExtension SIB1-v1740-IEs OPTIONAL } SIB1-v1740-IEs ::= SEQUENCE { si-SchedulingInfo-v1740 SI-SchedulingInfo-v1740 OPTIONAL, -- Need R nonCriticalExtension SIB1-v1800-IEs OPTIONAL } SIB1-v1800-IEs ::= SEQUENCE { ncr-Support-r18 ENUMERATED {true} OPTIONAL, -- Need S mt-SDT-ConfigCommonSIB-r18 MT-SDT-ConfigCommonSIB-r18 OPTIONAL, -- Need R musim-CapRestrictionAllowed-r18 ENUMERATED {true} OPTIONAL, -- Need R featurePriorities-v1800 SEQUENCE { msg1-Repetitions-Priority-r18 FeaturePriority-r17 OPTIONAL, -- Need R eRedCapPriority-r18 FeaturePriority-r17 OPTIONAL -- Need R } OPTIONAL, -- Need R si-SchedulingInfo-v1800 SI-SchedulingInfo-v1800 OPTIONAL, -- Need R cellBarredATG-r18 ENUMERATED {barred, notBarred} OPTIONAL, -- Need S cellBarredNES-r18 ENUMERATED {notBarred} OPTIONAL, -- Need R mobileIAB-Cell-r18 ENUMERATED {true} OPTIONAL, -- Need R eDRX-AllowedInactive-r18 ENUMERATED {true} OPTIONAL, -- Cond EDRX-RC intraFreqReselection-eRedCap-r18 ENUMERATED {allowed, notAllowed} OPTIONAL, -- Need S nonServingCellMII-r18 ENUMERATED {true} OPTIONAL, -- Need R sdt-BeamFailureRecoveryProhibitTimer-r18 ENUMERATED {ms50, ms100, ms200, ms500, ms1000, ms1500, ms2000, ms3000} OPTIONAL, -- Need R eRedCap-ConfigCommon-r18 ERedCap-ConfigCommonSIB-r18 OPTIONAL, -- Need R cellBarredFixedVSAT-r18 ENUMERATED {barred, notBarred} OPTIONAL, -- Cond NTN cellBarredMobileVSAT-r18 ENUMERATED {barred, notBarred} OPTIONAL, -- Cond NTN reselectionMeasurementsNR-r18 ENUMERATED{true} OPTIONAL, -- Need R cellBarred2RxXR-r18 ENUMERATED {true} OPTIONAL, -- Need R intraFreqReselection2RxXR-r18 ENUMERATED {allowed, notAllowed} OPTIONAL, -- Need R barringExemptRedCap-r18 ENUMERATED {true} OPTIONAL, -- Cond REDCAP-Barring nonCriticalExtension SEQUENCE {} OPTIONAL } UAC-AccessCategory1-SelectionAssistanceInfo ::= ENUMERATED {a, b, c} UAC-AC1-SelectAssistInfo-r16 ::= ENUMERATED {a, b, c, notConfigured} SDT-ConfigCommonSIB-r17 ::= SEQUENCE { sdt-RSRP-Threshold-r17 RSRP-Range OPTIONAL, -- Need R sdt-LogicalChannelSR-DelayTimer-r17 ENUMERATED { sf20, sf40, sf64, sf128, sf512, sf1024, sf2560, spare1} OPTIONAL, -- Need R sdt-DataVolumeThreshold-r17 ENUMERATED {byte32, byte100, byte200, byte400, byte600, byte800, byte1000, byte2000, byte4000, byte8000, byte9000, byte10000, byte12000, byte24000, byte48000, byte96000}, t319a-r17 ENUMERATED { ms100, ms200, ms300, ms400, ms600, ms1000, ms2000, ms3000, ms4000, spare7, spare6, spare5, spare4, spare3, spare2, spare1} } RedCap-ConfigCommonSIB-r17 ::= SEQUENCE { halfDuplexRedCapAllowed-r17 ENUMERATED {true} OPTIONAL, -- Need R cellBarredRedCap-r17 SEQUENCE { cellBarredRedCap1Rx-r17 ENUMERATED {barred, notBarred}, cellBarredRedCap2Rx-r17 ENUMERATED {barred, notBarred} } OPTIONAL, -- Need R ... } ERedCap-ConfigCommonSIB-r18 ::= SEQUENCE { cellBarredeRedCap-r18 SEQUENCE { cellBarredeRedCap1Rx-r18 ENUMERATED {barred, notBarred}, cellBarredeRedCap2Rx-r18 ENUMERATED {barred, notBarred} } } <Skip irrelated part> SIB1 field descriptions. barringExemptRedCap Indicates whether the cell allows IMS emergency bearer services for RedCap UEs, if these UEs consider the cell as acceptable cell as specified in TS 38.304 [20].. cellBarred2RxXR Indicates whether the cell is barred for 2Rx XR UEs. This field is ignored by all UEs that are not 2Rx XR UEs. This field may be configured only if the cell operates in a frequency band where 4Rx antenna ports are mandated as specified in TS 38.101-1 [15]. If this field is absent on a cell operating in a frequency band where 4RX antenna ports are mandated, a 2RX XR UE shall treat the cell as not barred, as specified in TS 38.304 [20].. cellBarredATG Value barred means that the cell is barred for connectivity to ATG, as defined in TS 38.304 [20]. Value notBarred means that the cell is allowed for connectivity to ATG. If not present, the UE considers the cell is not allowed for connectivity to ATG, as defined in TS 38.304 [20]. This field is only applicable to ATG-capable UEs.. cellBarred-eRedCap1Rx Value barred means that the cell is barred for an eRedCap UE with 1 Rx branch, as defined in TS 38.304 [20]. This field is ignored by non-eRedCap UEs.. cellBarred-eRedCap2Rx Value barred means that the cell is barred for an eRedCap UE with 2 Rx branches, as defined in TS 38.304 [20]. This field is ignored by non-eRedCap UEs..

Observation 1: In RAN2 solution, a new IE “barringExemptRedCap” is added in the SIB1 message.

  - RAN3 specification enhancement

In current XnAP and F1AP specifications, the “RedCap Broadcast Information” IE has indicated intraFreqReselection-eRedCap, halfDuplexRedCapAllowed, cellBarredRedCap1Rx and cellBarredRedCap2Rx, which are defined in SIB1.

For the same reason, it is proposed to modify the“RedCap Broadcast Information” IE, to add barringExemptRedCap indication, to align with RAN2 solution.

The below change is for example.

==============Start of Change ============================================

Served Cell Information

This IE contains cell configuration information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| ---------------------------------------------Skip unchanged part----------------------------------- |  |  |  |  |  |  |
| RedCap Broadcast Information | O |  | BIT STRING (SIZE(8)) | The presence of this IE indicates that the intraFreqReselectionRedCap IE is broadcast in SIB1 of the corresponding cell, see TS 38.331 [8]. Each position in the bitmap indicates which RedCap UEs are allowed access, according to the setting of RedCap barring indicators in SIB1, see TS 38.331 [8]. First bit = 1Rx, second bit = 2Rx, third bit = halfDuplex, fourth bit = barringExempt, other bits reserved for future use. Value '1' indicates 'access allowed'. Value '0' indicates 'access not allowed”. | YES | ignore |

==============End of Change ============================================

Observation 2: In both TS38.423 and TS38.473, the four IEs within SIB1 (i.e., intraFreqReselectionRedCap, cellBarred-eRedCap1Rx, cellBarred-eRedCap2Rx and halfDuplexRedCapAllowed) have been captured to align with RAN2.

Proposal 1: In both TS38.423 and TS38.473, the IE “RedCap Broadcast Information” shall be updated to include barringExemptRedCap indication.

  - Other issues

In the incoming LS (R3-243012/ R2-2403997), its attached R17CRs (i.e., R2-2402902, R2-2402903) are not correct, it shall be updated with R17 RAN2 specifications.

Proposal 2: Send LS to remind RAN2 to update their R17 CRs.

- Conclusion

After the above analysis, we provide the following proposals.

Observation 1: In RAN2 solution, a new IE “barringExemptRedCap” is added in the SIB1 message.

Observation 2: In both TS38.423 and TS38.473, the four IEs within SIB1 (i.e., intraFreqReselectionRedCap, cellBarred-eRedCap1Rx, cellBarred-eRedCap2Rx and halfDuplexRedCapAllowed) have been captured to align with RAN2.

Proposal 1: In both TS38.423 and TS38.473, the IE “RedCap Broadcast Information” shall be updated to include barringExemptRedCap indication.

Proposal 2: Send LS to remind RAN2 to update its R17 CRs.

Proposal 3: Agree with R17/R18 TS38.423 CRs (R3-243138/ R3-243139) and R17/R18 TS38.473 CRs (R3-243154/ R3-243159) to align with RAN2 solution.

Proposal 4: Send LS (R3-243140) to RAN2, to remind RAN2 to update their R17 CRs.

- References

- R3-243012 LS on emergency call support for (e)RedCap in barred cells LS in

- R3-243138 Introduction of barring exemption for RedCap UEs for emergency calls, Rel-17, TS 38.423

- R3-243139 Introduction of barring exemption for RedCap UEs for emergency calls, Rel-18, TS 38.423

- R3-243154 Introduction of barring exemption for RedCap UEs for emergency calls, Rel-17, TS 38.473

- R3-243159 Introduction of barring exemption for RedCap UEs for emergency calls, Rel-18, TS 38.473

- R3-243140 [Draft] Reply LS on emergency call support for (e)RedCap in barred cells