3GPP TSG-RAN WG3 Meeting #124	R3-243229

Fukuoka, Japan, 20 – 24 May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1418 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on MBS UP failure_option 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CBN, Nokia, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core, TEI18 |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-18 WI, the F1AP: Broadcast Transport Resource Request procedure was introduced to enable the gNB-DU to invoke the gNB-CU to trigger the Broadcast Context Modification procedure to establish the F1-U resources for the broadcast session. This procedure was introduced for the scenario of multiple cell ID broadcast network sharing scenario, as the F1-U tunnel is not established with some gNB-CUs during Broadcast Context Setup procedure. In previous RAN3 meeting, the UP-failure scenario was discussed, in such case, the gNB-DU may need to trigger the F1AP: Broadcast Transport Resource Request procedure towards the gNB-CU with which the shared F1-U tunnel has already been established. But in such case the gNB-CU handling is unclear, e.g., whether regard this as abnormal scenario, or release the already established F1-U tunnel and setup new one. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that in case the shared F1-U tunnel has already been established, upon receiving the BROADCAST TRANSPORT RESOURCE REQUEST message, the gNB-CU shall, if supported, consider that the previous established tunnel is released, and triggers the Broadcast Context Modification procedure to establish the F1-U resources. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it only clarifies the gNB-CU-CP handling upon receiving the BROADCAST TRANSPORT RESOURCE REQUEST message in case the shared F1-U tunnel has already been established. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The gNB-CU may regard the reception of BROADCAST TRANSPORT RESOURCE REQUEST message as abnormal case in case the shared F1-U tunnel(s) has already been established. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.14.14.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------Start of the Change-------------

### 8.14.14	Broadcast Transport Resource Request

#### 8.14.14.1	General

The purpose of the Broadcast Transport Resource Request procedure is to request the gNB-CU to trigger the establishment of F1-U resources for the broadcast session.

The procedure uses MBS-associated signalling.

#### 8.14.14.2	Successful Operation

Figure 8.14.14.2-1: Broadcast Transport Resource Request procedure. Successful operation

The gNB-DU initiates the procedure by sending the BROADCAST TRANSPORT RESOURCE REQUEST message to the gNB-CU.

Interaction with the Broadcast Context Modification procedure:

Upon reception of the BROADCAST TRANSPORT RESOURCE REQUEST message, the gNB-CU should trigger the Broadcast Context Modification procedure to establish the F1-U resources for the broadcast session. And in case the shared F1-U tunnel(s) has already been established, the gNB-CU shall, if supported, consider that the previous established tunnel(s) has been implicitly released in the gNB-DU and behave as specified in TS 38.401

#### 8.14.14.3	Unsuccessful Operation

Not applicable.

#### 8.14.14.4	Abnormal Conditions

Not applicable.

-------------End of the Change-------------