3GPP TSG-RAN WG3 Meeting #124	R3-243367

Fukuoka, Japan, May 20th – 24th 2024

Agenda item:	8.1

Source: 	Ericsson

Title: 	MBS Communication Service Type for QMC

Document for:	Agreement

# Introduction

In the incoming LS R3-243028 “LS on MBS Communication Service Type”, SA5 informed RAN3 of the following:

SA5 has discussed and agreed to add MBS Communication Service Type IE into the QMCJob, which indicates whether doing QoE measurement shall be performed for MBS broadcast or multicast. This IE is used for both signaling and management based QoE measurement.

For more details, please see the agreed CR.

In this contribution, we discuss further implications of the LS.

# Discussion

In TS 38.413, the MBS Communication Service Type IE was introduced as a part of the UE Application Layer Measurement Configuration Information IE.

| MBS Communication Service Type | O |  | ENUMERATED (broadcast, multicast, ...) | This IE indicates the type of MBS communication service for which the QoE measurement collection should be performed. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

According to the LS from SA5, the addition of the MBS Communication Service Type IE in the QMCJob indicates that this is relevant when QoE measurement shall be performed for MBS broadcast or multicast. This IE is sent to the gNB either via the CN and NGAP for the case of s-based QoE measurements, or directly from the OAM to the gNB for the case of m-based QoE measurements.

Observation 1: The incoming LS from SA5 clarifies that MBS Communication Service Type is used to indicate that QoE measurements shall be performed for MBS broadcast or multicast.

It is therefore important for the UE the collects the QoE measurements, to understand whether a certain QoE measurement pertains to MBS broadcast or multicast. However, as of today, the gNB does not transfer this information to the UE, which is left unaware of this requirement from the OAM. We propose that this behaviour is corrected, and that the gNB sends an indication to the UE as part of the QoE configuration, to let the UE know that QoE measurements are requested for MBS broadcast or multicast.

Proposal 1: The gNB can indicate to the UE whether QoE measurements shall be collected for MBS broadcast or multicast.

In relation to the requirement to collect QoE measurements for MBS broadcast or multicast, another aspect that can be noted is that, during a session delivered via multicast, a UE can move to an area where multicast is not supported, and the UE is reconfigured to use unicast. The session delivery continues but unicast will be used instead. If the original requirement from the OAM was to collect QoE measurements for MBS multicast, the QoE measurements that the UE will continue to perform while the session is delivered in unicast are not really needed. In addition, there is no indication in the reporting from the UE, about whether the QoE measurements were collected in multicast or in unicast. Therefore, the consumer of the QoE measurement cannot distinguish between the two sets of measurements, which defeats the whole purpose of requesting QoE measurements collection for multicast only.

Observation 2: The UE does not report whether QoE measurements were collected for MBS multicast, implying that a consumer cannot distinguish between QoE measurements collected for unicast or MBS multicast or broadcast in case of UE mobility from a gNB that supports MBS to a gNB that does not support MBS.

Based on the above, we conclude that some corrections/discussions are needed in relation to how QoE measurements are reported, which can impact RAN2 and CT1. To address this issue, we propose to liaise at least RAN2 and CT1 and we provide a draft LS in R3-243368. CT4 and SA5 should also remain in the loop.

Proposal 2: Agree and LS to RAN2, CT1 and CT4 in R3-243368.

# Conclusion

In this paper, we provide discussion based on the incoming LS R3-243028 from SA5. The following is observed and proposed:

Observation 1: The incoming LS from SA5 clarifies that MBS Communication Service Type is used to indicate that QoE measurements shall be performed for MBS broadcast or multicast.

Proposal 1: The gNB can indicate to the UE whether QoE measurements shall be collected for MBS broadcast or multicast.

Observation 2: The UE does not report whether QoE measurements were collected for MBS multicast, implying that a consumer cannot distinguish between QoE measurements collected for unicast or MBS multicast or broadcast in case of UE mobility from a gNB that supports MBS to a gNB that does not support MBS.

Proposal 2: Agree and LS to RAN2, CT1 and CT4 in R3-243368.