3GPP TSG-RAN WG3 #124	R3-243384

Fukuoka, Japan, 20th – 24th May 2024

Agenda Item:	11.2

Source:	Lenovo

Title: 		TP to TR38.743 for AIML based network slicing

Document for:	Discussion and Approval

# TP to TR38.743

***************************** CHANGE STARTS *******************************

## 4.1	AI/ML based Network Slicing

### 4.1.1	Use case description

Editor Note: Capture the description of use case

Support of network slicing in NG-RAN is defined in TS38.300 [x].

The NG-RAN plays a key role in taking mobility, load balancing and Radio Resources Management decisions for the purpose of meeting target requirements derived from the SLA of each supported network slice.

AI/ML function can analyze metrics related to network and UE level performance related to perform optimal resource management and mobility decisions for network slicing to meet the requirements.

### 4.1.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based network slicing:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

Note: gNB is also allowed to continue model training based on AI/ML model trained in the OAM

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.1.2.2	AI/ML Model Training at OAM and AI/ML Model Inference at NG-RAN

In this solution, NG-RAN manages network slices using AI/ML model trained from OAM.

Figure 4.1.2.1-1. Model Training at OAM, Model Inference at NG-RAN

Step 0: NG-RAN node 2 is assumed to have an AI/ML model optionally, which can provide NG-RAN node 1 with input information.

Step 1: NG-RAN node 1 configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report message(s) to NG-RAN node 1.

Step 4: NG-RAN node 1 further sends UE measurement reports together with other input data for Model Training to OAM.

Step 5: NG-RAN node 2 (assumed to have an AI/ML model optionally) also sends input data for Model Training to OAM.

Step 6: Model Training at OAM. Required measurements and input data from other NG-RAN nodes are leveraged to train AI/ML models for network slicing management.

Step 7: OAM deploys/updates AI/ML model into the NG-RAN node(s). The NG-RAN node can also continue model training based on the received AI/ML model from OAM.

Note: This step is out of RAN3 Rel-19 scope.

Step 8: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based network slicing management.

Step 9: UE sends the UE measurement report(s) to NG-RAN node 1.

Step 10: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2, NG-RAN node 1 generates model inference output(s) (e.g., predicted resource status per network slice, network slicing resource reallocation strategy etc).

Step 11: NG-RAN node 1 sends Model Performance Feedback to OAM if applicable.

Note: This step is out of RAN3 scope.

Step 12: NG-RAN node 1 executes Network Slicing actions according to the model inference output. NG-RAN node 1 may reallocate the resources for network slices, if the output is the network slicing resource reallocation strategy.

Step 13: NG-RAN node 2 provides feedback to OAM.

Step 14: NG-RAN node 1 provides feedback to OAM.

#### 4.1.2.3	AI/ML Model Training and AI/ML Model Inference at NG-RAN

In this solution, NG-RAN is responsible for model training and manages network slices using the trained AI/ML model.

Figure 4.1.2.2-1. Model Training and Model Inference at NG-RAN

Step 0: NG-RAN node 2 is assumed to have an AI/ML model optionally, which can provide NG-RAN node 1 with input information.

Step 1: NG-RAN node 1 configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report(s) to NG-RAN node 1 including the required measurement result.

Step 4: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model training of AI/ML-based network slicing.

Step 5: NG-RAN node 1 trains AI/ML model for AI/ML-based network slicing based on collected data. NG-RAN node 2 is assumed to have AI/ML model for AI/ML-based network slicing optionally, which can also generate predicted results/actions.

Step 6: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based network slicing.

Step 7: UE sends the UE measurement report(s) to NG-RAN node 1.

Step 8: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2, NG-RAN node 1 generates model inference output (e.g., predicted resource status per network slice, network slicing resource reallocation strategy etc).

Step 9: NG-RAN node 1 executes network slicing actions according to the model inference output. NG-RAN node 1 may select the most appropriate target cell for each UE before it performs handover, if the output is handover strategy.

Step 10: NG-RAN node 2 provides feedback to NG-RAN node 1.

#### 4.1.2.4	Input data of AI/ML based Network Slicing:

To predict the optimized network slicing decisions, a gNB may need the following information as input data for AI/ML-based network slicing:

From local node:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

-	Legacy measured/predicted UE trajectory

Measured/Predicted UE Traffic (e.g., data volume)

Measured/Predicted fulfilment of required QoS (according to SLA)

From neighbouring gNBs:

-	Measured/Predicted radio resource status per slice

-	Measured/Predicted slice available capacity

- Measured/Predicted UE Traffic (e.g., data volume) (during handover procedure)

- Measured/Predicted UE Trajectory (during handover procedure)

From the UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

#### 4.1.2.5	Output data of AI/ML based Network Slicing:

AI/ML-based network slicing model in a gNB can generate following information as output:

- Predicted radio resource status per slice

- Predicted slice available capacity

- Resource management decisions for resources within RRM policies (used by gNB internally)

- Slice aware mobility decisions (used by gNB internally)

- Predicted fulfilment of required QoS (according to SLA)

#### 4.1.2.6	Feedback of AI/ML based Network Slicing:

To optimize the performance of AI/ML-based network slicing model, following feedback can be considered to be collected from gNBs:

- Measured Radio resource status per slice

- Measured Slice available capacity

- Legacy UE performance feedback for those UEs handed over from the source gNB

- Fulfilment of required QoS according to SLA (of current NG-RAN node)

#### 4.1.2.7	Potential standard impacts:

Following standard impacts is listed for subsequent Rel-19 normative work compared with what was specified during Rel-18.

Xn interface:

- Enhance existing procedure to collect predicted information between gNBs:

  - Predicted radio resource status per slice

Predicted slice available capacity

- RAN3 may enhance the Handover Preparation procedure for the source NG-RAN node to provide the predicted UE Traffic (e.g., data volume) to the target NG-RAN node.

Potential E1 interface impact:

- RAN3 may introduce new procedure to support the exchange of the following between CU-CP and CU-UP:

  - Current/Predicted UE Traffic (e.g., data volume) from CU-UP to CU-CP

  - UE performance related measurements from CU-UP to CU-CP

Potential F1 interface impact:

- RAN3 may introduce new procedure to support the exchange of the following between CU and DU:

  - UE performance related measurements from DU to CU

  - Predicted Slice Available Capacity from CU to DU

  - Predicted Slice Radio Resource Status from CU to DU

  - Predicted fulfilment of required QoS (according to SLA) from CU to DU

***************************** CHANGE ENDS *******************************