3GPP TSG-RAN WG3 #124	R3-243386

Fukuoka, Japan, 20th – 24th May 2024

Agenda Item:	11.3

Source:	Lenovo

Title: 		TP to 38.743 for AIML based CCO

Document for:	Discussion and Approval

# Introduction

In this paper, we discuss use case related to AIML based network coverage and capacity optimization

# TP to TR38.743

***************************** CHANGE STARTS *******************************

## 4.2	AI/ML based Coverage and Capacity Optimization

### 4.2.1	Use case description

Editor Note: Capture the description of use case

The objective of NR Coverage and Capacity Optimization (CCO) function is to detect and resolve or mitigate CCO issues. An NG-RAN node may autonomously adjust within and switch among coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes with the list of cells and SSBs with modified coverage included.

In the legacy CCO solution, a reactive approach is used: when the gNB (gNB-CU in case of CU-DU split architecture) detects a CCO issue which negatively impacts network and UE performance after it has already occurred, the gNB (gNB-DU in case of CU-DU split architecture) attempts to resolve or mitigate it.

With an AI/ML based CCO, a more proactive approach is used to prevent (or limiting at an early stage) the rise of a CCO issue with the consequent degradation of network (and UE) performance.

### 4.2.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.2.2.1 Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting AI/ML-based CCO:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

Note: gNB is also allowed to continue model training based on AI/ML model trained in the OAM

In case of CU-DU split architecture, the following solutions are possible:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.

#### 4.2.2.2	AI/ML Model Training at OAM and AI/ML Model Inference at NG-RAN

In this solution, NG-RAN performs coverage and capacity optimization using AI/ML model trained from OAM.

Figure 4.2.2.1-1. Model Training at OAM, Model Inference at NG-RAN

Step 0: NG-RAN node 2 is assumed to have an AI/ML model optionally, which can provide NG-RAN node 1 with input information.

Step 1: NG-RAN node 1 configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report message(s) to NG-RAN node 1.

Step 4: NG-RAN node 1 further sends UE measurement reports together with other input data for Model Training to OAM.

Step 5: NG-RAN node 2 (assumed to have an AI/ML model optionally) also sends input data for Model Training to OAM.

Step 6: Model Training at OAM. Required measurements and input data from other NG-RAN nodes are leveraged to train AI/ML models for coverage and capacity optimization.

Step 7: OAM deploys/updates AI/ML model into the NG-RAN node(s). The NG-RAN node can also continue model training based on the received AI/ML model from OAM.

Note: This step is out of RAN3 Rel-19 scope.

Step 8: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based coverage and capacity optimization.

Step 9: UE sends the UE measurement report(s) to NG-RAN node 1.

Step 10: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2, NG-RAN node 1 generates model inference output(s) (e.g., predicted CCO issue, predicted coverage modification etc).

Step 11: NG-RAN node 1 sends Model Performance Feedback to OAM if applicable.

Note: This step is out of RAN3 scope.

Step 12: NG-RAN node 1 executes coverage and capacity optimization actions according to the model inference output. NG-RAN node 1 may extend the cell coverage, if the output is a predicted coverage issue in the near future.

Step 13: NG-RAN node 2 provides feedback to OAM.

Step 14: NG-RAN node 1 provides feedback to OAM.

#### 4.2.2.3	AI/ML Model Training and AI/ML Model Inference at NG-RAN

In this solution, NG-RAN is responsible for model training and coverage and capacity optimization using the trained AI/ML model.

Figure 4.2.2.2-1. Model Training and Model Inference at NG-RAN

Step 0: NG-RAN node 2 is assumed to have an AI/ML model optionally, which can provide NG-RAN node 1 with input information.

Step 1: NG-RAN node 1 configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report(s) to NG-RAN node 1 including the required measurement result.

Step 4: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model training of AI/ML-based network coverage and capacity optimization.

Step 5: NG-RAN node 1 trains AI/ML model for AI/ML-based network coverage and capacity optimization based on collected data. NG-RAN node 2 is assumed to have AI/ML model for AI/ML-based network coverage and capacity optimization optionally, which can also generate predicted results/actions.

Step 6: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based network coverage and capacity optimization.

Step 7: UE sends the UE measurement report(s) to NG-RAN node 1.

Step 8: Based on local inputs of NG-RAN node 1 and received inputs from NG-RAN node 2, NG-RAN node 1 generates model inference output (e.g., predicted CCO issue, predicted coverage modification etc).

Step 9: NG-RAN node 1 executes network coverage and capacity optimization actions according to the model inference output. NG-RAN node 1 may extend the cell coverage, if the output is a predicted coverage issue in the near future.

Step 10: NG-RAN node 2 provides feedback to NG-RAN node 1.

#### 4.2.2.4	Input of AI/ML-based Coverage and Capacity Optimization

NG-RAN may need following information as input data for AI/ML-based Coverage and Capacity Optimization:

From local node:

-	Measured/Predicted Radio Resource Status

-	Measured/Predicted UE Traffic (e.g., data volume)

-	Measured/Predicted UE trajectory

From UE:

-	UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements

-	UE location information (e.g., coordinates, serving cell ID, moving velocity)

-	UE Mobility History Information

-	CEF/RA/RLF report from the UE

From neighbouring NG-RAN nodes:

-	Measured/Predicted Radio Resource Status

-	Measured/Predicted UE Traffic (e.g., data volume)

-	Measured/Predicted UE trajectory

#### 4.2.2.5	Output of AI/ML-based Coverage and Capacity Optimization

AI/ML-based Coverage and Capacity Optimization model can generate following information as output:

-	type of the predicted CCO issue(s), e.g., coverage issue or cell edge capacity issue

-	id of affected cell(s) in which there is the predicted CCO issue(s)

-	SSB index(s) of affected beam(s) in which there is the predicted CCO issue(s)

Note: based on the predicted CCO issue(s), gNB may also generate the corresponding future Cell/SSB Coverage Modification(s) to resolve the predicted CCO issue(s). The future Cell/SSB Coverage Modification(s) can be determined by conventional non-AI/ML based method or AI/ML based method upon gNB implementation.

#### 4.2.2.6	Feedback of AI/ML-based Coverage and Capacity Optimization

To optimize the performance of AI/ML-based Coverage and Capacity Optimization model, following feedback can be considered to be collected:

-	UE performance (e.g., throughput, delay, packet loss rate after Cell Coverage Modification or resource reallocation or after handover)

-	Radio Resource Status (of serving and neighbouring NG-RAN node)

#### 4.2.2.7	Standard Impact

Potential Xn interface impact:

- RAN3 may enhance the Rel18 Data Collection Initiation/Report procedure to support the exchange of the following between neighbouring NG-RAN nodes:

  - Predicted Coverage and Capacity Optimization issue

  - Future Coverage Modification

Potential E1 interface impact:

- RAN3 may introduce new procedure to support the exchange of the following between CU-CP and CU-UP:

  - Measured/Predicted UE Traffic (e.g., data volume) from CU-UP to CU-CP

Potential F1 interface impact:

- RAN3 may introduce new procedure to support the exchange of the following between CU and DU:

  - Predicted Coverage and Capacity Optimization Issue from CU to DU

  - Future Coverage Modification from DU to CU

***************************** CHANGE ENDS *******************************