3GPP TSG-RAN WG3 #124	R3-243418

Fukuoka, Japan, 20-24 May 2024

Agenda Item:	11.4

Source:	Ericsson

Title:	Discussion on solutions for multi-hops UE Trajectory

Document for:	Discussion, Agreement

# Introduction

Multi-hop UE Trajectory Prediction or “Multi-hop UE trajectory across gNBs” as defined in the Study Item description [1], is one of the Release 18 leftovers which are candidate for normative work in Release 19, as explained in the SID objectives for Rel19 below.

- Rel-18 leftovers as candidates for normative work, based on the Rel-18 principles, as follows: - Mobility optimization for NR-DC - Split architecture support for Rel-18 use cases based on the conclusions from Rel-18 WI - Energy Saving enhancements, e.g., Energy Cost Prediction - Continuous MDT collection targeting the same UE across RRC states - Multi-hop UE trajectory across gNBs Note: RAN3 should take the Rel-18 discussions into account.

As for all the other leftovers, it is important at this stage of the discussions to stress out that normative work shall build up on the principles agreed in Release 18 and also that discussions shall take into account progress and agreements made during Release 18. As such, for multi-hop UE trajectory prediction the principles agreed in Release 18 for single-hop UE trajectory prediction shall be taken as a starting point for the normative work. The most relevant principles to consider are the following:

- A UE trajectory is calculated as a sequence of cells where the UE is served while in RRC_Connected

- Measured UE Trajectories are explicitly requested by a gNB to its neighbour nodes and reported back to the requesting gNB

- Predicted UE Trajectories are not subscription based and are spontaneously signalled by a gNB to its neighbour NG-RAN node as part of the handover signalling

# Discussion

The overall solution for multi-hop UE trajectory prediction needs to address all of the three following aspects of the problem:

- Sending the multi-hop prediction from the node inferring it to the next hop node,

- If a requesting node requests for a multi-hop measured trajectory, how such request is propagated to the nodes in the multi-hop chain,

- If a requesting node requests for a multi-hop measured trajectory, how the reporting nodes send the information back to the requesting node.

In the subsequent sections of this paper, we discuss a non-exhaustive list of options for each one of the aspects above.

## Sending the prediction

To send a multi-hop prediction to a neighbour gNB the source node has two options:

- Hop-by-hop: the source node, e.g., gNB0, sends the prediction to the first next hop node, which we call gNB1 for simplicity. gNB1 may choose to either forward the prediction to the second hop node, e.g., gNB2, if the UE is handed over there, or to generate a new prediction at the time the UE hands over to the next hop node (independently of whether the next hop is gNB2 or a different gNB).

- Point-to-multi-point: the source node sends the prediction to every node which is part of the multi-hop prediction.

When comparing the two options above, one could easily observe that in the first option, gNB1 will use the prediction from gNB0 to better understand future UE movements and e.g. better plan for upcoming mobility actions. If the UE is handed over from gNB1 to gNB2, and gNB2 is the second hop node in the prediction generated by gNB0, gNB1 may chose to forward the prediction inferred by gNB0 to gNB2. gNB1 may also choose to generate a new prediction, if deemed necessary. If the UE is not handed over from gNB1 to gNB2 (where gNB2 is the second hop node in the prediction from gNB0), but it is handed over to gNB3, then there is no reason to forward to gNB3 the prediction from gNB0, as this prediction is obviously inaccurate.
In this case gNB1 may decide to generate a new prediction and signal that to gNB3.

Therefore, the first option leaves it up to the serving node to decide whether to forward a previously inferred prediction or whether to produce a new one. We note that this approach is optimal as it adapts to all conditions in which the predictions are e.g., inaccurate and not usable. This approach also reuses the Handover signalling for prediction forwarding, which is a principle agreed in Rel18.

The second option, based on point-to-multi-point forwarding, is subject to a number of drawbacks. Firstly, a prediction may be forwarded to gNBs that may not serve the UE, e.g., in case the prediction is inaccurate. This creates the problem that a node may plan and reserve resources for serving a UE in a number of cells, but the UE may never be handed over to that node. 
A second drawback is that this solution requires a dedicated message to carry the prediction from the source node (gNB0) to every next hop node. In some cases where an Xn connection is not present between the source node and one of the next hop nodes, such forwarding is not even possible at all. This second option therefore departs from the Rel18 principles, and it implies a big impact (introduction of a new procedure) which in some cases does not lead to any benefit.

Proposal 1: RAN3 to respect the Release 18 agreements for UE trajectory prediction forwarding via the handover signalling procedures as baseline for multi-hop UE trajectory predictions and agree to use the HANDOVER REQUEST message to send the prediction from one hop node to the next hop node.

## Requesting and reporting measured multi-hop trajectories

A measured multi-hop UE trajectory may be requested by a source node, gNB0, to a target node. Similar to the case of multi hop trajectory prediction, two possible approaches are possible:

- Trigger a request for a multi-hop measured trajectory reporting in a hop by hop way to the next hop node serving the UE.

- Trigger a request for a multi-hop measured trajectory reporting in a point to multi-point way.

In the first option the source node, gNB0, triggers a DATA CONNECTION REQUEST towards nodes that are assumed to be the first hop target for the UE. Once the UE hands over to one of such nodes, e.g., gNB1, this node triggers a DATA COLLECITON REQUEST to following nodes that are assumed to be the second hop target for the UE, and so on. Therefore, each node serving the UE, and receiving a DATA COLLECITON REQUEST asking to report the measured UE trajectory, will report to the requesting node the trajectory within its set of served cells for the UE. This process allows each node in the chain to receive from the following serving node part of the UE trajectory that can be used to build a multi-hop UE trajectory. A full measured multi-hop UE trajectory will eventually be forwarded to the source node, gNB0.

The second option relies on reusing the DATA COLLECITON REQUEST message to task several neighbour NG-RAN nodes to report a measured UE trajectory for the UE, as per the Rel18 mechanisms. In this approach, gNB0 is able to receive, along the time, measured UE trajectories for the same UE, which come to gNB0 in chronological order from the NG-RAN nodes that served the UE. This enables gNB0 to collate an overall multi-hop measured trajectory for the UE.

We note that the first approach is more reliable because it ensures that a measured UE trajectory can be requested from a source node to a target node that will serve the UE and that is connected over the Xn to the source node. Namely, in the first approach the serving NG-RAN node can generate the measured trajectory request towards a target node that will serve the UE and that is Xn connected. 
the latter cannot be guaranteed with the second approach because the source node gNB0 may not be Xn connected to all the NG-RAN nodes in the multi-hop route of the UE. Besides, the source gNB0 may not be able to accurately select target NG-RAN nodes that will serve the UE far away in the future, hence creating the problem of missing parts of the measured trajectory at gNB0.

Moreover, for the first approach, signalling mechanisms are straightforward as the response message carrying the measured UE trajectory will travel back to the source node by rewiring the chain of DATA COLLECTION REPORTING sessions established between the nodes.

Proposal 2. RAN3 to agree that for multi-hop measured trajectory reporting, the solution is based on reporting of the UE trajectory within a serving node coverage on a hop by hop basis, namely based on DATA COLLECTION REQUEST message from source RAN node to next target RAN node and based on a DATA COLLECTION UPDATE carrying the partial UE trajectory all the way to the NG-RAN node that originated the request.

Although the hop-by-hop solution for requesting and reporting a multi-hop measured UE trajectory has very little or no standard impact as it can reuse the signaling standardized in Release 18 for single-hop measured UE trajectory reporting, the solution has not negligible implementation impact.

Indeed, each gNB that is part of a requested multi-hop measured UE trajectory, has to store the UE context until it receives all the downstream partial segments of the measured UE trajectory, so that it can build and report the downstream part of the measured UE trajectory to the preceding gNB in the trajectory.

Further complexity concerns reconstructing or relaying the partial trajectory segments. For the downstream trajectory segments a gNB receives as part of the multi-hop measured UE trajectory reporting, a gNB implementation may decide either to rebuild the full downstream UE trajectory and then send it to the preceding node or simply rely all the downstream trajectory segments it receives upstream towards the gNB at the origin of the measured UE trajectory request.

From the considerations above one could easily conclude that the challenges for supporting the multi-hop measured UE trajectory reporting are more implementation dependent than standard dependent. Hence, non e of the solutions identified here are free from considerable impact. One solution has a smaller standard impact but a considerable implementation impact, while the other solution has considerable standard impact while it is subject to errors and it does not cover all cases. 
For this reason, we propose to consider the multi-hop UE Trajectory prediction with a lower priority for the normative work in Release 19.

Proposal 3: RAN3 to consider the multi-hop UE trajectory prediction with a lower priority for the Release 19 normative work.

# Conclusions and Proposals

In this paper left over enhancements for multi-hop UE trajectories have been discussed

Our proposals are summarized below:

Proposal 1: RAN3 to respect the Release 18 agreements for UE trajectory prediction forwarding via the handover signalling procedures as baseline for multi-hop UE trajectory predictions and agree to use the HANDOVER REQUEST message to send the prediction from one hop node to the next hop node.

Proposal 2. RAN3 to agree that for multi-hop measured trajectory reporting, the solution is based on reporting of the UE trajectory within a serving node coverage on a hop by hop basis, namely based on DATA COLLECTION REQUEST message from source RAN node to next target RAN node and based on a DATA COLLECTION UPDATE carrying the partial UE trajectory all the way to the NG-RAN node that originated the request.

Proposal 3: RAN3 to consider the multi-hop UE trajectory prediction with a lower priority for the Release 19 normative work.

A TP reflecting the proposals above is available in R3-243417.