3GPP TSG-RAN WG3 Meeting #124	R3-243425

Fukuoka, Japan, 20th – 24th May, 2024

Agenda Item:	11.4

Source:	CATT

Title:	Discussion on anonymization requirement for M-based MDT

Document for:	Decision

- Introduction

In the previous meeting, we agreed continuous MDT will be discussed in REL-19. In REL-18, we note the below understanding as an end mark in RAN3 [1].

The following understanding is based on R18 use cases:

Signalling-based MDT could provide continuous Data Collection from a certain UE, however, it has scalability issues when selecting large number of UEs and lacks cell-level granularity.

Management-based MDT provides the advantage of being scalable when selecting large number of UEs. However, current management-based MDT mechanism cannot identify and group together MDT reports collected from the same UE across RRC states. UE selection is left to RAN implementation and it is not controllable by OAM.

For the s-based MDT, we note it faces scalability issues; and for the m-based MDT, the current mechanism does not offer the possibility to merge Immediate and Logged MDT data collected by the same UE across different RRC states.

Recently, SA5 do further work about UE level measurements, which may enable merge Immediate MDT and logged MDT for M-based MDT. However, we think it still not clearly about the relationship between the UE level measurement and MDT.

In this paper we raise the unclear point in SA5 which concerns with the immediate MDT and logged MDT correlation, and suggest sending LS to SA5 for further clarification.

- Discussion

During REL-18 discussion, the main challenge blocking immediate MDT and logged MDT correlation is the Anonymization of MDT Data.

As defined in TS 32.422, all job type defined in REL-17 should obey the anonymization requirements if MDT configuration is contained.NG-RAN node will not send the UE identifier to TCE by cell traffic trace.

## 4.7 Anonymization of MDT data for management based MDT If the Job Type is either Immediate MDT or Logged MDT or Logged MBSFN MDT or combined Immediate MDT and trace, the anonymization requirements are applicable as described in this clause.

Observation 1: In REL-17 defined job type, anonymization requirements are applicable for all MDT measurements.

In REL-18, SA5 has introduced the UE level measurement and informed RAN3.There are two unclear points from our point.

- Anonymization requirements for Rel-17 defined job type

In R3-240058[2], SA5 sent an LS to RAN3 stating the following:

All of the NG-RAN UE level measurements defined in draft TS 28.558 have been already supported by the MDT for NR (TS 37.320). However, RAN specs (e.g., 37.320, 38.314) refer to TS 28.552 in the definitions of certain MDT measurements, SA5 8 like to inform RAN2 and RAN3 that a proper reference for those measurements would be TS 28.558. SA5 kindly requests RAN2 and RAN3 to take this into account and consider future update of the references in relevant RAN TSs once the TS 28.558 is published.

In this LS, SA5 suggest RAN2 and RAN3 to update the reference of MDT from TS 28.552 to TS 28.558, which is a new specification for UE level measurements. In TS 28.558, SA5 defines each UE level measurement including the S-TMSI to identify UE.

Thus, follow the definition in TS 28.558, the S-TMSI information will always include in the MDT report, even the Job Type is Immediate MDT or Logged MDT or combined Immediate MDT and trace, which is against with Anonymization requirements.

Observation 2: If follow the definition in the TS 28.558, the S-TMSI information will always include in the MDT report, which is against with Anonymization requirements.

Proposal 1: RAN3 would like to ask SA5 how to meet Anonymization requirements if MDT measurement reference to TS 28.558, as the report of UE level measurement already contain S-TMSI.

- Anonymization requirements for Rel-18 new defined job type

Besides, SA5 also introduce new job type for UE level measurement in Rel-18,

-	5GC UE level measurements only (7);

-	Trace and 5GC UE level measurements (8);

-	Immediate MDT and 5GC UE level measurements (9);

-	Trace, Immediate MDT and 5GC UE level measurements (10).

For the new job type “Immediate MDT and 5GC UE level measurements (9)” and “Trace, Immediate MDT and 5GC UE level measurements (10)”, SA5 not defined Anonymization requirements, which is not follow the previous mechanism.

Observation 3: SA5 defined new job type in TS 32.422 concerned about MDT, different with previous job type for MDT, the new added job type not applicable Anonymization requirements

Proposal 2: RAN3 would like to ask SA5 whether the Anonymization requirements are still applied for below new added job type:

-	Trace and 5GC UE level measurements (8);

-	Immediate MDT and 5GC UE level measurements (9);

-	Trace, Immediate MDT and 5GC UE level measurements (10).

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Observation 1: In REL-17 defined job type, anonymization requirements are applicable for all MDT measurements.

Observation 2: If follow the definition in the TS 28.558, the S-TMSI information will always include in the MDT report, which is against with Anonymization requirements.

Proposal 1: RAN3 would like to ask SA5 how to meet Anonymization requirements if MDT measurement reference to TS 28.558, as the report of UE level measurement already contain S-TMSI.

Observation 3: SA5 defined new job type in TS 32.422 concerned about MDT, different with previous job type for MDT, the new added job type not applicable Anonymization requirements

Proposal 2: RAN3 would like to ask SA5 whether the Anonymization requirements are still applied for below new added job type:

-	Trace and 5GC UE level measurements (8);

-	Immediate MDT and 5GC UE level measurements (9);

-	Trace, Immediate MDT and 5GC UE level measurements (10).

4. Reference

[1]	RAN3_121_agenda_20230825_EOM

[2]	R3-240058, 	Reply LS on improved KPIs involving end-to-end data volume transfer time analytics