3GPP TSG-RAN3 Meeting # 124 	R3-243590

# Fukuoka, Japan, 20th - 24th May. 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0404 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.401 on procedures of enabling multicast reception in RRC_INACTIVE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Nokia, CBN, Huawei, Samsung, Lenovo, CATT, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_enh-Core |  |  |  |  |  | Date: |  |  | 2024-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN3 had defined new messages and introduced numerous IEs on F1AP to enable the Multicast reception for UE in RRC_INACTIVE, however the message flow is not clearly captured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An exemplified interaction of F1AP and RRC protocol functions is provided when Multicast RRC_INACTIVE reception is enabled, in section 8.15.1.x. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The interaction on F1AP is not clearly defined for the newly introduced multicast reception for UE in RRC_INACTIVE state. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.15.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

// change starts.

8.15.1.x	Multicast reception in RRC_INACTIVE

Figure 8.15.x.1-1 illustrates an exemplified interaction of F1AP and RRC protocol functions when Multicast reception is enabled for UEs in RRC_INACTIVE.

Figure 8.15.1.x-1: Multicast reception in RRC_INACTIVE

0.	UE1 and UE2 served by the same gNB-DU have already joined an active Multicast session, and the Multicast session context has been established in both gNB-CU and the gNB-DU beforehand.

1.	The gNB-CU determines and enables the multicast reception in RRC_INACTIVE by sending the MC BEARER CONTEXT MODIFICATION REQUEST message to the gNB-DU, indicating that Multicast RRC_INACTIVE Reception Mode is activated in the Multicast CU to DU RRC Information on a per cell level for the multicast MBS session. The gNB-CU may further request MBS Multicast Configuration to be used by UEs in RRC_INACTIVE state by setting the MBS Multicast Configuration Request to "query".

2.	The gNB-DU enables the Multicast RRC_INACTIVE Reception Mode and provides the lower layer configuration to the gNB-CU in MBS Multicast Configuration included in Multicast DU to CU RRC Information.

3.	The gNB-CU decides to release the UE1 and send RRCRelease message to the UE1 with the latest MBS Multicast Configuration to be used by the UE1 after transitioning into RRC_INACTIVE state.

4/5.	5GC decides to deactivate the Multicast session by sending MULTICAST SESSION DEACTIVATION REQUEST message to gNB-CU.

6/7. 	Upon receiving session deactivation from 5GC (or other reason e.g., there is temporarily no data), the gNB-CU decides to update the multicast session reception state to the gNB-DU by sending the F1AP MC BEARER CONTEXT MODIFICATION REQUEST message, containing MBS Multicast Session Reception State set to “stop monitoring G-RNTI”. The gNB-DU indicates the UEs to stop monitoring the G-RNTI for the corresponding multicast session in the MBS Multicast Configuration.

8/9.	The gNB-DU may update the MBS Multicast Configuration in the Multicast DU to CU RRC Information, included in the F1AP MULTICAST CONTEXT NOTIFICATION INDICATION message.

10.	The gNB-CU decides to release the UE2 and send RRCRelease message to the UE2 with the latest MBS Multicast Configuration to be stored by the UE2 after transitioning into RRC_INACTIVE state. The UE2 may apply the stored MBS Multicast Configuration and start the Multicast data reception in RRC_INACTIVE once the Multicast session is activated.

// change ends.