3GPP TSG-RAN3 Meeting #124	R3-243602

# Fukuoka, Japan, 20th – 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.483 | CR | 0138 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on user plane failure handling (E1AP) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | gNB-CU-UPs are incapable to indicate a GTP-U exception has occurred. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing “GTP-U Report” into (unicast/BC/MC) Bearer Context Modification Required messages to indicate GTP-U exceptions of per-session tunnels or per-RB tunnels. Impact assessment towards the previous version of the specification (same release): This CR has isolated impact towards the previous version of the specification (same release). This CR has impact on NG-RAN basic function. This CR is backward compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | gNB-CU-UPs are incapable to indicate a GTP-U exception has occurred. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.3.2, 8.6.1.3.2, 8.6.2.3.2, 9.3.2.x1 (new), 9.3.2.x2 (new), 9.3.2.x3 (new), 9.3.2.x4 (new), 9.3.3.23, 9.3.3.30, 9.3.3.36, 9.4.5, 9.4.7. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

8.3.3	Bearer Context Modification Required (gNB-CU-UP initiated)

8.3.3.1	General

The purpose of the Bearer Context Modification Required procedure is to allow the gNB-CU-UP to modify a bearer context (e.g., due to local problems) and inform the gNB-CU-CP. The procedure uses UE-associated signalling.

8.3.3.2	Successful Operation

Figure 8.3.3.2-1: Bearer Context Modification Required procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the BEARER CONTEXT MODIFICATION REQUIRED message to the gNB-CU-CP. The gNB-CU-CP replies with the BEARER CONTEXT MODIFICATION CONFIRM message.

If the S1 DL UP Transport Layer Information IE or the NG DL UP Transport Layer Information IE or the Redundant NG DL UP Transport Layer Information IE is contained in the BEARER CONTEXT MODIFICATION REQUIRED message, the gNB-CU-CP shall update the corresponding information.

If the gNB-CU-UP Cell Group Related Configuration IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUIRED message, the gNB-CU-CP shall try to change the cell group related configuration accordingly. If the gNB-CU-CP is not able to update the requested cell group related configuration, it shall include the Cell Group Information IE with the current cell group configuration in the DRB Modified List IE in the BEARER CONTEXT MODIFICATION CONFIRM message.

If the GTP-U Report Session IE or the GTP-U Report DRB List IE is contained in the BEARER CONTEXT MODIFICATION REQUIRED message, the gNB-CU-CP shall determine the cause of the report(s) and handle it accordingly.

8.3.3.3	Abnormal Conditions

Not applicable.

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

8.6.1.3	BC Bearer Context Modification Required

8.6.1.3.1	General

The purpose of the gNB-CU-UP initiated BC Bearer Context Modification Required procedure is to allow the gNB-CU-UP to request the gNB-CU-CP to initiate the modification MBS session resources for a broadcast MBS session and inform the gNB-CU-CP. The procedure uses MBS-associated signalling.

8.6.1.3.2	Successful Operation

Figure 8.6.1.3.2-1: BC Bearer Context Modification Required procedure, gNB-CU-UP initiated: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the BC BEARER CONTEXT MODIFICATION REQUIRED message to the gNB-CU-CP. The gNB-CU-CP replies to the gNB-CU-UP with the BC BEARER CONTEXT MODIFICATION CONFIRM message.

If the GTP-U Report Session IE or the GTP-U Report BC MRB List IE is contained in the BC BEARER CONTEXT MODIFICATION REQUIRED message, the gNB-CU-CP shall determine the cause of the report(s) and handle it accordingly.

8.6.1.3.3	Abnormal Conditions

void.

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

8.6.2.3	MC Bearer Context Modification Required (gNB-CU-UP initiated)

8.6.2.3.1	General

The purpose of the gNB-CU-UP initiated MC Bearer Context Modification Required procedure is to allow the gNB-CU-UP to request the gNB-CU-CP to initiate the modification of MBS session resources for a multicast MBS session and inform the gNB-CU-CP. The procedure uses MBS-associated signalling.

8.6.2.3.2	Successful Operation

Figure 8.6.2.3.2-1: MC Bearer Context Modification Required procedure, gNB-CU-UP initiated: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the MC BEARER CONTEXT MODIFICATION REQUIRED message to the gNB-CU-CP. The gNB-CU-CP replies to the gNB-CU-UP with the MC BEARER CONTEXT MODIFICATION CONFIRM message.

If the MC Forwarding Resource Release Indication IE is contained in the MC BEARER CONTEXT MODIFICATION REQUIRED message, the gNB-CU-CP shall, if supported, assume that the indicated MC Forwarding Resource was released by the gNB-CU-UP.

If the GTP-U Report Session IE or the GTP-U Report IE is contained in the MC BEARER CONTEXT MODIFICATION REQUIRED message, the gNB-CU-CP shall determine the cause of the report(s) and handle it accordingly.

8.6.2.3.3	Abnormal Conditions

void

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

9.3.2.x1	GTP-U Report

This IE contains GTP-U reports concerning a given GTP-U tunnel.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Local UP TNL Info | M |  | UP Transport Layer Information 9.3.2.1 | The IP address and TEID at the side from which the report is originated. |
| UP Path Failure Report | O |  |  |  |
| >Remote GTP-U Peer | M |  | Transport Layer Address 9.3.2.4 | The failure occurs between this address and the node originating this report. Shall not be used in IE groups for a session. |
| UP Path Recovery Report | O |  |  |  |
| >Remote GTP-U Peer | M |  | Transport Layer Address 9.3.2.4 | The failure between this address and the node originating this report has recovered. Shall not be used in IE groups for a session. |
| Error Indication Report | O |  |  |  |
| >Remote UP TNL Info | M |  | UP Transport Layer Information 9.3.2.1 | The GTP-U Error Indication is sent from this address and indicating this GTP-U TEID. |

9.2.3.x2	GTP-U Report Session

This IE contains GTP-U reports concerning a Session.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| GTP-U Report DRB Item |  | 1..<maxnoofNGUTNLInformation> |  |  |
| >GTP-U Report | M |  | 9.3.2.x1 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofNGUTNLInformation | Maximum no. of NG-U Information allowed towards one Session, the maximum value is 2. |

9.3.2.x3	GTP-U Report DRB

This IE contains GTP-U reports concerning a DRB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| GTP-U Report DRB Item |  | 1..<maxnoofUPParameters> |  |  |
| >Cell Group ID | M |  | INTEGER (0..3, …) | This IE corresponds to information provided in the CellGroupId IE as defined in TS 38.331 [10] (0=MCG, 1=SCG). In this version of the specification, values “2” and “3” are not used. For E-UTRA Cell Groups, the same encoding is used as for NR Cell Groups. NOTE: There is no corresponding IE defined in TS 36.331 [33]. |
| >GTP-U Report | M |  | 9.3.2.x1 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUPParameters | Maximum no. of UP parameters (e.g., GTP tunnels) for a DRB. Value is 8. |

9.3.2.x4	GTP-U Report BC MRB

This IE contains GTP-U reports concerning a BC MRB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| GTP-U Report BC MRB Item |  | 1..<maxnoofDUs> |  |  |
| >Broadcast F1-U Context ReferenceE1 | M |  | 9.3.1.139a |  |
| >GTP-U Report | M |  | 9.3.2.x1 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDUs | Maximum no. of DUs in a RAN node. Value is 512. |

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

9.3.3.23	PDU Session Resource Required To Modify List

This IE contains PDU session resource to modify related information used at Bearer Context Modification Required

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource Required To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  | - |  |
| >PDU Session ID | M |  | 9.3.1.21 |  | - |  |
| >NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - |  |
| >DRB To Modify List |  | 0.. 1 |  |  | - |  |
| >>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | - |  |
| >>>DRB ID | M |  | 9.3.1.16 |  | - |  |
| >>>gNB-CU-UP Cell Group Related Configuration | O |  | 9.3.1.34 |  | - |  |
| >>>Flow To Remove | O |  | QoS Flow List 9.3.1.12 |  | - |  |
| >>>Cause | O |  | 9.3.1.2 |  | - |  |
| >>>GTP-U Report DRB | O |  | 9.3.2.x3 |  | YES | reject |
| >DRB To Remove List |  | 0.. 1 |  |  | - |  |
| >>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  | - |  |
| >>>DRB ID | M |  | 9.3.1.16 |  | - |  |
| >>>Cause | M |  | 9.3.1.2 |  | - |  |
| >Redundant NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |
| >GTP-U Report Session | O |  | 9.3.2.x2 | For the NG-U tunnel. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

#### 9.3.3.30	BC Bearer Context To Modify Required

This IE contains MBS session resource related information used to request BC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| BC MRB To Remove List Required |  | 0..<maxnoofMRBs> |  |  | - |  |
| >MRB ID | M |  | 9.3.1.16a |  | - |  |
| BC MRB To Modify Required List |  | 0..<maxnoofMRBs> |  |  | YES | reject |
| >MRB ID | M |  | 9.3.1.16a |  | - |  |
| >GTP-U Report BC MRB | O |  | 9.3.2.x4 |  | - |  |
| GTP-U Report | O |  | 9.3.2.x1 | For the N3mb tunnel. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for one MBS Session. Value is 32. |
| maxnoofDUs | Maximum no. of DUs in a RAN node. Value is 512. |

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

#### 9.3.3.36	MC Bearer Context To Modify Required

This IE contains MBS session resource related information used to request MC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Multicast F1-U Context Descriptor | C-ifRemoved |  | 9.3.1.125 |  | - |  |
| MC MRB To Remove List Required |  | 0..<maxnoofMRBs> |  |  | - |  |
| >MRB ID | M |  | 9.3.1.16a |  | - |  |
| MC MRB To Modify List Required |  | 0..<maxnoofMRBs> |  |  | - |  |
| >MRB ID | M |  | 9.3.1.16a |  | - |  |
| >MBS PDCP COUNT | O |  | 9.3.1.35a |  | - |  |
| >GTP-U Report | O |  | 9.3.2.x1 |  | YES | reject |
| MC Forwarding Resource Release Indication | O |  | 9.3.1.138 | Indicates the release of an MC Forwarding Resource | YES | ignore |
| GTP-U Report | O |  | 9.3.2.x1 | For the N3mb tunnel. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for one MBS Session. Value is 32. |

| Condition | Explanation |
| --- | --- |
| ifRemoved | This IE shall be present if either the MC MRB To Remove List Required IE is included. |

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

id-BC-MRB-ToModifyRequired-List,

id-GTP-U-Report,

id-GTP-U-ReportSession,

id-GTP-U-ReportDRB,

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

#### -- B

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

##### -- BCBearerContextToModifyRequired

BCBearerContextToModifyRequired ::= SEQUENCE {

bcMRBToRemoveList	BCMRBRemoveConfiguration			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToModifyRequired-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToModifyRequired-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-BC-MRB-ToModifyRequired-List			CRITICALITY reject	EXTENSION	BC-MRB-ToModifyRequired-List	PRESENCE optional	}|

{ ID id-GTP-U-ReportSession						CRITICALITY reject	EXTENSION	GTP-U-Report						PRESENCE optional	},

...

}

BC-MRB-ToModifyRequired-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF BC-MRB-ToModifyRequired-Item

BC-MRB-ToModifyRequired-Item ::= SEQUENCE {

mrb-ID								MRB-ID,

gTP-U-ReportBC-MRB					GTP-U-ReportBC-MRB								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BC-MRB-ToModifyRequired-Item-ExtIEs} }	OPTIONAL,

...

}

BC-MRB-ToModifyRequired-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

#### -- D

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

DRB-Required-To-Modify-List-NG-RAN ::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Required-To-Modify-Item-NG-RAN

DRB-Required-To-Modify-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

gNB-CU-UP-CellGroupRelatedConfiguration	GNB-CU-UP-CellGroupRelatedConfiguration		OPTIONAL,

flow-To-Remove							QoS-Flow-List								OPTIONAL,

cause									Cause		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Required-To-Modify-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Required-To-Modify-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-GTP-U-ReportDRB							CRITICALITY reject	EXTENSION	GTP-U-ReportDRB			PRESENCE optional	},

...

}

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

#### -- G

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

GTP-U-Report ::= SEQUENCE {

localUP-TNL-Info					UP-TNL-Information,

uP-PathFailureReport				UP-PathFailureReport					OPTIONAL,

uP-PathRecoveryReport				UP-PathRecoveryReport					OPTIONAL,

errorIndicationReport				ErrorIndicationReport					OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {GTP-U-Report-ExtIEs} } OPTIONAL,

...

}

GTP-U-Report-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

UP-PathFailureReport ::= SEQUENCE {

remoteGTP-U-Peer					TransportLayerAddress,

iE-Extensions						ProtocolExtensionContainer { {UP-PathFailureReport-ExtIEs} } OPTIONAL,

...

}

UP-PathFailureReport-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

UP-PathRecoveryReport ::= SEQUENCE {

remoteGTP-U-Peer					TransportLayerAddress,

iE-Extensions						ProtocolExtensionContainer { {UP-PathRecoveryReport-ExtIEs} } OPTIONAL,

...

}

UP-PathRecoveryReport-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

ErrorIndicationReport ::= SEQUENCE {

remoteUP-TNL-Info					UP-TNL-Information,

iE-Extensions						ProtocolExtensionContainer { {ErrorIndicationReport-ExtIEs} } OPTIONAL,

...

}

ErrorIndicationReport-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GTP-U-ReportBC-MRB ::= SEQUENCE (SIZE(1..maxnoofDUs)) OF GTP-U-ReportBC-MRB-Item

GTP-U-ReportBC-MRB-Item ::= SEQUENCE {

broadcastF1U-ContextReferenceE1				BroadcastF1U-ContextReferenceE1,

gTP-U-Report								GTP-U-Report,

iE-Extensions				ProtocolExtensionContainer { {GTP-U-ReportBC-MRB-Item-ExtIEs } }	OPTIONAL

}

GTP-U-ReportBC-MRB-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

GTP-U-ReportDRB ::= SEQUENCE (SIZE(1.. maxnoofUPParameters)) OF GTP-U-ReportDRB-Item

GTP-U-ReportDRB-Item ::= SEQUENCE {

cell-Group-ID				Cell-Group-ID,

gTP-U-Report				GTP-U-Report,

iE-Extensions				ProtocolExtensionContainer { {GTP-U-ReportDRB-Item-ExtIEs } }	OPTIONAL

}

GTP-U-ReportDRB-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

GTP-U-ReportSession ::= SEQUENCE (SIZE(1..maxnoofNGUTNLInformation)) OF GTP-U-ReportSession-Item

GTP-U-ReportSession-Item ::= SEQUENCE {

gTP-U-Report				GTP-U-Report,

iE-Extensions				ProtocolExtensionContainer { {GTP-U-ReportSession-Item-ExtIEs } }	OPTIONAL

}

GTP-U-ReportSession-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

#### -- M

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

##### -- MCBearerContextToModifyRequired

MCBearerContextToModifyRequired ::= SEQUENCE {

mbsMulticastF1UContextDescriptor					MBSMulticastF1UContextDescriptor				OPTIONAL,

-- This IE shall be present if either the MC MRB To Remove List Required IE is included.

mcMRBToRemoveRequiredList						MCMRBRemoveConfiguration						OPTIONAL,

mcMRBToModifyRequiredList						MCMRBModifyRequiredConfiguration				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextToModifyRequired-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextToModifyRequired-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ID id-MCForwardingResourceReleaseIndication	CRITICALITY ignore	EXTENSION MCForwardingResourceReleaseIndication		PRESENCE optional}|

{ID id-GTP-U-ReportSession						CRITICALITY reject	EXTENSION GTP-U-Report											PRESENCE optional	},

...

}

MCMRBModifyRequiredConfiguration ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MCMRBModifyRequiredConfiguration-Item

MCMRBModifyRequiredConfiguration-Item ::= SEQUENCE {

mrb-ID							MRB-ID,

mBS-PDCP-COUNT					MBS-PDCP-COUNT															OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MCMRBModifyRequiredConfiguration-Item-ExtIEs} }	OPTIONAL,

...

}

MCMRBModifyRequiredConfiguration-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ID id-GTP-U-Report								CRITICALITY reject	EXTENSION GTP-U-Report											PRESENCE optional	},

...

}

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

#### -- P

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

PDU-Session-Resource-Required-To-Modify-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Required-To-Modify-Item

PDU-Session-Resource-Required-To-Modify-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

nG-DL-UP-TNL-Information				UP-TNL-Information						OPTIONAL,

dRB-Required-To-Modify-List-NG-RAN		DRB-Required-To-Modify-List-NG-RAN		OPTIONAL,

dRB-Required-To-Remove-List-NG-RAN				DRB-Required-To-Remove-List-NG-RAN				OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Required-To-Modify-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Required-To-Modify-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-redundant-nG-DL-UP-TNL-Information		CRITICALITY ignore	EXTENSION 	UP-TNL-Information	PRESENCE optional	}|

{ ID id-GTP-U-ReportSession						CRITICALITY reject	EXTENSION	GTP-U-Report			PRESENCE optional	},

...

}

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

9.4.7	Constant Definitions

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

maxnoofNGUTNLInformation					INTEGER ::= 2

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

//////////////////////////////////////////////////////////////////skip unrelated//////////////////////////////////////////////////////////////////

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-BC-MRB-ToModifyRequired-List									ProtocolIE-ID ::=

id-GTP-U-Report													ProtocolIE-ID ::=

id-GTP-U-ReportSession											ProtocolIE-ID ::=

id-GTP-U-ReportDRB												ProtocolIE-ID ::=

//////////////////////////////////////////////////////////////////end//////////////////////////////////////////////////////////////////