3GPP TSG-RAN WG3 Meeting #124	R3-243680

# Fukuoka, Japan, 20 - 24 May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR | draft | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of pre-configured SRS activation – Set2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Huawei, ZTE, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As specified in TS 38.331 and TS 38.305, a UE may be pre-configured with one or more SRS for positioning configurations with validity area. A UE may send a RRC Resume Request message to the receiving gNB to request activation of the pre-configured SRS for positioning. However, the overall procedure is currently missing in Stage 2. In case the receiving gNB could identify the “reserved” pre-configrued SRS configuration for the UE, it notifies the last serving gNB pre-configured SRS configuration(s) is reserved, the last serving gNB shall, if supported, generate RRCRelease and include it in the Retrieve UE Context Failure for pre-configured SRS activation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add preconfigured SRS Configuration Activation Procedure in the stage 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing Stage 2 support for preconfigured SRS Configuration Activation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.14.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 38.423 CR 1302 TS/TR 38.455 CR 0155 TS/TR 38.473 CR 1432 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of the change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 7.14.y	Pre-configured SRS Configuration Activation Procedure

Figure 7.14.y-1 shows the Pre-configured SRS Configuration Activation procedure, with pre-configured SRS reserved in the Receiving gNB.

Figure 7.14.y-1: Pre-configured SRS Configuration Activation, with pre-configured SRS reserved

0.	The UE in RRC_INACTIVE state is pre-configured with one or more area-specific SRS for positioning configurations as specified in clause 7.14.2 and receives a request from upper layers to activate a pre-configured SRS for positioning. The pre-configured SRS configuration(s) is reserved in the Receiving gNB.

1.	The UE sends a RRCResumeRequest message with the resume cause "srs-PosConfigOrActivationReq" to request activation of the pre-configured SRS.

2.	The receiving gNB identifies the reserved pre-configured SRS for the UE, triggers the Retrieve UE Context procedure towards the last serving gNB and indicates pre-configured SRS reserved in the Retrieve UE Context Request message.

Note: If the receiving gNB does not have the valid pre-configured SRS for the UE, it triggers the Retrieve UE Context procedure towards the last serving gNB and indicates UE is request for SRS Positioning Config or Pre-configured SRS Activation.

3.	The last serving gNB sends a NRPPa Positioning Information Update message to the LMF including the Cell ID of the receiving gNB where the UE resumes from, and a pre-configured SRS activation indicator to notify the LMF that the pre-configured SRS is activated for the UE.

4.	The last serving gNB sends Retrieve UE Context Failure to the Receiving gNB, which includes the RRCRelease message.

5. The receiving gNB forwards the RRCRelease message to the UE, the UE activates the pre-configured SRS when receives the RRCRelease message.

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of the change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>