3GPP TSG-RAN WG3 #124		R3-243747

Fukuoka, Japan, 20 - 24 May 2024

Title:	[DRAFT] Reply LS on inter-node RRC message for intra-SN SCPAC in MN format

Response to:	R3-243113

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	Ericsson [to become RAN3]

To:	RAN2

Cc:

Contact Person:

#### Name:	Liwei Qiu

E-mail Address:	liwei.qiu@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

1. Overall Description:

RAN3 has discussed the following understanding from RAN2.

RAN2 understands both CG-Config message associated with the source SCG and CG-CandidateList message associated with candidate PSCell(s) can be included in one SN Modification Required message for intra-SN subsequent CPAC in MN format.

RAN3 concluded that the two RRC messages don't need to be sent within one SN MODIFICATION REQUIRED message. Instead, it should allow network implementations to trigger the procedure multiple times if necessary. Thus, no specification change is required.

2. Actions:

To RAN2 group.

ACTION: 	RAN3 respectfully asks RAN2 to take the above into account.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #125	19-23 August 2024			Maastricht, The Netherlands