3GPP TSG-RAN WG3 Meeting #124                      R3-243796

Fukuoka, Japan, 20 – 24 May, 2024

Title: 		Summary of Positioning offline discussion

Source: 	CATT

Agenda item:	9.2

Document Type:	Decision

- Introduction

This is the SoD of the discussion for:

CB: # 13_Positioning

- For pre-configured SRS activation: Start with Solution a)

- Check 3794 and 3795

- Check other CRs if time is allowed

(moderator - CATT)

Summary of offline disc R3-243796

- For Chair’s Notes

/////////////////////////////////////////////////// Begin /////////////////////////////////////////////////////

For pre-configured SRS activation:

Proposed Way Forward: further work on solutions a + c2, with following assumptions:

- For the solution a),

  - pending to RAN2 decision (RRC container based or Xn based solution) to transfer the pre-configured SRS from the last serving gNB to the receiving gNB.

  - New indicator (e.g. SRS Positioning Configuration Or Activation Request) should be introduced in XnAP Retrieve UE Context Request.

  - gNB should notify LMF the pre-configured SRS is activated for the UE (via a new indicator, or extend the SRS Transmission Status) in the NRPPa Positioning Information Update.

- For the solution c2), pre-configured SRS is already included in the RRCRelease message generated by the last serving gNB, it works without any extra stage 3 impact.

For the CR Checking:

CR 3794/3795: Agreeable.

CR 3751/3752: Agreeable.

CR 3779 is revised to 3805, agreeable

CR 3780 is revised to 3806, agreeable

CR 3481 is revised to 3855, agreeable

CR 3482 is revised to 3856, agreeable

CR 3359 is revised to 3877, agreeable

CR 3360 is revised to 3878, agreeable

CR 3438/3439:

Further check and come back next meeting, if the change is needed, or check with RAN1 the understanding on using of the measurement windows. And also whether need to add the start time in the IE.

draftCR 3570:

Take it into account, Companies could check and consider the further refinement the next meeting. (e.g. For TRP measurement, use Time Window Information or Time Window Information Measurement List?)

3440 (TRP ID), No time to further discuss the details this meeting, to be continued.

/////////////////////////////////////////////////// End /////////////////////////////////////////////////////

- Discussion

3.1 Pre-configured SRS activation

For solution a), take 3430 and 3568 into account for XnAP:

- New code-point should be introduced in Retrieve UE Context Request, the naming in 3568 seems better, due to the receiving gNB does not know what’s the real intension of the UE.

In 3430:

| Preconfigured SRS Activation Request | O |  | ENUMERATED(true, …) |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

If the Preconfigured SRS Activation Request IE is included in the RETRIEVE UE CONTEXT REQUEST message, the old NG-RAN node shall, if supported, include the Positioning Validity Area Configuration IE in UE Context Information – Retrieve UE Context Response IE contained in the RETRIEVE UE CONTEXT RESPONSE message.

In 3568:

| SRS Positioning Configuration Or Activation Request | O |  | (true, …) |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

If the SRS Positioning Configuration Or Activation Request IE set to "true" is included in the RETRIEVE UE CONTEXT REQUEST message, the old NG-RAN may include the Positioning Information IE in the RETRIEVE UE CONTEXT RESPONSE message and make corresponding response to LMF when positioning a UE.

Use the encoding “SRS Positioning Configuration Or Activation Request”

- Pre-configured SRS configurations should be relocated to the new gNB in case of full context relocation.

We should discuss and decide how to relocate the pre-configured SRS, two options:

- Via RRC Container? E.g. the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331?

- Via XnAP IE? How to encode the IE?

9.2.3.X	Positioning Validity Area Configuration

This IE contains positioning Validity Area SRS configuration of the UE.

| IE/Group Name |  | Presence |  | Range | IE type and reference | Semantics description |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SRS-PosRRC-InactiveValidityAreaConfigList | M |  |  |  |  |  | OCTET STRING | Includes the SRS-PosRRC-InactiveValidityAreaConfigList IE as defined in TS 38.331 [8]. |  |
| Routing ID | M |  |  |  |  |  | OCTET STRING | The maximum length corresponds to NfInstanceId defined in TS 29.571 [50]. |  |
| NRPPa Transaction ID | M |  |  |  |  |  | INTEGER (0..32767) | NRPPa Transaction ID, as defined in TS 38.455 [49] |  |

- Pre-configured SRS in RRC container or not is to be decided by RAN2, check RAN2 progress this week.

For solution a), 3307 and 3684 provides the similar changes to NRPPa

3307 extend the enumerated value for SRS Transmission Status IE in Positioning Information Update message.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| SFN Initialisation Time | O |  | Relative Time 1900 9.2.36 |  | YES | ignore |
| UE Tx TEG Association List | O |  | 9.2.78 |  | YES | ignore |
| SRS Transmission Status | O |  | ENUMERATED (stopped, ..., preconfigured SRS activated) |  | YES | ignore |
| New Cell Identity | O |  | NR CGI 9.2.9 |  | YES | ignore |

3684 introduced a new IE in Positioning Information Update message to indicate the pre-configured SRS is activated.

| Pre-configured SRS Activation | O |  | ENUMERATED(true, …) |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

The moderator understands both ways are workable, let’s decide which way to go.

Another issue is, whether C2 could be supported with/without stage 3 impact?

The moderator understand either a) or c2) could be decided by the last serving gNB. Even if MAC-CE is used to activate the semi-persistent SRS for the UE, for periodic SRS case, the last serving gNB may decide to use Retrieve failure by including RRCRelease to activate the pre-configured SRS for the UE. In this case, not necessary to relocate the pre-configured SRS towards the receiving gNB.

for c2), pre-configured SRS is already included in the RRCRelease message generagted by the last serving gNB, it works without any stage 3 impact.

Proposed WF: Further work on a+c2

- for the solution a),

  - pending to RAN2 decision (RRC container based or Xn based solution) to transfer the pre-configured SRS from the last serving gNB to the receiving gNB.

  - New indicator (e.g. SRS Positioning Configuration Or Activation Request) should be introduced in XnAP Retrieve UE Context Request.

  - gNB should notify LMF the pre-configured SRS is activated for the UE (via a new indicator, or extend the SRS Transmission Status) in the NRPPa Positioning Information Update.

- for the solution c2), pre-configured SRS is already included in the RRCRelease message generated by the last serving gNB, it works without any extra stage 3 impact.

2.2 CR Checking

- Check 3794 and 3795

| R3-243434 | UE Context Release Command ASN.1 correction (Huawei, Nokia, CATT, ZTE, Samsung) | CR1423r, TS 38.473 v17.8.0, Rel-17, Cat. F E///: Why E/// is not added - Add E/// as co-source - Editorial updates needed: Unnecessary failure, rejection and stop of procedure involving the UE CONTEXT RELEASE COMMAND message Rev in R3-243794 |
| --- | --- | --- |
| R3-243435 | UE Context Release Command ASN.1 correction (Huawei, Nokia, CATT, ZTE, Samsung) | CR1424r, TS 38.473 v18.1.0, Rel-18, Cat. A - Add E/// as co-source - Editorial updates needed: Unnecessary failure, rejection and stop of procedure involving the UE CONTEXT RELEASE COMMAND message Rev in R3-243795 |

Moderator’s view: Any further issue or comment? If not, please CR owner revise the coversheet as mentioned in the Chair Notes above, and the revised CRs are agreeable.

with the proposed change on the coversheet, the CRs above could be agreed.

- Check other CRs if time is allowed

| R3-243436 | Correction on Measurement Reporting for BWA (Huawei, ZTE, Xiaomi) | CR0149r, TS 38.455 v18.1.0, Rel-18, Cat. F Rev in R3-243779 |
| --- | --- | --- |
| R3-243437 | Correction on Measurement Reporting for BWA (Huawei, ZTE, Xiaomi) | CR1425r, TS 38.473 v18.1.0, Rel-18, Cat. F Rev in R3-243780 |

9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA Measurement |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >>UL RSCP |  |  | 9.2.92 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >Mobile TRP Location Information | O |  | 9.2.88 |  | YES | ignore |
| >Measured Frequency Hops | O |  | ENUMERATED (singleHop, multiHop, …) |  | YES | ignore |
| >Aggregated Positioning SRS Resource ID List |  | 0..1 |  | Indicates the used SRS for positioning resources across aggregated carriers. | YES | ignore |
| >>Aggregated Positioning SRS Resource ID Item |  | 2..< maxnoaggregatedPosSRS-Resources > |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |  |  |
| > Measurement Based On Aggregated Resources | O |  | ENUMERATED(true, …) |  | YES | ignore |

Companies are fine with the changes, revision is needed, add Nok, Ericsson,CATT as co-source.

| R3-243438 | Correction on Measurement Time Window (Huawei) | CR0150r, TS 38.455 v18.1.0, Rel-18, Cat. F |
| --- | --- | --- |
| R3-243439 | Correction on Measurement Time Window (Huawei) | CR1426r, TS 38.473 v18.1.0, Rel-18, Cat. F |

9.2.91	Time Window Information Measurement List

This IE contains the time window(s) when UL SRS measurement is requested.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Time Window Information Measurement List |  | 1 |  |  |
| >Time Window Information Measurement Item |  | 1..<maxnoofTimeWindowMeas> |  |  |
| >>CHOICE Time Window Duration | M |  |  | Duration of time window with start time given by the System Frame Number IE and Slot Number IE. |
| >>>Slots |  |  |  |  |
| >>>>Duration in Slots | M |  | ENUMERATED (1, 2, 4, 6, 8, 12, 16, …) |  |
| >>Time Window Type | M |  | ENUMERATED (single, periodic, …) |  |
| >>Time Window Periodicity | C-ifTimeWindowTypePeriodic |  | ENUMERATED (160, 320, 640, 1280, 2560, 5120, 10240, 20480, 40960, 61440, 81920, 368640, 737280, 1843200, …) | Unit: Milli-seconds |
| >>Indicated UL SRS Resource List |  | 0..1 |  |  |
| >>>Indicated UL SRS Resource Item |  | 1..<maxnoSRS-PosResources> |  |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |
| >>Time Window Measurements Indicator | O |  | ENUMERATED(true, …) |  |

Moderator’s view: why need to introduce a new IE Time Window Measurements Indicator for each of the time window.

Further check and come back next meeting, if the change is needed, or check with RAN1 the understanding on using of the measurement windows. And also whether need to add the start time in the IE.

| R3-243481 | Correction on UL-RSCP (Ericsson, Nokia, Qualcomm Inc.) | CR0153r, TS 38.455 v18.1.0, Rel-18, Cat. F |
| --- | --- | --- |
| R3-243482 | Correction on UL-RSCP (Ericsson, Nokia, Qualcomm Inc.) | CR1430r, TS 38.473 v18.1.0, Rel-18, Cat. F |

Moderator’s view: It’s correct, but whether need to add the abnormal conditions in the procedure texts:

If the requested measurements include the UL-RSCP measurement but do not include either the gNB-RxTxTimeDiff or UL-RTOA measurements, the NG-RAN node shall respond with a MEASUREMENT FAILURE message with an appropriate cause value.

Or just need to add some texts in the semantics description?

| >>Positioning Measurement Type | M |  | ENUMERATED (gNB RX-TX, UL-SRS-RSRP, UL AoA, UL RTOA, …, Multiple UL AoA, UL SRS-RSRPP, UL-RSCP) | The UL-RSCP measurement is applicable only when the UL-RTOA and/or gNB-RxTxTimeDiff measurement(s) is also requested. | - | - |
| --- | --- | --- | --- | --- | --- | --- |

Semantics description is a soft way, take this way. Revise the CRs, just keep the semantics only.

| R3-243570 | Update of stage 2 positioning information (Nokia, Ericsson, Huawei) | draftCR |
| --- | --- | --- |

Table 8.10.2.4-2: TRP Measurement request information that may be transferred from LMF to gNBs.

Information

TRP ID, and NCGI of the TRP to receive UL-SRS

UE-SRS configuration

UL timing information together with timing uncertainty, for reception of SRS by candidate TRPs

Report characteristics for the measurements

Measurement Quantities

Measurement periodicity

Measurement beam information request

Search window information

Expected UL AoA/ZoA and uncertainty range

Number of TRP Rx TEGs

Number of TRP RxTx TEGs

Response time

Measurement characteristics request indicator

Measurement time occasions for a measurement instance

Time window information

Table 8.13.2.3-1: Requested UL-SRS transmission characteristics information that may be transferred from LMF to gNB.

Information

Number Of Transmissions/duration for which the UL-SRS is requested

Bandwidth

Resource type (periodic, semi-persistent, aperiodic)

Pathloss reference: - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID

Spatial relation info - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID - NZP CSI-RS Resource ID - SRS Resource ID - Positioning SRS Resource ID

SSB Information

Periodicity of the SRS for each SRS resource set

Carrier frequency of SRS transmission bandwidth

Bandwidth Aggregation Request Indication

Positioning Validity Area Cell List

Validity Area Specific SRS Information

Time Window Information SRS List

Moderator’s view: Generally, it’s good to add the information missed in the stage 2.

For TRP measurement, Time Window Information or Time Window Information Measurement List?

Take it into account, Companies could check and consider the further refinement the next meeting.

| R3-243751 | Correction on SIB23 in TS38.470 (ZTE, Ericsson, Huawei, CATT, Xiaomi, Nokia) | CR0151r, TS 38.470 v18.1.0, Rel-18, Cat. F |
| --- | --- | --- |
| R3-243752 | Correction on SIB23 in TS38.473 (ZTE, Ericsson, Huawei, CATT, Xiaomi, Nokia) | CR1437r, TS 38.473 v18.1.0, Rel-18, Cat. F |

Moderator’s view: Straightforward, check if they’re agreeable to us.

Agreeable.

| R3-243359 | Correction to DL-PRS Aggregation (Qualcomm Incorporated) | CR0143r1, TS 38.455 v18.1.0, Rel-18, Cat. F |
| --- | --- | --- |
| R3-243360 | Correction to DL-PRS Aggregation (Qualcomm Incorporated) | CR1420r, TS 38.473 v18.1.0, Rel-18, Cat. F |

9.2.61	Requested DL PRS Transmission Characteristics

This IE contains the requested PRS configuration for transmission by the LMF.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Requested DL-PRS Resource Set List |  | 1 |  |  | - |  |
| >Requested DL-PRS Resource Set Item |  | 1..<maxnoofPRSresourceSet> |  |  | - |  |
| >>PRS bandwidth | O |  | INTEGER(1..63) | 24,28,…,272 PRBs | - |  |
| >>Comb Size | O |  | ENUMERATED(2, 4, 6, 12, …) |  | - |  |
| >>Resource Set Periodicity | O |  | ENUMERATED(4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560,5120,10240,20480,40960,81920,…,128, 256, 512) | Slots | - |  |
| >>Resource Repetition Factor | O |  | ENUMERATED(rf1,rf2,rf4,rf6,rf8,rf16,rf32,…) |  | - |  |
| >>Resource Number of Symbols | O |  | ENUMERATED(n2,n4,n6,n12,…,n1) |  | - |  |
| >>Requested DL-PRS Resource List | O |  | 9.2.62 |  | - |  |
| >>Resource Set Start Time and Duration | O |  | Start Time and Duration 9.2.63 | This IE is ignored if the Start Time and Duration IE is present | - |  |
| Number of Frequency Layers | O |  | INTEGER(1..4) |  | - |  |
| Start Time and Duration | O |  | 9.2.63 |  | - |  |
| PRS Bandwidth Aggregation Request Information | O |  | 9.2.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPRSresourceSet | Maximum no of PRS resources set. Value is 8. |

9.2.95	Aggregated PRS Resource Set List

This information element is used to indicate the aggregated PRS Resource Set List.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Aggregated PRS Resource Set List |  | 1..<maxnoAggLists> |  |  |
| >Aggregated Positioning PRS Resource Set Item |  | 1.. < maxnoAggPosPRSResourceSets> |  |  |
| >>DL-PRS Resource Set Index |  |  | INTEGER(1..8) | This IE specifies the PRS Resource Set Item's that are linked for DL-PRS bandwidth aggregation. The Integer Value defines an index to the PRS Resource Set Item in IE PRS Configuration (9.2.44). Integer value 1 defines the first entry in PRS Resource Set Item, Integer value 2 defines the second entry in PRS Resource Set Item and so on. |

| Range bound | Explanation |
| --- | --- |
| maxnoAggLists | Maximum number of aggregated frequency layer (carrier) combinations. Value is 2 |
| maxnoAggPosPRSResourceSets | Maximum no of PRS resource sets aggregated. Value is 3. |

9.2.x	PRS Bandwidth Aggregation Request Information

This IE contains the aggregated DL-PRS configuration information requested.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PRS Bandwidth Aggregation Request Information |  | 1..<maxnoAggLists> |  |  |
| >PRS Bandwidth Aggregation Request Information Item |  | 1..< maxnoAggPosPRSResourceSets> |  |  |
| >>Requested DL-PRS Resource Set Index | M |  | INTEGER(1..8) | This IE specifies which of the indicated Requested DL-PRS Resource Set Item's are requested for aggregation. The Integer Value defines an index to the Requested DL-PRS Resource Set Item in IE Requested DL PRS Transmission Characteristics (9.2.61). Integer value 1 defines the first entry in Requested DL-PRS Resource Set Item, Integer value 2 defines the second entry in Requested DL-PRS Resource Set Item and so on. |

| Range bound | Explanation |
| --- | --- |
| maxnoAggLists | Maximum number of aggregated frequency layer (carrier) combinations. Value is 2. |
| maxnoAggPosPRSResourceSets | Maximum no of PRS resources sets aggregated. Value is 3. |

Moderator’s view: Changes to “9.2.95	Aggregated PRS Resource Set List” is ok. Whether need to change PRS Bandwidth Aggregation Request Information need to be discussed.

Base on the offline discussion, changes #1, #2 are agreeable. Change #3 could be considered after further checking with RAN1/RAN2.

The CRs should be updated to take changes #1, #2 only.

| R3-243440 | Logical TRP IDs management (Huawei) | discussion |
| --- | --- | --- |

Moderator’s view: Seems like implementation/deployment method, no need to specify.

No time to further discuss the details this meeting, to be continued.

- Reference

- R3-243678 Further Consideration on pre-configured SRS activation (CATT, Huawei, ZTE, Samsung)

- R3-243568 Support of pre-configured SRS activation- Set1 (Huawei, CATT, Samsung, ZTE)

- R3-243684 Support of Preconfigured SRS Activation - Set 1 (ZTE, CATT, Huawei, Samsung)

- R3-243679 Support of pre-configured SRS activation – Set1 (CATT, Huawei, ZTE, Samsung)

- R3-243616 [draft] LS on area specific SRS configuration (Samsung, Huawei, CATT, ZTE)

- R3-243569 Support of pre-configured SRS activation- Set2 (Huawei, CATT, Samsung, ZTE)

- R3-243680 Support of pre-configured SRS activation – Set2 (CATT, Huawei, ZTE, Samsung)

- R3-243685 Support of Preconfigured SRS Activation - Set 2 (ZTE, CATT, Huawei, Samsung)

- R3-243615 Support of pre-configured SRS activation - Set2 (Samsung, CATT, Huawei, ZTE)

- R3-243307 Support of the pre-Configured SRS activation (Xiaomi, Ericsson, Nokia, Qualcomm Incorporated)

- R3-243347 Missing procedure description for pre-configured SRS activation (Qualcomm Incorporated, Ericsson, Nokia, Xiaomi)

- R3-243430 Support of pre-Configured SRS activation using full context relocation (Ericsson, Xiaomi, Nokia, Qualcomm Incorporated)

- R3-243434 UE Context Release Command ASN.1 correction (Huawei, Nokia, CATT, ZTE, Samsung)

- R3-243435 UE Context Release Command ASN.1 correction (Huawei, Nokia, CATT, ZTE, Samsung)

- R3-243436 Correction on Measurement Reporting for BWA (Huawei, ZTE, Xiaomi)

- R3-243437 Correction on Measurement Reporting for BWA (Huawei, ZTE, Xiaomi)

- R3-243438 Correction on Measurement Time Window (Huawei)

- R3-243439 Correction on Measurement Time Window (Huawei)

- R3-243481 Correction on UL-RSCP (Ericsson, Nokia, Qualcomm Inc.)

- R3-243482 Correction on UL-RSCP (Ericsson, Nokia, Qualcomm Inc.)

- R3-243570 Update of stage 2 positioning information (Nokia, Ericsson, Huawei)

- R3-243751 Correction on SIB23 in TS38.470 (ZTE, Ericsson, Huawei, CATT, Xiaomi, Nokia)

- R3-243752 Correction on SIB23 in TS38.473 (ZTE, Ericsson, Huawei, CATT, Xiaomi, Nokia)

- R3-243359 Correction to DL-PRS Aggregation (Qualcomm Incorporated)

- R3-243360 Correction to DL-PRS Aggregation (Qualcomm Incorporated)

- R3-243440 Logical TRP IDs management (Huawei)

| R3-243434 | UE Context Release Command ASN.1 correction (Huawei, Nokia, CATT, ZTE, Samsung) |
| --- | --- |
| R3-243435 | UE Context Release Command ASN.1 correction (Huawei, Nokia, CATT, ZTE, Samsung) |
| R3-243436 | Correction on Measurement Reporting for BWA (Huawei, ZTE, Xiaomi) |
| R3-243437 | Correction on Measurement Reporting for BWA (Huawei, ZTE, Xiaomi) |
| R3-243438 | Correction on Measurement Time Window (Huawei) |
| R3-243439 | Correction on Measurement Time Window (Huawei) |
| R3-243481 | Correction on UL-RSCP (Ericsson, Nokia, Qualcomm Inc.) |
| R3-243482 | Correction on UL-RSCP (Ericsson, Nokia, Qualcomm Inc.) |
| R3-243570 | Update of stage 2 positioning information (Nokia, Ericsson, Huawei) |
| R3-243751 | Correction on SIB23 in TS38.470 (ZTE, Ericsson, Huawei, CATT, Xiaomi, Nokia) |
| R3-243752 | Correction on SIB23 in TS38.473 (ZTE, Ericsson, Huawei, CATT, Xiaomi, Nokia) |
| R3-243359 | Correction to DL-PRS Aggregation (Qualcomm Incorporated) |
| R3-243360 | Correction to DL-PRS Aggregation (Qualcomm Incorporated) |
| R3-243440 | Logical TRP IDs management (Huawei) |