3GPP TSG-RAN WG3 #124	R3-243844

Fukuoka, Japan, 20– 24 May 2024

Agenda Item:	12.2

Source:	NTTDOCOMO (moderator)

Title:	Summary of Offline Discussion on additional topological enhancement

Document for:	Approval

Introduction

This document provides a summary of the offline discussion on additional topological enhancements.

CB: # WAB

- Resolve the FFS captured above

- Converge on the TPs below, where agreements taken above will be captured. If any more agreements are taken, they can be included in the TPs below:

  - TP for Architecture (Nokia)

  - TP for Integration procedure (Huawei)

  - TP for Authorization (CATT)

  - TP for Mobility (Ericsson)

  - TP for miscellaneous issues (ZTE)

    - WAB configuration

    - Etc

- SA2 reply LS (Qualcomm)

(Moderator – Docomo)

Summary of offline disc in R3-243844

Discussion

- FFS issues

Upon WAB-gNB mobility:

If needed, the WAB-gNB may power up one or more new cells with new configuration parameters related to its current location and handover UEs between the old and new cell served by the WAB-gNB.

FFS if this procedure is needed for AMF relocation.

Discuss whether in case this procedure is used for AMF relocation, new cells have to belong to different logical gNBs or can belong to the same gNB.

Q1. If needed, the WAB-gNB may power up one or more new cells with new configuration parameters related to its current location and handover UEs between the old and new cell served by the WAB-gNB. Is this needed for AMF relocation?

| Company | comments |
| --- | --- |
| ZTE | Yes, if AMF relocation is needed, intra-RAN node N2 based HO is performed between the old and new cells for the AMF relocation. |
| Ericsson | For example, it is needed when AMF relocation is accompanied by change of parameters that affect UE connectivity, such as TAC. The two may often go hand-in-hand. |
| CATT | This is needed for AMF relocation if cell ID of WAB-gNB is changed. In our view, this is a feasible mechanism to handle each case where the Cell ID of WAB-gNB is changed during WAB-node’s movement, no matter AMF is relocated or not for UEs. UEs should be handed over from the cell with old PCI to another cell with new PCI. The Cell ID changing may happen during AMF relocation (may be depending on configuration?). We are not sure whether the cell ID of WAB-gNB must be changed during AMF relocation, if that’s the case this certainly should be performed. Therefore, we further propose following rewording to the agreement: If needed cell ID of WAB-gNB is changed, the WAB-gNB may power up one or more new cells with new configuration parameters related to its current location and handover UEs between the old and new cell served by the WAB-gNB. |
| Huawei | No. If AMF relocation is needed, the TAC of cells can be changed and the UE can perform mobility registration update for the AMF relocation. Please refer to the following citation from TS 23.502. From description in 4.2.2.1, the CM-connected UE (which can be RRC connected or RRC inactive) can trigger registration procedure upon changing to a new TA outside the UE’s RA. And from the description in 4.2.2.2.3, the AMF re-allocation can be triggered by UE’s registration procedure. So, our view is: the handover of UEs between two cells is not needed for AMF relocation which can be achieved by updating the TAC broadcasted by the WAB-cell. Citation from TS 23.502: ----------skip the flow chart to save some space----------- |
| China Telecom | Agree with CATT. We support to specify a general mechanism for all mobility scenarios. |
| DCM | We understand that it is needed for AMF relocation (when some parameters change). |
| LGE | We also agree that this procedure can be used for AMF relocation. |
| Qualcomm | Huawei is raising a good point. The TAC will certainly change when the AMF is changed. Also, the new TAC cannot belong to the UE’s registration area configured by the old AMF. Therefore, the UE can perform the MRU. However, we want to verify with internal SA2 folks. |
| Qualcomm 2 | Filip pointed out that the TAC is up to RAN, and OAM can simply configure the same TAC for both AMFs. In this case, there would be no trigger for the UEs to do MRU. In such scenarios, cell change (NG handover or reselection) would be necessary to trigger AMF relocation. |

Summary: In case TAC change, it seems MRU can be used. In case OAM can simply configure the same TAC for both AMFs, it is necessary to further check whether the scenario is valid. RAN3 to further discuss this issue in the next meeting.

Q2. In case this procedure above is used for AMF relocation, whether new cells have to belong to different logical gNBs or can belong to the same gNB?

| Company | comments |
| --- | --- |
| ZTE | We think the new cells can belong to the same gNB. According to 23.502, the inter NG-RAN node N2 based handover procedure specified in clause 4.9.1.3 in 23.502 may also be used to change AMF for UEs for intra-NG-RAN node handover scenario when the source and target gNB are the same gNB. So currently we don’t see any issue if the new cells has the same gNB ID with the old cells. |
| Ericsson | In our understanding, if source and target UE’s AMF belongs to different AMF sets, a second logical WAB-gNB needs to be instantiated. |
| CATT | First, as we comment in Q1, it should be noted that above UE handover procedure is used when cell ID is changed during AMF relocation. Then, Q2 discuss whether gNB-ID is changed during AMF relocation. If the gNB-ID is changed the WAB-gNB different logical gNBs can be instantiated to connect to different AMFs, but that is up to implementation. Then the issue is that whether different logical gNBs should be instantiated when the gNB-ID is not changed. A gNB can connect to more than one AMFs even if those AMFs belong to different AMF sets. So, we tend to believe the gNB-ID can be kept unchanged, however, this is totally OAM configuration. When gNB-ID is not changed, there is no need to instantiate two logical gNBs. Following agreement is proposed: During AMF relocation when cell ID of WAB-gNB is changed, if gNB-ID is not changed the two cells belong to the same gNB, while if gNB-ID is also changed the two cells can belong to different logical gNBs. |
| Huawei | First, we think the handover of UEs is not necessary, refer to our comment for previous question. In case the handover is needed (needs to be confirmed by SA2), we Agree with ZTE. There is no limitation that the WAB-gNB can only connect to one AMF set. Hence, the new cells can belong to the same gNB. |
| China Telecom | Agree with ZTE. One WAB-gNB could connect to different AMF sets, thus WAB only instantiate one logical gNB at any time. |
| DCM | It depends on whether the gNB ID call be unchanged or not when powering up the new cells. If the gNB ID has to be changed, then new cells have to belong to different logical gNBs. |
| LGE | We also think that the new cells can belong to the same gNB if there is no gNB-ID change. |
| QC | Let’s converge on this topic in next meeting. |

Summary: As it depends on the discussion result of Q1, RAN3 to further discuss this issue in the next meeting.

Q3. Do you agree to capture the following text for IP address allocation for WAB-node in the TR 38.799:

4.3.x IP address allocation for WAB-node

A WAB-MT may obtain IP address(es) as a normal UE. The WAB-MT may deliver the allocated IP address(es) to the co-located WAB-gNB, which is used by the WAB-gNB for traffic exchange via the backhaul.

Alternatively, the WAB-gNB may obtain dedicated IP address(es) from operator. In this case, separate IP addresses are used by the WAB-gNB and co-located WAB-MT. In this case, a tunnel (e.g. based on IPsec or L2TP) could be established to transfer the WAB traffic by implementation. If a tunnel is established, a gateway may be deployed to terminate the tunnel.

| Company | comments |
| --- | --- |
| ZTE | Yes. We need to capture how IP address of WAB-gNB is allocated, which is used for OAM/NG/Xn traffic transfer. There is some benefits for the WAB-gNB to use separate static/dedicated IP address rather than use the same IP address as WAB-MT. By using static/dedicated IP address, the IP address of the WAB-gNB can be maintained and doesn’t need to be re-allocated upon change of BH-UPF. In this case, the OAM connection, SCTP connection, NG/Xn connection don’t need to be re-established/updated upon change of BH-UPF. And the NG-U GTP-U tunnel doesn’t need to be redirected to be new IP address. Moreover, it is flexible to configure multiple IP addresses/prefixes in different domain for the WAB-gNB for different usages (e.g., OAM, CP/UP traffic) if the WAB-gNB doesn’t use the same IP address as WAB-MT. Another potential solution is that the WAB-gNB may obtain its IP addresses anchored at BH-UPF via OAM. We suggest to capture all potential solutions in the TR and further discuss them and evaluate during next meeting. A WAB-MT may obtain IP address(es) as a normal UE. The WAB-MT may deliver the allocated IP address(es) to the co-located WAB-gNB, which is used by the WAB-gNB for traffic exchange via the backhaul. Or the WAB-gNB may obtain its IP addresses via OAM. Alternatively, the WAB-gNB may obtain dedicated IP address(es) from operator. In this case, separate IP addresses are used by the WAB-gNB and co-located WAB-MT. In this case, a tunnel (e.g. based on IPsec or L2TP) could be established to transfer the WAB traffic by implementation. If a tunnel is established, a gateway may be deployed to terminate the tunnel. |
| Ericsson | Yes, we have commented on the related TP in the present CB. |
| CATT | It should be clarified the IP address obtained from WAB-MT or operator is used for WAB-gNB NG interface setup. |
| Huawei | Yes, but from our understanding, it is more reasonable for the WAB-gNB to reuse the IP address of the WAB-MT. Even the static/dedicated IP address is allocated for the WAB-gNB, Xn/NG connection may be re-established/updated since the change of BH-UPF will cause the BH PDU session interruption. Suggest some rewording based on ZTE’s version, because it is hard to understand “obtain dedicated IP address(es) from operator”, even if the IP address is static, it should be allocated by OAM. A WAB-MT may obtain IP address(es) as a normal UE. The WAB-MT may deliver the allocated IP address(es) to the co-located WAB-gNB, which is used by the WAB-gNB for traffic exchange via the backhaul. Or the WAB-gNB may obtain its IP addresses via OAM. Alternatively, the WAB-gNB may obtain dedicated IP address(es) from operator. In this case, separate IP addresses are used by the WAB-gNB and co-located WAB-MT. In this case If the OAM assign a static IP address to the WAB-gNB, a tunnel (e.g. based on IPsec or L2TP) could be established to transfer the WAB traffic by implementation. If a tunnel is established, a gateway may be deployed to terminate the tunnel. |
| China Telecom | We understand OAM can also preconfigure IP address for WAB-gNB which is used to establish Xn/NG/OAM interface. Therefore, we propose to not preclude other implementation solution for IP address of WAB-gNB. |
| DCM | Yes. |
| Qualcomm | No. There is no “normal” UE. The MT does not “deliver” IP addresses, since this is out of scope of the SI as we agreed. Further, T2TP is not mandated for NG and Xn. Therefore, using T2TP will not provide inter-vendor interoperability, and therefore, it is out-of-scope for the study. Yes, we can add the figures to the TR, but we need to mention the above, i.e., out of scope since no interoperability. As a viable alternative to T2TP, IPsec tunnel mode can be used. NG and Xn need to support IPsec based on TS 33.501. Therefore IPsec tunnel mode must be supported. As a consequence, OAM could configure the inner IP address of this tunnel. We can capture this in the TR. Let’s please leave this for next meeting. |

Summary: Majority companies are fine to capture the above text for IP address allocation in the TR 38.799. “This is out of scope since no interoperability” need to be added.

Q4. Do you agree to capture in the TR 38.799 the following architecture and protocol stack as a potential option.

The WAB architecture for using dedicated IP addresses for WAB-gNB via tunnel when the WAB-gNB’s NG traffic is transported via PDU session backhaul is shown in Figure 3.

Figure 3: The WAB architecture example for 5GS using dedicated IP addresses for WAB-gNB when the WAB-gNB traffic is encapsulated in a tunnel and transported via PDU session backhaul

The corresponding protocol stack for NG-U and NG-C transport using layer 2 tunnel is shown in Figure 4.

Figure 4: The protocol stack for NG-U and NG-C transport using layer 2 tunnel

| Company | comments |
| --- | --- |
| ZTE | Yes. In our understanding, in study item phase, it’s open to discuss and capture all potential solutions which is in the scope of the SID. And then further down-selection or evaluation/recommendation can be made based on the potential solutions. As discussed in Q3, we see benefits for the WAB-gNB to use static/dedicated IP address rather than use the same IP address as WAB-MT. If separate IP addresses are used for WAB-gNB and WAB-MT, a tunnel needs to established to transfer the BH traffic. In this case, it would be much more clear to capture the corresponding architecture and protocol stack in the TR, to let people understand how it works. |
| Ericsson | OK to capture. However, at least Fig.3 needs update to align with the newest agreed terminology and also with the format already used in the TR. |
| Huawei | Can be captured as an alternative of architecture, then we need evaluation next meeting. |
| DCM | Yes. |
| QC | Yes, but only if we add that this solution is out of scope since it does not support inter-vendor interoperability since support of L2TP is not mandated for NG and Xn. |

Summary: Majority companies are fine to capture architecture and protocol stack as a potential option in the TR 38.799. “This is out of scope since no interoperability” need to be added.

Conclusion

Proposal1: RAN3 to further discuss the following FFS in the next meeting.

If needed, the WAB-gNB may power up one or more new cells with new configuration parameters related to its current location and handover UEs between the old and new cell served by the WAB-gNB.

FFS if this procedure is needed for AMF relocation.

Discuss whether in case this procedure is used for AMF relocation, new cells have to belong to different logical gNBs or can belong to the same gNB.

Proposal2: Add following text for IP address allocation in TR 38.799. “This is out of scope since no interoperability” need to be added.

4.3.x IP address allocation for WAB-node

A WAB-MT may obtain IP address(es) as a normal UE. The WAB-MT may deliver the allocated IP address(es) to the co-located WAB-gNB, which is used by the WAB-gNB for traffic exchange via the backhaul.

Alternatively, the WAB-gNB may obtain dedicated IP address(es) from operator. In this case, separate IP addresses are used by the WAB-gNB and co-located WAB-MT. In this case, a tunnel (e.g. based on IPsec or L2TP) could be established to transfer the WAB traffic by implementation. If a tunnel is established, a gateway may be deployed to terminate the tunnel.

Proposal3: Add the following architecture and protocol stack in TR 38.799. “This is out of scope since no interoperability” need to be added.

Figure 3: The WAB architecture example for 5GS using dedicated IP addresses for WAB-gNB when the WAB-gNB traffic is encapsulated in a tunnel and transported via PDU session backhaul

Figure 4: The protocol stack for NG-U and NG-C transport using layer 2 tunnel

References

| R3-243021 | LS on FS_VMR_Ph2 solution impacts to RAN (SA2(Qualcomm)) | LS in |
| --- | --- | --- |
| R3-243174 | Discussion on Wireless Access Backhaul (NTTDOCOMO, INC.) | discussion |
| R3-243199 | (draft Reply LS) Discussion on reply LS to SA2 on VS_VMR_Ph2 solution impacts on RAN (Qualcomm Inc.) | other |
| R3-243200 | (TP to TR 38.799) WAB requirements and archtiecture (Qualcomm Inc.) | other |
| R3-243201 | (TP to TR 38.799) WAB network integration and mobility (Qualcomm Inc.) | other |
| R3-243202 | (TP to TR 38.799) WAB resource coordination (Qualcomm Inc.) | other |
| R3-243217 | (TP to TR 38.799) Discussion on architecture and protocol stack for R19 WAB (ZTE) | other |
| R3-243218 | (TP to TR 38.799) Discussion on supporting WAB and the reply LS on FS_VMR_Ph2 solution impacts to RAN (ZTE) | other |
| R3-243219 | (TP to TR 38.799) Discussion on WAB mobility and resource multiplexing (ZTE) | other |
| R3-243247 | Consideration on architecture and mobility for WAB (NEC) | discussion |
| R3-243305 | (TP for TR 38.799) General aspects of WAB (Xiaomi) | other |
| R3-243306 | (TP for TR 38.799) SA2’s LS for WAB (Xiaomi) | other |
| R3-243327 | (TP to TR 38.799) Discussion on impact of WAB (CATT) | other |
| R3-243328 | (draft reply LS) Discussion on LS for VMR from SA2 (CATT) | other |
| R3-243329 | (TP to TR 38.799) Discusson on resource multiplexing for WAB (CATT) | other |
| R3-243339 | Discussion on the Architecture, Access control and QoS support of WAB (Huawei) | pCR |
| R3-243340 | Discussion on the procedures related to the WAB (Huawei) | pCR |
| R3-243341 | Discussion on SA2's LS (S2-2405822/R3-243021) on FS_VMR_Ph2 solution impacts to RAN (Huawei) | discussion |
| R3-243351 | (TP to draft TR 38.799) Discussion on WAB architecture and high level aspects (Nokia, Nokia Shanghai Bell) | other |
| R3-243352 | Discussion on WAB integration and mobility (Nokia, Nokia Shanghai Bell) | discussion |
| R3-243353 | Resource Multiplexing for WAB (Nokia, Nokia Shanghai Bell) | discussion |
| R3-243361 | (pCR for TR 38.799) WAB Architecture and Scenarios (Ericsson) | pCR |
| R3-243362 | (pCR for TR 38.799) Functional Aspects of WAB-Nodes (Ericsson) | pCR |
| R3-243363 | Handling of Backhaul Link Degradation and Resource Multiplexing for WAB (Ericsson) | discussion |
| R3-243389 | Discussion on integration procedure for WAB node (Lenovo) | discussion |
| R3-243390 | Discussion on migration procedure for WAB node (Lenovo) | discussion |
| R3-243391 | Discussion on resource multiplexing for WAB node (Lenovo) | discussion |
| R3-243581 | Discussion on enhancements for WAB (CANON Research Centre France) | discussion |
| R3-243583 | Discussion on RAN impact of SA2 solution in WAB (LG Electronics) | discussion |
| R3-243584 | (TP for TR 38.799) Architecture and protocol stack for WAB’s Xn (LG Electronics) | other |
| R3-243585 | Reply LS on FS_VMR_Ph2 solution impacts to RAN (LG Electronics) | LS out To: SA2 CC: RAN2 |
| R3-243588 | Views on FS_VMR_Ph2 Solution Impacts to RAN (China Telecom) | discussion |
| R3-243648 | (TP to TR 38.799) Discussion on network integration for WAB (Samsung) | pCR |
| R3-243649 | (TP to TR 38.799) Discussion on WAB mobility (Samsung) | pCR |

| R3-243020 | LS on Support of UE move between CAG cell of 5G Femto and CSG cell (SA2(Docomo)) | LS in |
| --- | --- | --- |
| R3-243025 | LS to request clarification on the potential baseline system architecture of 5G NR Femto (SA3(China mobile)) | LS in |
| R3-243175 | Discussion on NR Femto (NTTDOCOMO, INC.) | discussion |
| R3-243176 | draft Reply LS on Support of UE move between CAG cell of 5G Femto and CSG cell (NTTDOCOMO, INC.) | LS out To: SA2 CC: RAN2 |
| R3-243187 | 5G femto architecture considerations (AT&T Services, Inc.) | discussion |
| R3-243197 | Comment on Femto architecture options 1 and 2 (Charter Communications, Inc) | other |
| R3-243198 | Xn Gateway in Femto architecture (Charter Communications, Inc) | discussion |
| R3-243203 | (draft Reply LS) Discussion on reply LS to SA2 on Support of UE move between CAG cell of 5G Femto and CSG cell (Qualcomm Inc.) | other |
| R3-243232 | Access to local services from the 5G Femto via distributed UPF (Huawei) | pCR |
| R3-243252 | Open issues on NR femto (NEC) | discussion |
| R3-243315 | Discussion on NG connection and interface for Option 2 for NR Femto Architecture (Baicells) | discussion |
| R3-243316 | (TP to TR 38.799)NG connection and interface for Option 2 for NR Femto Architecture (Baicells) | discussion |
| R3-243330 | (TP to TR 38.799) On 5G Femto architecture (CATT) | other |
| R3-243331 | (TP to TR 38.799) On 5G Femto local service access (CATT) | other |
| R3-243332 | (TP to TR 38.799) On 5G Femto access control mechanism (CATT) | other |
| R3-243342 | Discussion on the architecture and access control for NR Femto (Huawei) | pCR |
| R3-243343 | Discussion on SA2’s LS (S2-2405813/ R3-243020) on Support of UE move between CAG cell of 5G Femto and CSG cel (Huawei) | discussion |
| R3-243374 | NR Femto Architecture and Ongoing Issues (Ericsson LM) | pCR |
| R3-243392 | Architecture for NR Femto (Lenovo) | discussion |
| R3-243393 | Access control and handover for NR Femto with CAG (Lenovo) | discussion |
| R3-243394 | Discussion on interworking between CAG and CSG cells (Lenovo) | discussion |
| R3-243409 | NR Femto Node Access Control with CAG (Ericsson LM) | pCR |
| R3-243411 | Access to Local Services via Local UPF (Ericsson LM) | pCR |
| R3-243562 | [TP for TR 38.799] Evaluation of NR Femto Architecture Options (Nokia, TMO US, AT&T, Verizon Wireless, KDDI, British Telecom, NTT Docomo Charter) | other |
| R3-243563 | Support of UE Move between CAG cell and CSG cell (Nokia) | discussion |
| R3-243564 | Reply LS on Support of UE move between CAG cell of 5G Femto and CSG Cell (Nokia) | LS out To: SA2 CC: |
| R3-243565 | [TP for TR 38.799] Access to Local Services (Nokia ) | other |
| R3-243586 | Discussion on SA2 solutions to support HO between CAG and CSG cell (LG Electronics) | discussion |
| R3-243587 | Reply LS on Support of UE move between CAG cell of 5G Femto and CSG cell (LG Electronics) | LS out To: SA2, RAN2 CC: |
| R3-243607 | On Access Control for NR Femto (China Telecom) | discussion |
| R3-243608 | On Architecture for NR Femto Support (China Telecom) | discussion |
| R3-243650 | (TP to TR 38.799) Discussion on Femto architecture (Samsung) | pCR |
| R3-243651 | (TP to TR 38.799) Discussion on access control for NR Femto (Samsung) | pCR |
| R3-243686 | draft Reply LS to request clarification on the potential baseline system architecture of 5G NR Femto (NTTDOCOMO, INC.) | LS out To: SA3 CC: SA2 |
| R3-243753 | (TP to TR 38.799)Discussion on architecture and access control of NR Femto (ZTE) | discussion |
| R3-243754 | (TP to TR 38.799)Discussion on support of local services (ZTE) | discussion |
| R3-243755 | Discussion on CSG-CAG mobility (ZTE) | discussion |
| R3-243761 | Draft Reply LS to request clarification on the potential baseline system architecture of 5G NR Femto (CMCC) | LS out To: SA3 CC: SA2 |